3GPP TSG RAN WG2 Meeting #114-e	                                        R2-210xxxx

- Electronic, 19th – 27th May 2021

Agenda item:		8.14.2.2

Source:			Lenovo

Title:		Report from email discussion [AT114-e][027][QoE] Start and Stop (Lenovo) – Phase 2 (LS out)

Document for:	 	Discussion and decision

# Introduction

As result of the online discussion of the report [AT114-e][027][QoE] Start and Stop (Lenovo) in R2-2106661 the following agreement was made on QoE report handling during QoE pause:

- On whether to store reports in the AS or the application layer at Pause, Send LS to SA4/SA5/SA3 to inform them about the options and their pros/cons (if possible) and ask them for feedback. RAN2 will continue work on this topic based on the feedback received.

Scope of this email discussion is to discuss and agree on the content of the LS to SA4/SA5/SA3:

- On the description of the pros/cons for each option.

- On the specific actions to ask to SA4/SA5/SA3.

# Discussion

## Description of Option 1

As result of the email discussion the description of the pros/cons for Option 1 was updated as shown below.

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Option 1, RAN transparent approach (i.e. QoE reports during “pause” are stored at application layer) | - There is no need to store the QoE reports in AS layer, which utilizes BP memory which is a scarce resource. It makes no sense to buffer a very limited amount in AS layer. - Considering UE storage is large (e.g. 256G), QoE data amount could be stored as much as possible. - Buffering in application layer gives the chance to transfer stored QoE data when the application layer is terminated. - Future-proof to consider QoE measurement in IDLE and Inactive state. - Very limited impact to RAN2 specifications, e.g. there is no need to discuss details of QoE reports storing in AS layer (e.g. maximum storing time, maximum size of stored reports, priorities etc.) or to define reporting of stored QoE reports after resume is indicated (i.e. QoE reports are handled in the same way as during normal operation) | - Application layer behavior upon reception of pause/resume indications needs to be specified by SA4. - Indicating pause to the applications basically means telling the applications that RAN is overloaded. This could be a security risk, if there are malicious applications that would like to take down the network. This needs to be checked with SA3. - Handling of reports from different applications when the overload situation has passed may need to be specified by SA4, This needs to be checked with them. |

Question 1: Companies are requested to provide their comments on the description of the pros/cons for Option 1, i.e. which of the bullet points are agreeable / not agreeable.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree with some | - Indicating pause to the applications basically means telling the applications that RAN is overloaded. This could be a security risk, if there are malicious applications that would like to take down the network. This needs to be checked with SA3. - We are not indicating an overload, we are just indicating that the application layer should pause sending the reports to the AS layer. We do not think there is any security issue. - Handling of reports from different applications when the overload situation has passed may need to be specified by SA4, This needs to be checked with them. - This bullet is a repetition of the first bullet, just using different wording. It can be removed. If the table is to be captured in the LS, then we should also not use statements like: “This needs to be checked with them”. |
| Ericsson | Disagree with some | Reply to HW’s comment: RAN overload would be the most common (only?) use case for using the pause mechanism, so indicating pause is basically the same as indicating overload. RAN2 cannot judge whether it is a security issue, it needs to be checked with SA3. What is meant by this? ” Buffering in application layer gives the chance to transfer stored QoE data when the application layer is terminated”. For both solutions the files will be transferred after resume. Does “terminated” refer to something else? [QC] “Application terminated” means application is finished and closed. This is the same as today application layer log files are stored after application if finished. How can we say the solution is future-proof? We don’t know yet if the files will be stored in application layer for IDLE and INACTIVE state. Propose to remove “Future-proof to consider QoE measurement in IDLE and Inactive state”. [QC] If we expect to support IDLE and INACTIVE QoE measurement for MBS, then we need enough space to store QoE log. Buffer such QoE log in AS layer for a long time is not definitely practice. We can remove the bullet regarding the specification impact, or add it as both advantage and disadvantage for both option 1 and 2. Either the impact will be in RAN2 specifications or in SA4 specifications. Whether it is an advantage or disadvantage just depends on from what angle you look at it. For option 1, large impact on SA4 specifications can be added as a disadvantage. The disadvantage listed for option 2 can be added here: “Requires App Layer – AS layer coordination, besides impacts to RRC the interaction goes beyond RAN2, thus impact functionality definition and maintenance in cross-WGs specifications”. |
| OPPO |  | Reply to Ericsson’s comment: RAN overload could be detected by malicious person in other ways, e.g., monitoring downloading speed, monitoring the response time to a instructing command to the browser. We do not think Option 1 could cause a security problem and agree with Huawei that the ‘Indicating pause to the applications basically means telling the applications that RAN is overloaded. This could be a security risk, if there are malicious applications that would like to take down the network. This needs to be checked with SA3.’ should be deleted. |
| Qualcomm | Disagree with some | Not convinced with the security issue. QoE data buffering is not only used in RAN overload case, but also used in other cases, e.g. QoE measurements in IDLE and INACTIVE state for future release; or used in moving to a gNB not supporting QoE capability for QoE continuity; or any other case gNB wants to pause QoE reporting. AS layer only informs application layer pause or resume indication, not exposes the pause cause to application layer, so application layer is unaware of the RAN level situation. Besides, the application layer supporting QoE is 3GPP defined application not the 3rd party or OTT application layers and these application layers can be controlled and tested by 3GPP. So we don’t see the security issue exists. Also agree with OPPO there are many methods today for application layer to detect RAN situation. Suggest to replace “BP” with “modem” for easy understanding. |
| Intel | Partially disagree | - Future-proof to consider QoE measurement in IDLE and Inactive state. It is not clear to us what does the “future-proof” mean here. If it refers to keep QoE measurement when UE goes to RRC_IDLE or RRC_INACTIVE state in future release, we think both option 1 and option 2 can support such forward compatibility. Hence, we suggest removing this point. - Considering UE storage is large (e.g. 256G), QoE data amount could be stored as much as possible. We also wonder whether there’s a need to store that much amount of QoE measurement report during QoE pause. We suggest checking with SA4 and SA5 about “whether all QoE reports are necessary to be reported back to the network after QoE resume, and what size is required?”. Before that, we may not consider this as an advantage. - Buffering in application layer gives the chance to transfer stored QoE data when the application layer is terminated. Storing at AS layer can also support transferring the stored QoE data to the network after QoE resume. The difference between the two options is whether all QoE reports during QoE pause is important to the network. Hence, we also think this advantage may need to be reconsidered after we receive feedback from SA4/SA5 about question above. As for the disadvantage, this option also collides with SA4 conclusion “Regarding temporary stop and restart, SA4 agrees that this seems to be a useful functionality to handle temporary RAN overload. However, as we also stated in the above reply, SA4 believes that there should be no need for involving the application in this scenario”. |
| Lenovo | Disagree some | On advantages: We agree with Intel that the below bullet point can be removed since we don’t know yet the potential solutions for idle/inactive states. “Future-proof to consider QoE measurement in IDLE and Inactive state.” On disadvantages: We agree with Nokia comment to “Application layer behavior upon reception of pause/resume indications needs to be specified by SA4”. QoE measurement collection will continue at pause, so the issue is only on handling QoE reports which are generated during QoE pause. But this is already reflected in the third bullet point. |

## Description of Option 2

As result of the email discussion the description of the pros/cons for Option 2 was updated as shown below.

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Option 2, Application transparent approach (i.e. QoE reports during “pause” are stored at AS layer) | - Application layer is unaffected | - The QoE reports need to be stored at AS layer, which has more limited storage capacity - May impact non-QoE data processing performance due to reduced AS buffer size. - High workload and specifications impact in RAN2, e.g.to discuss the details of QoE reports storage and reporting after UE receives pause/resume indications - Collide with SA5 specification “Temporary stop and restart of QoE information reporting during RAN overload.” In TS 28.405. - Requires App Layer – AS layer coordination, besides impacts to RRC the interaction goes beyond RAN2, thus impact functionality definition and maintenance in cross-WGs specifications |

Question 2: Companies are requested to provide their comments on the description of the pros/cons for Option 2, i.e. which of the bullet points are agreeable / not agreeable.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree | We do not understand the comments from companies about AS layer using non-AS layer memory for storing the reports. What is the difference between option 1 and option 2 if the reports are anyway stored in application layer memory? Also, if AS layer can utilize any kind of memory, then why did we introduce 64 kBytes limitation for MDT? It is obvious to us there will be storage limitations if we choose option 2 and we should make SA4 aware of that. We should also provide a rough number, e.g. 64 kBytes as in the case of MDT, so that they are not surprised in future. |
| Ericsson | Disagree | We can remove the bullet regarding the specification impact, or add it as both advantage and disadvantage for both option 1 and 2. Either the impact will be in RAN2 specifications or in SA4 specifications. Whether it is an advantage or disadvantage just depends on from what angle you look at it. For option 2, we can add an advantage that there will be no or limited impact on SA4 specifications. Regarding the collision with SA5 specifications, it is better removed. RAN2 has not followed what SA5 has specified in 28.405 before. SA5 asked RAN2 to introduce the QoE functionality in 28.405 for LTE for more than two years in multiple LSes which RAN2 never did. Also, SA4 replied that they prefer the functionality to be in RAN, so in such case it should be added as an advantage that the SA4 request is fulfilled for this option. It is not clear to us why there would be any application layer-AS layer coordination in this solution. Everything will be handled by RAN in this case (could be added as an advantage). Propose to remove or clarify why coordination would be needed. |
| OPPO | Agree |  |
| Qualcomm |  | Comment on the question about limited buffering size raised by Lenovo, if buffered in AS layer, it is definitely buffered in modem. The memory size in modem is limited, that is why we limit the memory size of MDT logged measurements is 64KB and the RRC buffer size is 45Kbytes. Comment on comment raised by Lenovo for SA5 specification. All the requirement in TS 28.405 is for UMTS and LTE, there is no requirement for NR. SA4 didn’t follow SA5 requirement and RAN2 has technical concerns on SA4 decision. If we just follow SA4 current decision so far, it is unfair to RAN2. Actually, RAN2 can do the same thing with SA4 that we don’t support pause/resume functionality, but in order to make QoE feature more useful and more practice, we are willing to discuss this issue and resolve this issue together with SA4/SA5. |
| Intel | Disagree | We share the same understanding as Lenovo and E/// commented before. Besides, we also would like to point out that SA5 also noted the reply from SA4 about “temporary stop does not need AL to be involved”. |
| Lenovo | Disagree some | On disadvantages we have the same view as Ericsson. |

## Description of Option 3

As result of the email discussion the description of the pros/cons for Option 3 was updated as shown below.

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Option 3, RAN transparent approach (i.e. AS discard the QoE data during “pause”) | - There is no need to store the QoE reports in AS layer, which utilizes RAM memory which is a scarce resource - Not touch application behavior. - Very limited impact to RAN2 specification. - Fits purpose of using RAN as interface to pass available QoE measurements, without introducing new requirements going beyond RAN | - Application layer may or may not buffer the QoE data during pause, which can be decided by SA4. - Restrict the supporting for partially Pause - No benefit of pause mechanism if the reports are anyhow discarded. Then the network might as well deconfigure QoE. |

Question 3: Companies are requested to provide their comments on the description of the pros/cons for Option 3, i.e. which of the bullet points are agreeable / not agreeable.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree with some | - Fits purpose of using RAN as interface to pass available QoE measurements, without introducing new requirements going beyond RAN - The problem with this option is that it actually does not serve the purpose of passing the QoE reports to the network. This statement seems incorrect to us. - Restrict the supporting for partially Pause - We are not sure what this bullet means. |
| Ericsson | Disagree with some | Agree with Huawei’s comments. |
| OPPO | Disagree with some | Agree with Huawei’s comments |
| Qualcomm | Disagree with some | Agree with Huawei’s comments Need to replace “RAM” with “modem” for accurate description. |
| Lenovo | Disagree with some | We agree with the comments from Huawei, HiSilicon. |

## Specific actions to ask to SA4/SA5/SA3

Referring to the 3GPP calendar SA4, SA5 and SA3 have their next meetings in Q3 as follows:

- SA3#103-e: 16-27 August 2021

- SA4#115-e: 18-27 August 2021

- SA5#138-e: 23-31 August 2021

That means, responses from them we can expect in our meeting(s) in Q4/2021 at the earliest. Therefore, we should be very specific about the actions we want to ask them. Referring to the discussions we had, rapporteur drafted a first description of the LS and actions to the concerned groups as shown below.

Overall description RAN2 discussed the handling of QoE reports during RAN overload and agreed that the RAN node can send a QoE pause indication to instruct the UE to temporarily stop sending QoE reports of the affected QoE measurements configuration until receiving a QoE resume indication from the RAN node. With regards to the handling of QoE reports in the UE which are generated during QoE pause, RAN2 identified three options: - Option 1: Application layer is responsible for storing QoE reports when the UE receives QoE pause indication. - Option 2: AS layer is responsible for storing QoE reports when the UE receives QoE pause indication. - Option 3: The QoE container received from application layer is discarded during pause. In the table below the description of the pros/cons for each option has been summarized. RAN2 did not reach consensus on the option to pursue and seek guidance from SA4, SA5 and SA3. <Table to add> Action to SA4: RAN2 kindly asks SA4 to take the information about the options for handling QoE reports during QoE pause into account and provide their feedback whether QoE reports during QoE pause can be stored at the application level. Action to SA5: RAN2 kindly asks SA5 to take the information about the options for handling QoE reports during QoE pause into account and provide their feedback on the feasibility of each option. Action to SA3: RAN2 kindly asks SA3 to provide their feedback on the potential security issue raised for Option 1.

Question 4: Companies are requested to provide their comments on the action to SA4.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | OK in general, but… | We think we should ask SA4 for feedback on all the options, similarly as SA5, e.g.: “RAN2 kindly asks SA4 to take the information provide feedback about the options for handling QoE reports during QoE pause into account and in particular provide their feedback on whether QoE reports during QoE pause can be stored at the application level. “ |
| Ericsson | Agree. | Looks fine for now. |
| CATT |  | Agree with HW, we should ask their feedback. |
| ZTE |  | We share the same view with Huawei. |
| OPPO |  | Agree with Huawei |
| Intel | Agree with comment | We think it would be good to make the statement more neutral (i.e. similar as “Action to SA5”): Action to SA4: RAN2 kindly asks SA4 to take the information about the options for handling QoE reports during QoE pause into account and provide their feedback whether QoE reports during QoE pause can be stored at the application level on the preference of each option. |

Question 5: Companies are requested to provide their comments on the action to SA5.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Agree in general, but | We should ask for feedback in general, not on feasibility. It does not seem to us that any of the options is infeasible. |
| Ericsson | Agree | Looks fine for now. |
| CATT | Agree | Huawei’s concern is reasonable. We just use the first half sentence as to SA4 “RAN2 kindly asks SA5 to take the information provide feedback about the options for handling QoE reports during QoE pause” Or merger the action to SA4 and action to SA5 in one item RAN2 kindly asks to take the information provide feedback about the options for handling QoE reports during QoE pause into account and in particular SA4 provide their feedback on whether QoE reports during QoE pause can be stored at the application level |
| ZTE | Agree | We are fine for current wording. |
| OPPO |  | Agree with Huawei |
| Intel | Agree with comment | Suggest to change “feasibility” to “preference”. |

Question 6: Companies are requested to provide their comments on the action to SA3.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree | We do not see how pause indication can be a security issue. Following this logic, enabling UAC or cell barring can also be a security issue as the UE may interpret it as the cell being overloaded. There is no need to involve SA3 in this discussion. |
| Ericsson | Agree | RAN2 cannot judge whether there is any security issue, we need to ask SA3. This cannot be compared with cell barring, it is about (basically) telling the applications that the network is overloaded. Also, RAN2 already agreed to send the LS to SA3: “On whether to store reports in the AS or the application layer at Pause, Send LS to SA4/SA5/SA3 to inform them about the options and their pros/cons (if possible) and ask them for feedback. RAN2 will continue work on this topic based on the feedback received.”. |
| CATT | Agree | Agree to check with the professional WG for assessing the security issue |
| ZTE | Agree | We are fine for current wording. |
| OPPO | Disagree | As we also said previously, APP layer has many more other ways to detect if or not the RAN has been overloaded. No need to involve SA3 in this discussion. |
| Qualcomm | Disagree | Not convinced with SA3 action as previous comment. |
| Lenovo | Agree | We are not security experts so it is good to ask SA3 for the issue raised by Ericsson. |

# Conclusion