3GPP TSG RAN WG2 Meeting #114-e	                                        R2-21xxxxx

- Electronic, 19th – 27th May 2021

Agenda item:		8.14.2.2

Source:			Lenovo

Title:		Report from email discussion [AT114-e][027][QoE] Start and Stop (Lenovo)

Document for:	 	Discussion and decision

# Introduction

This report summarizes the email discussion below that took place during RAN2#114-e meeting:

- [AT114-e][027][QoE] Start and Stop (Lenovo)

Scope: Start from the baseline, the tdocs under 8.14.2.2, identify easy agreements, potential agreements, discussion points, open points. Shall at this meeting attempt to decide R2 opinion whether/to what extent application shall be involved in Pause Resume or whether this is AS internal.

Intended outcome: Report.

# Reference

The following documents are treated in this email discussion:

[1]	R2-2104992	QoE pause and resume handling	Qualcomm Incorporated

[2]	R2-2105215	QoE report handling during RAN overload	Lenovo, Motorola Mobility

[3]	R2-2105337	Discussion on start and stop of QoE measurement	vivo

[4]	R2-2105525	Discussion on QoE measurement pausing and resuming	OPPO

[5]	R2-2105581	QoE measurement handling at RAN overload	Huawei, HiSilicon

[6]	R2-2105646	Discussion on NR QoE	China Unicom

[7]	R2-2105894	Pause and resume of QoE measurements	Ericsson

[8]	R2-2105920	QoE reporting control	Nokia, Nokia Shanghai Bell

[9]	R2-2106159	Discussion on QoE collection start and stop	CATT

[10]	R2-2106222	Further discussion on start and stop	CMCC

[11]	R2-2106346	Stop and start for QoE measurement reporting	LG Electronics Inc.

[12]	R2-2106431	Discussion on pause/resume NR QoE reporting	ZTE Corporation, Sanechips

# Contact information

| Company | Contact Name, Email |
| --- | --- |
| Qualcomm | Jianhua Liu, jianhua@qti.qualcomm.com |
| Huawei | Dawid Koziol, dawid.koziol@huawei.com |
| Ericsson | Cecilia Eklöf, cecilia.eklof@ericsson.com |
| Apple | Pavan Nuggehalli, pnuggehalli@apple.com |
| LGE | SangWon Kim, sangwon7.kim@lge.com |
| Lenovo | Hyung-Nam Choi, hchoi5@lenovo.com |
| Nokia, Nokia Shanghai Bell | Malgorzata.tomala@nokia.com |
| ZTE | Liu.yansheng@zte.com.cn |

# Discussion

## QoE report handling at QoE release

Details of the QoE release command will be discussed in the other email discussion [AT114-e][026][QoE]. Nonetheless, several contributions [3], [5], [8] discussed the QoE report handling at QoE release and made the following proposal:

At reception of QoE release, the UE shall discard any unsent QoE reports corresponding to the released QoE configuration.

Question 1: Do companies agree on the proposal that at reception of QoE release, the UE shall discard any unsent QoE reports corresponding to the released QoE configuration?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Huawei, HiSilicon | Yes | This is similar case to the one handled in the other e-mail discussion, i.e. where the AS layer receives a QoE report for a non-existing QoE configuration. |
| Ericsson | Yes |  |
| Apple | Yes |  |
| LGE | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes |  |
| ZTE | Yes |  |

Rapporteur’s summary: <tbd>

## Scope of QoE pause/resume wrt affected QoE configurations

During RAN overload the RAN node may send a QoE pause command to instruct the UEs to stop/suspend QoE measurement reporting for configured QoE measurements, and when RAN overload has been relieved the RAN node may send a QoE resume command to the concerned UEs to (re)start/resume QoE measurement reporting for configured QoE measurements. The relevant point to discuss here is the scope of a QoE pause/resume command wrt the affected QoE configurations, and different proposals were made in several contributions:

- In [1] and [5] it is proposed that the received pause/resume command is applicable for one or more QoE configurations.

- In [8] and [11] it is proposed that the received pause command is applicable for all QoE configurations.

- In [9] it is proposed that the received pause/resume command is appliable for all or partial set of QoE configurations.

Question 2: Companies are requested to provide their views on the scope of a QoE pause/resume command wrt the affected QoE configurations.

| Company | Scope of a QoE pause/resume command | Additional comments |
| --- | --- | --- |
| Qualcomm | One or more QoE configurations | Give the flexibility to gNB to mitigate the overload situation. |
| Huawei, HiSilicon | One or more QoE configurations | Depending on the severity of the overload, it might not be required to pause all the QoE reports at once. |
| Ericsson | Choice between all, or a partial set of QoE configurations | It can be a choice between pausing all or selecting a subset of the QoE configurations to pause/resume. Pausing all command can be used at severe situation and selecting a subset can be used when still there is a chance for the gNB to collect QoE measurements. |
| Apple | Pause all QoE configurations | It is not clear on what basis QoE configurations can be selectively paused by RAN. Assuming RAN overload is not a frequent event, we don’t see the need to optimize. |
| LGE | Pause all QoE configurations | Considering the QoE measurement results have the lowest priority, it seems reasonable that the QoE reporting is suspend for all configured QoE measurements at RAN overload. We don’t want to introduce a complex mechanism to pause/resume a certain QoE measurement. |
| Lenovo | One or more QoE configurations | From NW point of view such flexibility may be beneficial to cope with overload situation. |
| Nokia | Pause all configurations | Any further granularity can be seen as an optimization, however RAN should have the option to pause all QoE configurations. |
| ZTE | One or more QoE configuration |  |

Rapporteur’s summary: <tbd>

## RAN functionality/responsibility at RAN overload

In RAN2#113bis-e meeting the agreements below were made:

RAN2 assumes that QoE support for NR includes (as the LTE framework): activation by Trace Function, both signalling and management-based configuration and RRC procedures supporting AppLayer config and report.

From RAN2 point of view, the UE shall follow gNB commands and, NG-RAN can in principle release by RRC the application layer measurement configuration towards the UE at any time, e.g. if required due to load or other reasons (Note that other WGs are responsible to define the normal system procedures for release and which nodes are responsible etc).

In view of above agreements several contributions further discussed the RAN functionality/responsibility at RAN overload and made the following proposals:

- In [3] it is proposed that the RAN is allowed to release QoE configuration only if instructed by CN/OAM. Otherwise, the RAN node is allowed to instruct the UE to pause QoE measurement reporting. - In [6] it is proposed to discuss two options for handling new QoE configuration which is sent from the OAM/CN to RAN: - Option 1: When RAN is overloaded, gNB sends an overload indication to the core network to stop QoE configuration. - Option 2: When RAN is overloaded, gNB can decide which new QoE measurement configurations should be stopped based on service type and/or slice priority. - In [8] it is proposed that RAN can release the application layer measurement configuration towards the UE at any time.

Although the interaction between RAN and CN/OAM at RAN overload is more in the scope of other groups, rapporteur thinks that it might be still good to gather RAN2 view on the RAN functionality/responsibility at RAN overload in general.

Question 3: Companies are requested to provide their views on the RAN functionality/responsibility at RAN overload. That means:

- whether RAN is responsible for release or pause of a QoE measurements configuration;

- whether CN/OAM is responsible for release of a QoE measurements configuration.

| Company | Scope of RAN functionality/ responsibility at RAN overload (RAN or CN/OAM) | Additional comments |
| --- | --- | --- |
| Qualcomm | Out of RAN2 scope | From RAN2 point of view, we already agree RAN can release QoE configurations from signalling point of view; and it is also agreed other WGs are responsible to define the normal system procedures for release and which nodes are responsible. And we should leave this issue to RAN3 discussion. “Note that other WGs are responsible to define the normal system procedures for release and which nodes are responsible etc.” |
| Huawei, HiSilicon |  | During RAN overload, it is of course up to RAN to pause or release some configurations to address this situation (and this we already agreed). In normal case, the deactivation command is coming from the CN/OAM, but this is out of RAN2 scope. Then, there is no need to discuss/agree anything additional with that respect. |
| Ericsson |  | Agree with Huawei’s comment. No need to discuss this further. |
| Apple |  | We think RAN should be able to able to pause QoE configurations based on its implementation. We don’t think any additional RAN-CN interaction is required. We should solicit the views of other groups (SA4/SA5) on release functionality. |
| LGE |  | Agree with Huawei’s comment. No need to discuss this further. |
| Lenovo |  | Agree that interaction between RAN and CN/OAM at RAN overload is out of RAN2 scope. However, we think that from RAN2 point of view we can make following confirmation: “At RAN overload the RAN can in-principle release or pause the application layer measurement configuration towards the UE at any time.” |
| Nokia |  | Agree with Lenovo, this is also following QoE in LTE, where eNB is allowed to release at any time (as per 36.300) |
| ZTE |  | We share the same view with Huawei. This is out of RAN2 scope. |

Rapporteur’s summary: <tbd>

## QoE report handling during QoE pause

In RAN2#113bis-e meeting the outcome of discussion on QoE report handling during QoE pause was as follows:

“QoE pause” indication from the network is used to temporarily stop QoE reports from being sent from the UE to the network. Application layer behaviour upon UE receiving “pause/resume” indications is out of RAN2 scope.

The following are options considered by RAN2 for QoE report handling during RAN overload via “QoE report pause indication”:

- Option 1: Application layer is responsible for storing QoE reports when the UE receives QoE pause indication.

- Option 2: AS layer is responsible for storing QoE reports when the UE receives QoE pause indication.

- Option 3: The QoE container received from application layer is discarded during pause.

The options for QoE report handling during RAN overload was discussed offline in [AT113bis-e][037], and the main advantages and disadvantages of the 3 options were summarized in the Table below.

Table: Summary of the main advantages and disadvantages of the options

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Option 1, RAN transparent approach (i.e. QoE reports during “pause” are stored at application layer) | - There is no need to store the QoE reports in AS layer, which utilizes BP memory which is a scarce resource. It makes no sense to buffer a very limited amount in AS layer. - Considering UE storage is large (e.g. 256G), QoE data amount could be stored as much as possible. - Buffering in application layer gives the chance to transfer stored QoE data when the application layer is terminated. - Future-proof to consider QoE measurement in IDLE and Inactive state. - Very limited impact to RAN2 specifications, e.g. there is no need to discuss details of QoE reports storing in AS layer (e.g. maximum storing time, maximum size of stored reports, priorities etc.) or to define reporting of stored QoE reports after resume is indicated (i.e. QoE reports are handled in the same way as during normal operation) | - Application layer behavior upon reception of pause/resume indications needs to be specified by SA4. - Indicating pause to the applications basically means telling the applications that RAN is overloaded. This could be a security risk, if there are malicious applications that would like to take down the network. This needs to be checked with SA3. - Handling of reports from different applications when the overload situation has passed may need to be specified by SA4, This needs to be checked with them. |
| Option 2, Application transparent approach (i.e. QoE reports during “pause” are stored at AS layer) | - Application layer is unaffected | - The QoE reports need to be stored at AS layer, which has more limited storage capacity - May impact non-QoE data processing performance due to reduced AS buffer size. - High workload and specifications impact in RAN2, e.g.to discuss the details of QoE reports storage and reporting after UE receives pause/resume indications - Collide with SA5 specification “Temporary stop and restart of QoE information reporting during RAN overload.” In TS 28.405. - Requires App Layer – AS layer coordination, besides impacts to RRC the interaction goes beyond RAN2, thus impact functionality definition and maintenance in cross-WGs specifications |
| Option 3, RAN transparent approach (i.e. AS discard the QoE data during “pause”) | - There is no need to store the QoE reports in AS layer, which utilizes RAM memory which is a scarce resource - Not touch application behavior. - Very limited impact to RAN2 specification. - Fits purpose of using RAN as interface to pass available QoE measurements, without introducing new requirements going beyond RAN | - Application layer may or may not buffer the QoE data during pause, which can be decided by SA4. - Restrict the supporting for partially Pause - No benefit of pause mechanism if the reports are anyhow discarded. Then the network might as well deconfigure QoE. |

Based on the contributions submitted to this topic the company’s views are still divergent.

- In [1], [3], [5] it is proposed that QoE reports are stored at application layer (Option 1).

- In [2], [6], [7], [9], [11], [12] it is proposed that QoE reports are stored at AS layer (Option 2).

- In [8] it is proposed that QoE reports are discarded (Option 3).

- In [10] it is proposed not to discard QoE reports (irrespective of whether QoE reports are stored in AS layer or application layer).

- In [2] it is proposed as a compromise solution that the network may signal explicitly in the QoE pause command whether the UE shall discard any QoE reports received during QoE pause.

- Furthermore, in [4] it is proposed to send an LS to SA4 to reconsider their decision to not implement the temporary stop and restart functionality at the application level.

However, in order to make some progress on this topic, rapporteur makes following observations based on above proposals:

- There seems to be a clear majority not to discard QoE reports during QoE pause (irrespective of whether they are stored in AS layer or application layer), so at least the proposal from [10] may be taken as baseline.

- As a compromise, companies may consider the solution from [2] in which the network may signal explicitly in the QoE pause command whether the UE shall discard any QoE reports received during QoE pause.

- It is not very likely that the deadlock between Option 1 and Option 2 can be resolved during this meeting. Therefore, one possible way forward is to make a working assumption/agreement on Option 1/Option 2 based on majority view, and to send LS to SA4/SA5 to inform them about the status of RAN2 discussion and to ask them for their feedback.

Based on above observations the following questions are raised.

Question 4: Do companies agree on the proposal to not discard QoE reports during QoE pause (irrespective of whether they are stored in AS layer or application layer?

| Company | Yes/No | Additional comments |
| --- | --- | --- |
| Qualcomm | Depends | Understand this question is specific to AS layer behaviour. We don’t talk about whether application layer should discard QoE data or not. In general, support AS layer discards QoE report if received. Can accept UE application layer buffer or discard QoE report. |
| Huawei, HiSilicon |  | We agree that discarding all reports during pause would be a bad approach. However, in case of storing the reports at AS layer, we will have a very limited storage capacity. We think we will have to share storage capacity with MDT, i.e. maximum of 64 kBytes would be available. In this case, discarding the subsequent reports exceeding this limitation would be inevitable. |
| Ericsson | Yes, agree to not discard QoE reports | We agree with Huawei that discarding the reports can not be a good approach as it is important to know the QoE at overload situation. We think similar to MDT reports it is possible to define a memory with maximum size of 64 kbytes to be used to store the QoE measurements at AS. However, sharing the memory with MDT may requires additional standardization effort to manage the shared memory between MDT and QoE. |
| Apple | No | The UE cannot be expected to store all QoE reports for an unpredictable duration of time (e.g., when RAN is overloaded) so some maximum size as indicated by Huawei and Ericsson is essential. |
| LGE | Yes | The QoE results should be stored during pause. |
| Lenovo | Yes | We agree with Ericsson’s comment (in the table above) that there would be no benefit of pause mechanism if the reports are anyhow discarded. We assume QoE pause is used to cope with a temporary RAN overload situation. Otherwise, the NW can anyway release QoE measurements configuration. |
| Nokia, Nokia Shanghai Bell | No | The UE AS layer can pass available data from other layer based on best effort, but should not sacrifice RAN resources to deliver data that by default are not immediately utilized by RAN |
| ZTE |  | Discarding all reports during the QoE pause is not an appropriate way for the pause/resume function. But we wonder whether force UE to keep all generated reports is valuable. It is hard to discuss this question without consider whether AS layer or app layer stores the reports during paused period. Considering the QoE report container size and available buffer size in this two layers, the cost of keeping all QoE reports in AS layer or app layer is different. |

Question 5: If the answer to Question 4 above is no, companies are requested to provide their views on the compromise solution in which the network may signal explicitly in the QoE pause command whether the UE shall discard any QoE reports received during QoE pause.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |
| Qualcomm | Disagree | No need. If we specify UE should buffer QoE data, then UE buffers; if not, then no buffer happens. |
| Huawei, HiSilicon | Disagree | We do not see the need for this. In case the reports are to be discarded, then the network may equally simply release the QoE configuration from the UE. |
| Ericsson | Disagree | No need for this. |
| Apple | Disagree | Agree with views above. |
| LGE | Disagree | We do not see the need for this. |
| Nokia, Nokia Shanghai Bell |  | Compromised solution could be to pause per service type |
| ZTE | Disagree |  |

Question 6: Companies are requested to provide their views on the possible way forward on Option 1/Option 2, i.e. to make a working assumption/agreement on Option 1/Option 2 based on majority view, and to send LS to SA4/SA5 to inform them about the status of RAN2 discussion and to ask them for their feedback.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |
| Qualcomm | Disagree | SA5 specification already descripts AS layer provides pause AT command to application layer if received from network. SA4 didn’t follow SA5 specification and put the task to RAN2. This is unfair to RAN2. So far, no technical advantages are identified for UE to buffer QoE report in AS layer; while a number of disadvantages are identified. We need to ask SA4 to reconsider SA5 requirements and technical concerns from RAN2. In order to make progress, we can send LS to SA4/SA5 with the advantages and disadvantages listed in the “Table: Summary of the main advantages and disadvantages of the options” for SA4/SA5 consideration. |
| Huawei, HiSilicon |  | We think the decision should be based on technical merits of each solution. It would be good if we could list the main characteristics of each approach and clarify this to SA4/SA5 in an LS, e.g. for AS layer solution SA4/SA5 should be definitely aware that AS layer is only able to offer a very limited storage space (we think 64 kBytes as specified for MDT can be assumed) and that at AS layer we would be able only to store QoE reports in a “first come first stored” manner and the subsequent reports would be discarded. In case more storage is needed or some more sophisticated logic for deciding which reports should be stored and which discarded, this will have to be done at the application layer. |
| Ericsson |  | We think it is a good approach to ask SA4, SA5 and also SA3 for feedback, listing the characteristics with the different solutions. It is important to ask SA3 also regarding the security issue with storing the reports in the application layer. Exposing the overload situation information to the upper layers may lead to security risk if malwares try to sniff the RAN nodes configuration and information is exposed to the upper layers. E.g., a denial-of-service attack (DoS attack) can be arranged in such situations that a RAN node may be vulnerable to such attacks at overload condition. We think RAN overload condition may not be a long period and not comparable with the QoE reporting periodicity (a reasonable QoE reporting periodicity may be 10-15 mins). Hence, we don’t expect that AS receives a lot of QoE reports at overload condition and a limited memory at AS (64kbytes) would easily handle the issue. If the overload situation lasts for a long time, the network needs to deconfigure QoE. |
| Apple |  | Another approach would be to leave it UE implementation. Unlike MDT, QoE information is not generated at the AS layer so we do not think there is a need to tie them together. The real question is what is the maximum buffer size needed at the UE for storing QoE reports. Where it is stored does not need to be specified. |
| LGE |  | Same view as Ericsson. It would be good to ask SA3 also regarding the security issue. |
| Lenovo | Agree | To move forward we should send LS to SA4, SA5 and SA3 to inform them about the options we discussed incl. the updated table of their main advantages and disadvantages and ask them for their feedback. The only concern we have is that this topic then may need to put on hold until we get feedback from them. However, in view of the R17 schedule we cannot wait for long. Otherwise, it means that in worst case the pause/resume functionality cannot be supported for NR QoE. So, we should be very clear and specific on the feedback we request from SA4, SA5, SA3. Furthermore, we need to clarify how we want to handle this topic in RAN2 in the meantime. |
| Nokia, Nokia Shanghai Bell |  | Agree with Apple. The same functionality in LTE was left to high extend to UE implementation, as even we can refer to AT command we cannot fully control by 3GPP specifications UE’s Application layer behaviour. |

Rapporteur’s summary: <tbd>

## UE buffer requirements for QoE reports

Several contributions discussed UE buffer requirements for QoE reports and handling of them during QoE pause, and made the following proposals:

- In [1] storage of QoE reports in application layer is assumed. But nonetheless, it is proposed to define a required storage size. The UE is required to support to store QoE measurement results up to the required storage size. It can be left to UE implementation whether to store more than the required storage size. Furthermore, stored QoE measurements can be overwritten by new QoE measurements e.g. measurements for higher priority service type, or for higher priority slice.

- In [6] storage of QoE reports in AS layer is assumed. Regarding the handling of them during QoE pause it is proposed to define a discard timer or discard event on which QoE reports should be discarded.

- In [7] it is proposed to define a maximum memory size at AS layer. The UE may discard the upcoming QoE measurements from the application when the memory is full.

- In [9] storage of QoE reports in AS layer is assumed, and it is proposed to define a timer and a storage size for storing QoE report. If timer expires and storage size exceeds its limit then:

- Option 1: Keep the stored QoE reports and inform Application layer to stop the QoE measurements for the corresponding QoE configuration.

- Option 2: Discard the stored first-in QoE reports and continue the QoE measurements for the corresponding QoE configuration

- In [12] storage of QoE reports in AS layer is assumed. It is proposed to discuss and introduce the limitation for the QoE buffer.

Although most of the proposals assume storage of QoE reports in AS layer, rapporteur thinks that it is beneficial to discuss the need of specifying UE buffer requirements for QoE reports in general (irrespective of whether they are stored in AS layer or application layer).

Question 7: Companies are requested to provide their views on specifying UE buffer requirements for QoE reports (irrespective of whether they are stored in AS layer or application layer) and UE behaviour when QoE reports exceed the buffer size.

| Company | Need of UE buffer requirements (Yes/No) | Additional comments |
| --- | --- | --- |
| Qualcomm | Yes | Regardless UE buffers in application layer or AS layer, need of UE buffer requirements. |
| Huawei, HiSilicon | Yes, but… | RAN2 only needs to discuss this in case AS layer solution is chosen. Otherwise this can be sorted out by SA4/SA5. We also do not believe that in case the reports are to be stored at AS layer, we can apply any logic for storing the reports based on priorities as RAN/AS layer have no knowledge about this. Application layer solution on the other hand could take this into consideration. For AS layer solution, it may also need to consider whether UE buffer requirement is per UE (a total buffer limit), or per QoE measurement (a buffer limit specific to a QoE measurement), or mixed (a total buffer limit+a specific limit for each QoE measurement). |
| Ericsson |  | Same view as Huawei. |
| Apple | Yes | It would be beneficial to both UE and RAN implementations to know how much storage is needed. Our understanding that QoE report storage is a relatively rare event, and that RAN has little idea how to prioritize different QoE measurement configurations. So we prefer a single total buffer limit. |
| LGE | Yes, only for the AS layer based solution. | In the app layer based solution, the issue should be discussed by SA4/SA5 |
| Lenovo | Yes but | From RAN2 point of view UE buffer requirements are only needed when we go with the AS layer solution. Details (e.g. whether it is a per UE or per QoE configuration requirement) can be discussed further. |
| Nokia, Nokia Shanghai Bell | Yes for AS | One single value for buffer in AS can be discussed |
| ZTE |  | Same view with Huawei |

Rapporteur’s summary: <tbd>

## Other topics

Some more topics were discussed in the contributions [1] to [12] but rapporteur suggests not to treat them in this email discussion because they either better belong to the discussion in AI 8.14.2.1 on configuration architecture general aspects or are of lower priority at this stage. Those topics include amongst other:

- Details of QoE release (either by releasing the QoE measurements configuration or SRB4) as discussed in [8].

- QoE context handling during RRC resume as discussed in [1] and [5].

- Details of start/stop/pause/resume command as discussed in [6], [10] and [11].

- Support of common QoE pause/resume procedure for both signalling based QoE and management based QoE as discussed in [12].

Question 8: Do companies have comments on the topics which were not treated in this email discussion?

| Company | Topic | Comments |
| --- | --- | --- |
| Qualcomm | QoE context handling during RRC resume | QoE context handling during RRC resume is RAN2 topic, should be discussed in RAN2 firstly. For this meeting, we would like to confirm the following scenarios need to be considered: Scenario 1: UE resumes RRC in a gNB not supporting QoE. Scenario 2: UE resumes RRC in a gNB supporting QoE. Scenario 3: UE reusmes RRC in a gNB in case of overload. |
| Huawei, HiSilicon |  | Even though some of these topic definitely fall under RAN2 scope, we understand this offline was supposed to be limited to pause/resume mechanism. |
| Ericsson |  | We can discuss further in the next meeting. Not enough time now. |

Rapporteur’s summary: <tbd>

# Conclusion

Based on company’s feedback the following proposals are made:

<To be updated>