3GPP TSG-RAN WG2 #114-e electronic	R2-210xxxx

Electronic Meeting, 19th – 27th May, 2021

Agenda Item:	8.17.2

Source:	Samsung

Title:	[AT114-e][036][feMIMO] InterCell mTRP and L1/L2 mobility (Samsung)

Document for:	Report and Decision

# 1	Introduction

This contribution summarizes the following discussion:

[AT114-e][036][feMIMO] InterCell mTRP and L1/L2 mobility (Samsung)

Scope: Continue discussion, based on R2-2106314 (and other submitted tdocs if applicable). Try to agree on replies to R1 LS questions. If possible take a step back and elaborate P1 and P2 into agreeable R2 assumptions, identify FFSes, identify questions to ask to RAN1.

Intended outcome: Report, Draft LS out

Deadline: Monday May 24 for on-line CB (if possible, backup time is Thu May 27).

Contact person for each participating company:

| Company | Name | Email Address |
| --- | --- | --- |
| Samsung | Seungri Jin | seungri.jin@samsung.com |
| Qualcomm | Ozcan Ozturk | oozturk@qti.qualcomm.com |
| OPPO | Zhongda Du | duzhongda@oppo.com |
| Lenovo | Lianhai | Wulh5@lenovo.com |
| Ericsson | Pradeepa Ramachandra | pradeepa.ramachandra@ericsson.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |

It is suggested to provide comments before the first recommendated deadline what chairman suggested (Monday May 24 for on-line CB), based on the feedback from companies more time may required to draft LS reply.

# 2	Discussion

During the online session in RAN2#114-e meeting, RAN2 intensively discussed the L1/L2 centric mobility based on the summary of email discussion [1]. Some agreements were made but still there are controversal issues in terms of RAN2 impact on both mTRP-like model and HO-like model.

R2-2106314	Summary of email discussion [Post113bis-e][061][feMIMO] InterCell mTRP and L1/L2 mobility (Samsung)	Samsung	report	NR_feMIMO-Core

DISCUSSION

-	Mediatek think we need to ask R1 some questions, e.g. which configurations of non-serving cells are needed, and e.g. is RACH needed.

-	Xiaomi agres with MTK that multiTRP and mobility scenarios are seemingly similar. If we focus on intra-DU it gets simpler.

-	vivo think R1 need to discuss he scope is it one or two models? Think also that R2 need to clarify the common configurations.

-	Nokia agrees that we should ask some questions. Think that event triggered measurements from R2 can be reused. Ok with Rapporteur proposal.

-	ZTE also think we need to ask questions, e.g. if resources in src cell are released after HO.

-	Oppo think we should understand the model and understand what issues we need to resolve. Oppo think we can also consider that there is only one cell. With two cells the workload gets too high.

-	Apple also think the two models are similar, but think e.g. mTRP assumes overlap of src and TGT cell. Think that if we limit to intra-DU there are no issues with security.

-	Intel think that the current proposal is a good starting point, but we need to clarify things. Think e.g. that it is easier if neighbour cell resource is just an assiting resource.

-	Huawei think P1 as described here is one possibility, but R1 has not decided this (yet)

-	Samsung think this is for common understanding. Agree with Huawei that some contents in P1 is just one possibility, but can generalize when we draft the LS. Samsung agrees with Intel. Think we need to ask R1 whether switching between TRPs can be done.

-	Chair wonder if we have same MAC entity for all TRPs. Intel think yes (for intra-DU).

-	LG think singelprotocol stack is applicable to both models. Think the second model is unclear in R1. Think we should avoid speculating too much on mobility model. Oppo agrees.

-	Ericsson think we have single protocol stack, and scenario is intra-DU. Think the work can start by looking at the common parts.

-	QC think multi-TRP and L1L2 mobility are different, think we need to understand L1L2 mobility. Think we don’t need to discuss the MAC CE details now.

-	FW think that the mobility scenario should be worked on and defined in RAN2. Think there is lots of overlap, can start with multi-TRP.

-	Chair think that we need to understand what would be the objective of HO

P3-P7

-	LG has concerns on multiple C-RNTI. Chair wonder if from R2 perspecitve there is an issue, is this a R1 issue. Xiaomi as well think that multiple CRNTI may cause issues with RACH BFR.

-	ZTE think P3 need to be rephrased.

-	Oppo think P3-P5 there is just a single cell. Chair think the multi-cell is in the WID.

-	Nokia think that if cells is a bad word, then we can use resources

-	FW and Huawei are ok with current proposals knowing that the difference between cells may be just the PCI ..

P4

-	Chair proposes to not agree. Samsung think R1 asked for this. Intel think we can ask R1 is there is any issue.

P1

-	Huawei think we canot agree to P1 as there is too much details.

Chairman:

-	It seems that with the intra-DU restriction everyone assumes a single MAC entity / single L2 protocol stack that can use L1 transmission resources of > 1 cell, both for multi-TRP-scenario and mobility-scenario, and that the protocol stack doesn’t need to be relocated.

-	Such assumption makes it also quite unclear why a mobility / handover fuction is needed, it is needed in order to reconfigure security? In order to change roles of the cells? Do they have differnet capability? (e.g. in terms of which Phy channels are supported).

-	Without clarifying the objective of a mobility function it will not be possible to design one.

RRC provides the configuration for “the cells for L1/L2 centric mobility”, and L1/L2 signaling can be used/feasible for the dynamic usage/switching of the configured value.

R2 didn’t see a problem with using different C-RNTIs for different cells. Different C-RNTI seems more natural in a mobility scenario. No conclusion in R2 for mTRP scenario.

RRC configurations of the cells for L1/L2 centric mobility, including C-RNTI, are configured by RRC.

RAN2 prefer to restrict the scope of the deployment only for intra-DU case in Rel-17.

RAN2 assumes to prioritize intra-frequency case in Rel-17, but RAN2 follows the RAN4 decision to support inter-frequency case.

Use P1 and P2 as baseline for further discussion, aiming to reply to the LS. (P1 seems to be too detailed need generalizing).

In this offline discussion, RAN2 aim to reply to the RAN1 LS [2] questions, and identify questions to ask to RAN1, and iddentify the FFSes (i.e. RAN2 impacts). In addition, elaborate P1 and P2 in R2-2106314 into agreeable R2 assumptions, if possible.

## RAN2 impacts on L1/L2-centric inter-cell mobility

RAN2 considered two main scenarios (i.e. mTRP-like model and HO-like model), and Rapportuer describes the detail procedures for each scenario as below:

- Scenario 1: Inter-cell multi-TRP-like model (i.e. without serving cell change)

- UE receives from serving cell, configuration of SSBs/CSI-RSs of the cell having TRP with different PCI for beam measurement.

- UE performs beam measurement for the cell having TRP with different PCI and report it to serving cell.

- Based on the above reports, TCI state of the cell having TRP with different PCI is activated from the serving cell (by L1/L2 signaling).

- UE starts receiving/transmitting using UE-dedicated channel on the cell having TRP with different PCI.

FFS whether UE switch to the UE-dedicated channel to non-serving cell or UE use both channels (i.e. serving cell and non-serving cell)

- L3 mobility (e.g. HO) is not involved by L1/L2 signaling i.e. independent legacy HO procedure is used.

- Scenario 2: Inter-cell HO-like model (i.e. with serving cell change)

- UE receives from serving cell, configuration of SSBs/CSI-RSs of the cell having TRP with different PCI for beam measurement/ serving cell change.

- UE performs beam measurement for the cell having TRP with different PCI and report it to serving cell.

FFS additional RRM structure e.g. event-triggered reporting or filtering for ensuring robustness of the L1/L2 mobility.

- Based on the above reports, TCI state of the cell having TRP with different PCI is activated along with the serving cell change (by L1/L2 signaling).

- UE changes the serving cell and starts receiving/transmitting using the pre-configured UE-dedicated channel and TCI states.

Rapportuer think it would be beneficial to clealy described both Scenarios

Q1-1: Do you agree the above scenarios 1? If you think further aspects are reuqired please explaine what you understand for scenario 1.

| Company |  | Yes/No |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| Samsung |  | Yes |  | We think it would be better to share the RAN2 understanding to RAN1. FFS points should be checked by RAN1 |  |
| Qualcomm |  | No |  | Our understanding of Scenario 1 may also not be compatible with RAN1. Since there was no question on this scenario in the LS, it is not clear what benefit it will achieve. |  |
| OPPO | No |  | The procedure is described as such that still two different cells are assumed. But we think it is naturally assumed that there is only one cell considering there is only single user plane protocol stack is assumed during online discussion. Based on single cell assumption there is no mobility issue at all and the connection switch between two TRPs with different PCI become purely physical layer procedure with some assistant operation in L2/L3. Here is updated scenario description: - Scenario 1: multi-TRP-like model - UE receives from serving cell, configuration of SSBs/CSI-RSs of the TRP with different PCI for beam measurement. - UE performs beam measurement for the TRP with different PCI and report it to serving cell. - Based on the above reports, TCI state of theTRP with different PCI is activated from the serving cell (by L1/L2 signaling). - UE starts receiving/transmitting using UE-dedicated channel on the TRP with different PCI. |  |
| Lenovo&MM |  | See comments |  | The following questions should be further clarified. - 1st item: RAN1 has not agreed to use CSI-RS for beam measurement purpose yet. - 2nd item: is L1 beam measurement reported by the UE? In addition, UE needs to continue to perform L3 measurement report. - 3rd item: one or multiple TCI state of the cell can be activated? It is RAN1 issue. - 4th item: before transmtting/receiving the data from another cell, whether does UE perform random access for scenario 1? In additon, it is not clear of ‘FFS whether UE switch to the UE-dedicated channel to non-serving cell’? after the non-serving cell is activated, UE switches to the non-serving cell and does not receive/transmit the data from/to the serving cell. |  |
| Ericsson |  | No |  | We propose to not include inter-cell mTRP in this LS reply as the LS sent by RAN1 was clearly about L1/L2 centric inter-cell mobility. Though we support both scenarios, we should not mix them in our LS reply. This is going to cause more confusion. |  |
| Xiaomi |  | Yes, but |  | We think that if we are going to list the RAN2 FFS issues for the better understanding of RAN1, RLM/System information/RACH should be added. |  |

Q1-2: Do you agree the above scenarios 2? If you think further aspects are reuqired, please explaine what you understand for scenario 2.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | We think it would be better to share the RAN2 understanding to RAN1. FFS points should be checked by RAN1 |
| Qualcomm | Yes |  |
| OPPO | No | For this scenario, inter-cell mobility and intra-cell TRP switch assumption can both work. But we still prefer intra-cell assumption due to the fact that inter-cell mobility assumption will result in complicated control plane procedures which can most likely not be finished in time in Rel17. By assuming intra-cell TRP swtich, the main difference between this scenario and mTRP-Like scenario is that UE has to switch TRP while in mTRP-Like scenario UE may or may not switch TRP. Here is updated scenario description: - Scenario 2: HO-like model - UE receives from serving cell, configuration of SSBs/CSI-RSs of the TRP with different PCI for beam measurement. - UE performs beam measurement for the TRP with different PCI and report it to serving cell. . - Based on the above reports, TCI state of the TRP with different PCI is activated along with TRP change (by L1/L2 signaling). - UE changes the TRP and starts receiving/transmitting using the pre-configured UE-dedicated channel and TCI states. |
| Lenovo&MM | See comments | The following questions should be futher clarified. - 1st item: RAN1 has not agreed to use CSI-RS for beam measurement/serving cell change purpose yet. - 2nd item: is L1 beam measurement reported by the UE? In addition, UE needs to continue to perform L3 measurement report. - 3rd item: one or multiple TCI state of the cell can be activated? It is RAN1 issue. - 4th item: When to release the source serving cell? |
| Ericsson | Yes but | In our understanding, the discussions on robustness impact is premature. When the mobility occurs within the DU (both source cell and the traget cell are in the same DU) then pingpong is not a large overhead in terms of signaling as there is no X2/Xn overhead and this does not increase the probability of mobility failure as the DU is aware of the previous optimal beam directions towards the UE and nothing precludes the DU from using this information. Further, the following is not clear to us. FFS additional RRM structure e.g. event-triggered reporting or filtering for ensuring robustness of the L1/L2 mobility. In our understanding event triggered reporting does not improve the robustness. This is mainly for signaling reduction. Regarding the L3 filtered measurements usage for handover decision making by CU-CP; we agree that filtered measurements remove the fast fading impacts and thus more trust worthy but this does not mean that RAN2 has discussed the L1 measuements‘ trustworthiness. Obtaining multiple L1 samples periodically could aid the DU to perform filtering and take the mobility decisions in a robust way. So, we are a bit concerned in the phrasing. We propose the following phrasing. FFS any need of additional RRM structure for L1/L2 mobility if L1/L2 mobility is deemed not robust. |
| Xiaomi | Yes, but | The L3 mobility (i.e. handover) should not be involved by L1/L2 signaling. Othewise it seems there is no benefit to use the L1/L2 signaling for the inter-cell mobility, as the L2 (i.e. RLC/PDCP) reset would anyway introduce interruption like the legacy handover. RLM/RACH/System information should be added as FFS. |

As results of the post email discussion [1], RAN2 collected expected RAN2 impacts on both Scenario 1 and 2, these are summarized as P1 and P2 below based on the input from companies.

Proposal 1: For inter-cell multi-TRP-like model (i.e. without serving cell change), following RAN2 impact can be considered:

- ‘serving cell’ definition update, if UE transmit/receive data to/from more than one PCI

- Addition/release/modification of inter-cell multi-TRP: PxxCH configuration with different TCI states linked to a different PCI than serving cell PCI

- Common configuration of the cells for L1/L2 centric mobility e.g. SSB, paging/SI monitoring, RACH, etc.

- L1 measurement/ report procedures to use the inter-cell multi-TRP

- Introducing the new MAC CE/DCI to start/stop receiving (i.e. TCI state switching) data from/to a cell with different PCI

- RRM/RLM measurement on the cells for L1/L2 centric mobility

- Handling of MAC/RLC/PDCP entities at the change of TRP or TCI state e.g. timing management

Proposal 2: For inter-cell HO-like model (i.e. with serving cell change), following RAN2 impact can be considered:

- Addition/release/modification of the candidate cell(s) for L1/L2 centric inter-cell mobility: contents of what can and needs to be pre-configured: common configurations (e.g. SSB, SI, paging, RACH, etc) and dedicated configurations (e.g. PxxCH configurations, etc)

- Analysis of security of L1/L2 centric inter-cell mobility to avoid attacks causing unnecessary cell changes

- How to ensure reliability and robust for the L1-triggered serving cell change

- Measurement reporting for L1 measurement, and how do RRM/RLM measurements work with L1 triggered serving cell change (e.g. event-triggered reporting, network implementation, etc.)

- Introducing the new MAC CE/DCI to trigger the serving cell change (with TCI state update) from/to a cell with different PCI

- Handling of MAC/RLC/PDCP entities at the change of TRP or TCI state e.g. timing management

- Interaction with existing features e.g. CA/DC, legacy HO mechanism

Q2-1: Do you agree above proposal 1 could be the baseline RAN2 assumption for further work? If not, which part could be updated/removed/added?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Qualcomm | No | We don’t know what RAN1 inter-cell mTRP design yet. |
| OPPO | No | With single cell model, no impact is expected on user plane protocol stacks. And furthermore UE can run common channel operation in both TRPs by monitoring CSS. Here is udpated RAN2 impact with single cell asummption: Proposal 1: For multi-TRP-like model, following RAN2 impact can be considered: - ‘serving cell’ definition update, if UE transmit/receive data to/from more than one PCI - Addition/release/modification of multi-TRP: PxxCH configuration with different TCI states linked to a different PCI - L1 measurement/ report procedures to use the multi-TRP - Introducing the new MAC CE/DCI to start/stop receiving (i.e. TCI state switching) data from/to a TRP with different PCI It is assumed the timing difference between two TRPs is less than one CP. It is up to network’s implemenation to guarantee how to realize it. |
| Lenovo&MM | Yes |  |
| Ericsson | No | We propose to discuss inter-cell mTRP separately compared to L1/L2 centric mobility. They are different WI objectives. Otherwise it causes confusion. |
| Xiaomi | No | Bullet P1-4) is not needed, as L1 measurement/report is in the scope of RAN1. For Bullet P1-7), we could add extra clarification that intra-DU is assumed. |

Q2-2: Do you agree above proposal 2 could be the baseline RAN2 assumption for further work? If not, which part could be updated/removed/added?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Qualcomm | Partially | We should first focus on the essential components within RAN2 scope. Reliability and robustnes are optimizations and can be open ended. Security is also not within RAN2 domain. So at least #2 and #3 are not needed. |
| OPPO | No | With single cell model, no impact is expected on user plane protocol stacks. And furthermore UE can run common channel operation in both TRPs by monitoring CSS. Here is udpated RAN2 impact with single cell asummption: Proposal 2: For HO-like model, following RAN2 impact can be considered: - ‘serving cell’ definition update, if UE transmit/receive data to/from more than one PCI - Addition/release/modification of the candidateTRP dedicated configurations (e.g. PxxCH configurations, etc) - Measurement reporting for L1 measurement - Introducing the new MAC CE/DCI to trigger theTRP change (with TCI state update) from/to a TRP with different PCI - It is assumed the timing difference between two TRPs is less than one CP. It is up to network’s implemenation to guarantee how to realize it. |
| Lenovo&MM | Yes |  |
| Ericsson | Yes but | As we wrote in the comment previously, discussions on robustness is premature at this stage. How the network uses the L1 measurements to trigger mobility is network implementation and already doubting the robustness of such a mobility decision is too early in our opinion. Regarding the MAC/RLC/PDCP handling, it is good if RAN2 can indicate that in the intra-DU mobility scenario, there is no impact on MAC/RLC/PDCP. This is one of the reason for restricting to intra-DU mobility in Rel-17. Therefore, we can either remove this or narrow it down to only address the timing informaiton updation at inter-cell mobility. Regarding the security, in our understanding the model that we are discussing is the one wherein the RRC provides the configurations and they are activated/deactivated via lower layer signaling. |
| Xiaomi | No | For Bullet P2-2, we should mention that SA3 needs to be involved. For Bullet P2-3, L1 relaiblity discussion should be in RAN1. RAN2 can inform RAN1 that the L1 reliabiliy needs to be considered while using the L1 signaling to indicate the cell change. For Bullet P2-4, the first sentence “Measurement reporting for L1 measurement” can be removed, as the L1 measurement and reporting is in the RAN1 scope. For Bullet P2-6, we shoud mention that intra-DU is assumed. |

Q3: Do you agree RAN2 share the RAN2 impact (e.g. P1 and P2) to RAN1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No | Those listed RAN2 impact could be used for the baseline for RAN2 further work. |
| Qualcomm | Mostly No | No need to share anything other than requested in the LS at this point. |
| OPPO | Yes | But with udpated content |
| Lenovo&MM | Yes with comments | Only the agreements having RAN1 impact can be shared with RAN1. |
| Ericsson | No | The listed issues are related to RAN2. |
| Xiaomi | No strong view | No strong view on sharing the RAN2 impacts with RAN1. It seems that RAN2 may not easily agree on all the impacts within the short period. More discussion in the RAN2 is needed. |

## Required questions to ask to RAN1

According to the contributions from companies [4][5][19], there are some unclear aspects to understand the scenarios and work scopes from RAN2 perspective so it is suggested to ask RAN1 for ambiguous aspects directly.

Rapporteur collected some questions from the contributions [4][5][19] as below:

      - Whether RAN1 assume both mTRP-like model and HO-like model as WI scope.

      - For mTRP-like model, whether serving cell TRP is still activated and/or is deactivated when Secondary TRP is activated.

      - For mTRP-like model, how PUSCH/PUCCH in multi-TRP operation is supported especially with activation of downlink channels.

      - For HO-like model, how frequently HO is expected between serving cell and secondary TRP cell.

      - For HO-like model, how downlink measurement and uplink TA are supported for the secondary TRP cell.

      - which measurement events for the additional cell having different PCI than serving cell are supported for HO-like model?

Q4: Do you agree to include above RAN2 questions into the reply LS to RAN1? If you have further comments/question to be asked, please provide it in your comments.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No |  |
| Qualcomm | Mostly no | RAN1 doesn’t know about measurement events in #6. #4 does not impact RAN2 design. As mentioned above, #1, #2, #3 are not relevant until RAN1 completes their work and informs RAN2. Only #5 can give RAN2 some useful information. |
| OPPO | Yes for 1 and no for 2~6 | With the agreed scenario description, RAN2 need to understand whether RAN1 want to proceed with which scenario in Rel17. It would help RAN2 to decide to focus on which scenario. |
| Lenovo&MM | No |  |
| Ericsson | Yes to 5, (May be for 1 as a compromise) | We can include 1) just as an information (if every company wants to include inter-cell mTRP aspects in this reply LS) as this has caused too much confusion in RAN2. In our understanding both these should be considered and they should not be mixed up. The mobility discussion should not involve Scenario-1. We are fine with asking RAN1 about the UL TA assumptions. |
| Xiaomi | Yes | We think that the uplink TA issue is common for both models. We would also like to understand whether the UE is required to have the simulatneous reception or transmission on some PHY channels (e.g. common search space) of different cells, especially for the mTRP-mode. |

Some companies have concerns on the feMIMO TUs in Rel-17, i.e. RAN2 have not enough TUs to investigate both mTRP-liked modle and HO-like model. Especially, HO-like mode requires further study/check to stable operation in terms of security and robustness of mobility.

Q5: Do you agree to provide RAN2 concerns on the TU to RAN1? Especially for HO-like mode requires more effort to support.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | No strong view but slightly prefer | It may efficient for RAN1 to consider the work item scope in Rel-17. We think RAN2 TUs are not enough to support both scenario 1 and . |
| Qualcomm | No | TU allocation is not up to RAN1; it should be discussed at RAN. |
| OPPO | Yes | We’d better provide current RAN2 TU plan to RAN1. Our opinion is that current RAN2 TU allocation (0.5/0.5/0.5/0.5/1) may be able to finish both scenaria with single cell assumption. |
| Lenovo&MM | No strong view | If scenarion#1 and scenario#2 can be merged to one procedure as DAPS handover, this question is not needed. |
| Ericsson | No | Same view as QC. |
| Xiaomi | No | It seems that there are lots of commonalities between the two models. Maybe it is up to RAN1 to decide whether some scenarios can be de-prioritized in Rel-17. |

## Answers on questions from RAN1 in LS [2]

RAN2 made some agreements which will help to make answer on questions from RAN1. Some companies provided the draft LS reply [17][19][20], but it is very difficult to make draft before RAN2 understanding is converged.

So, Rapporteur assume that Q1 and Q2 would be further determined in phase 2 discussion, but the draft response from companies [17][19][20] could be helpful to make answer.

            - Alt1: R2-2105999 [17]

            - Alt2: R2-2106315 [19]

            - Alt3: R2-2106355 [20]

Question 1: In regard of serving cell, - Is there a need for a UE to change a serving cell for DL reception from or UL transmission to another (non-serving) cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH? - If so, how can the addition, release or change of a non-serving cell for DL reception and/or UL transmission be done? For example, would any of such actions require L3 handover and/or selection/activation among pre-configured candidate cells from RAN2 perspective? - If so, how can the TCI states associated with the previous serving cell be handled? - If so, what is the impact on the system information reception by the UE? - If so, what is the impact on the RACH and PUCCH-related procedures and configurations? - If not, what is the impact on the applicable use cases? That is, in what scenarios can the UE be configured for DL reception from or UL transmission to another (non-serving) cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH, if the serving cell does not change?

Question 2: In regard of RRC configuration, RAN1 is discussing whether to allow a UE to be configured for DL reception from or UL transmission to a non-serving cell on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH. From RAN2 perspective - Depending on the answer to question 1-1, what would be the impact of allowing the UE to transmit and/or receive on some or all of those channels and which RRC parameter(s) would need to be reconfigured for the UE? - Is it feasible to update some of the above RRC parameter(s) via dynamic signaling (e.g. MAC CE and/or DCI, potentially selecting pre-configured values) without any additional RRC reconfiguration signaling?

Q6: Do you agree the answers for Q1/Q2 will be treated in phase 2 discussion?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Results of this offline discussion and the answers from the alternatives could be the baseline. |
| Qualcom | Yes |  |
| OPPO | Comment | Only if RAN2 achieved consensus on the modelling issue first |
| Lenovo&MM | Yes |  |
| Ericsson | Yes |  |
| Xiaomi | Yes |  |

Q7: Which one do you prefer as the baseline draft LS? Please provide the good answer if you have for Q1/Q2.

| Company | Alt1/Alt2/Alt3 | Comments |
| --- | --- | --- |
| Samsung | Alt2 | Anyhow, many changes seem required. |
| Qualcomm | Alt3 | R2-2105355 |
| OPPO |  | Without further discussion and conclusion it is difficult to comment |
| Lenovo&MM | see comments | More discussions are required to decide it. |
| Ericsson | Alt1 | We can as well merge the contents of these draft LS replies as some changes are necessary anyway. |
| Xiaomi | Alt2 |  |

Question 3: In regard of C-RNTI: - Is there a need to assign a UE a separate C-RNTI for DL reception from and UL transmission to a non-serving cell, or can the same C-RNTI from the serving cell be reused, at least for transmission and reception on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH? - In restricting the use of the same C-RNTI for serving and non-serving cells, what would be the impact in applicable use cases and/or required specification support, if any? - If separate C-RNTIs are considered necessary in some cases, for serving and non-serving cells, how would this be configured for UE, i.e. is RRC reconfiguration signaling or some other (dynamic) signaling needed for configuring the separate C-RNTI(s)?

Regarding Q3, RAN2 made following agreements during the online session.

RRC provides the configuration for “the cells for L1/L2 centric mobility”, and L1/L2 signaling can be used/feasible for the dynamic usage/switching of the configured value.

R2 didn’t see a problem with using different C-RNTIs for different cells. Different C-RNTI seems more natural in a mobility scenario. No conclusion in R2 for mTRP scenario.

RRC configurations of the cells for L1/L2 centric mobility, including C-RNTI, are configured by RRC.

However, RAN2 did not reached to conclude the restriction on same C-RNTI for mTRP scenario.

Q8: Do you agree to make answers what RAN2 made as it is? i.e. not include the restriction on same C-RNTI for mTRP scenario. Please provide the comments ff you have good suggestion.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Further details could be updated during the phase 2 discussion. |
| Qualcomm | Yes |  |
| OPPO | No | After further discussion we can check whether response LS can already cover everything |
| Lenovo&MM | Yes | We can answer the question from RAN1 based on our agreement. |
| Samsung | Yes | Further details could be updated during the phase 2 discussion. |
| Ericsson | Yes |  |
| Xiaomi | Yes |  |

Question 4: In regard of CU-DU split, from RAN2/3 perspective, is there any difference between supporting intra-DU only and supporting inter- in addition to intra-DU, in terms of the following? - The associated RAN2 specification impact, - Applicable use cases (e.g. deployment scenarios), and - Network inter-operability (e.g. across different gNB vendors)

Regarding Q4, RAN2 made following agreements during the online session.

RAN2 prefer to restrict the scope of the deployment only for intra-DU case in Rel-17.

Q9: Do you agree to make answers what RAN2 made as it is?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Further details could be updated during the phase 2 discussion. |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Lenovo&MM | Yes |  |
| Ericsson | Yes |  |
| Xiaomi | Yes |  |

Question 5: In regard of CA issues, RAN1 is discussing whether the operation is supported only for intra-band CA scenario (i.e. UE is configured to operate with serving and non-serving cells that belong to the same frequency band) or for both intra-band CA and inter-band CA scenarios. Note that one common TCI state ID associated with a non-serving cell, if supported, may be optionally applied for CCs in a band. - Are there specific RAN2/4 issues (including higher-layer impact) that need to be considered for deciding between the two alternatives?

Question 6: In regard of inter-frequency issues, from RAN2/4 perspective, what would be the higher-layer and RRM impact assuming inter-frequency scenarios as opposed to intra-frequency scenarios? For intra-frequency scenario, it is assumed that SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell. - Note: RAN1 has agreed to support intra-frequency scenarios, whereas the support for inter-frequency scenarios is still for further study.

Regarding Q5/Q6, RAN2 made following agreements during the online session.

RAN2 assumes to prioritize intra-frequency case in Rel-17, but RAN2 follows the RAN4 decision to support inter-frequency case.

Q10: Do you agree to make answers what RAN2 made as it is?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | Further details could be updated during the phase 2 discussion. RAN2 can provide RAN2 impact on inter-frequency case (e.g. measurement gap). |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Lenovo&MM | Yes |  |
| Ericsson | Yes |  |
| Xiaomi | Yes |  |

Q11: Any further comments on drafting LS reply?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | This comment is applicable only if RAN2 decides that the reply LS should include inter-cell mTRP aspects as well. In view of the confusion caused by the LS in RAN2, we could request RAN1 to send separate LSs regarding inter-cell mTRP and inter-cell mobility related questions. If they want to send a single LS, then we could request them to explicitly mention which question refers to inter-cell mTRP, which question refers to inter-cell mobility and which question refers to both. |

# 3	Conclusion

To be updated.

# 4	References

- R2-2106314	Summary of email discussion [Post113bis-e][061][feMIMO] InterCell mTRP and L1/L2 mobility (Samsung)	Samsung	report	NR_feMIMO-Core

- R2-2102627	LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility	RAN1

- R2-2104908	Discussion on L1 L2-Centric Inter-Cell Mobility	vivo	discussion	Rel-17	NR_feMIMO-Core	R2-2102855

- R2-2104988	Multi-cell support for multi-TRPand L1 mobility	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_feMIMO-Core

- R2-2105026	Further aspects on L1/L2-Centric Inter-Cell Mobility	Intel Corporation	discussion	Rel-17	NR_feMIMO

- R2-2105033	Discussion on L1/2 centric mobility	OPPO	discussion	Rel-17	NR_feMIMO-Core

- R2-2105103	L1/L2-centric inter-cell mobility	Apple	discussion	Rel-17	NR_feMIMO-Core

- R2-2105294	Procedures of L1L2-Centric Inter-Cell Mobility	MediaTek Inc.	discussion

- R2-2105354	L1/L2 Mobility Overview	Qualcomm Incorporated	discussion

- R2-2105621	On Scenarios for L1 L2 mobility for FeMIMO	CATT	discussion	Rel-17	NR_feMIMO-Core

- R2-2105622	Discussions on L1 L2 mobility for FeMIMO without serving cell change	CATT	discussion	Rel-17	NR_feMIMO-Core

- R2-2105826	Discussion on the support of inter-cell multi-TRP operation	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2105827	Discussion on the support of L1/L2 centric inter-cell mobility	Lenovo, Motorola Mobility	discussion	Rel-17

- R2-2105857	Consideration on the L1L2 centric mobility	ZTE, Sanechips	discussion	Rel-17	NR_feMIMO-Core

- R2-2105991	L1/L2 centric-mobility: Multi-TRP	Huawei, HiSilicon	discussion	Rel-17	NR_feMIMO-Core

- R2-2105992	Handover-like mechanism for L1/L2-centric inter-cell mobility	Huawei, HiSilicon	discussion	Rel-17	NR_feMIMO-Core

- R2-2105999	On L1/L2 centric inter-cell mobility	Ericsson	discussion

- R2-2106295	Potential RAN2 work for feMIMO	LG Electronics	discussion	Rel-17

- R2-2106315	DRAFT LS Reply on TCI State Update for L1/L2-Centric Inter-Cell Mobility	Samsung	LS out	NR_feMIMO-Core	To:RAN1	Cc:RAN3, RAN4

- R2-2105355	Responses to RAN1 LS for L1/L2 Mobility	Qualcomm Incorporated	discussion