3GPP TSG-RAN WG2 Meeting #114 Electronic	R2-2106502

Elbonia, 19 – 27 May 2021

Agenda item:	8.3.2

Source:	vivo (Rapporteur)

Title:	[AT114-e][240][Multi-SIM] UE assistance information of paging collision (vivo)

WID:	LTE_NR_MUSIM-Core

Document for:	Discussion and Decision

Introduction

This document aims to collect views from companies for the following discussion agreed during RAN2#114e:

[AT114-e][240][Multi-SIM] UE assistance information of paging collision (vivo)

Scope:

    - Discuss whether and which UE assistance information is needed for avoiding paging collision in MUSIM.

    - Should explain what happens if 1) if no assistance information is provided and 2) if assistance information is provided.

Intended outcome:

    - Discussion summary in R2-2106502 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  2nd week Tue, UTC 1200

    - Initial deadline (for rapporteur's summary):  2nd week Wed, UTC 1200

To make it easier to find the correct contact delegate in each company for potential follow-up questions, the rapporteur encourages the delegates who provide input to provide their contact information in this table:

| Company | Contact: Name (E-mail) |
| --- | --- |

Discussion

During online discussion, 15 companies support that UE can indicate the assistant information (AS or NAS) to network for paging collision avoidance, while 9 companies against this assistant information. According to the contributions submitted in this meeting, the benefit of providing UE assistant information include:

- UE has the best knowledge to determine the best offset for avoiding paging collision by considering RAN parameters in this network and the POs in the other attached network(s), and thereby reduce the number of requests to resolve the PO collision.

- UE can provide preferred value for better power saving, e.g., simultaneously paging monitoring for dual RX multi-USIM device;

While the reasons for doubting the necessary of UE assistant information are given as follows:

- Paging collision is a very low probability issue. If the old 5G-S-TMSI causes collision, then in principle a new 5G-S-TMSI will avoid the collision at least in the current cell. If it happens, the UE can always request a further reassignment or solve it via UE or NW implementation.

- paging collision can be solved without assistance information for that the PO is periodically distributed and the possible paging cycle is specified to be {rf32, rf64, rf128, rf256}.

- UE assistance information may force network to choose a right 5G-GUTI value and avoid paging collision. If it works like other assistance information, it is likely that the network still free to have its own assignment. So, the additional information could not guarantee to solve the problem.

RAN2 has agreed that NAS signalling is baseline for UE reporting paging collision issue in 5GS side. So, the rapporteur understands that it is more reasonable for the UE to send assistant information to CN via NAS signalling. However, there is one contribution [8] propose that for RRC inactive UE can provide RRC assistant information to RAN for avoiding collision in RRC_INACTIVE state. The RRC assistance information can indicate the RAN paging cycle to be selected to avoid the collision in RRC_INACTIVE state. The RRC assistance information may also indicate offset to be applied on paging occasion calculation for RRC_INACTIVE. So, the rapporteur would like to separate the different cases (NAS assistant information NAS or RRC assistant information) to discuss this issue.

Now, companies are kindly invited to answer the below question, with providing your technical reasons to support/against UE assistant information for paging collision avoidance(e.g., what happens if UE assistant information is provided or not). Hopefully, these technical reasons can help us to reach some consensus or go in a direction with clear majority support.

Do you think whether and which signalling shall be supported for sending NAS assistant information?

| Company | Need assistant information or not | Which UE assistance information (e.g., UE_ID offset) if need | Technical reasons |
| --- | --- | --- | --- |

Summary:

TBD.

Do you think whether and which signalling shall be supported for sending RRC assistant information?

| Company | Need assistant information or not | Which UE assistance information (e.g., UE_ID offset) if need | Technical reasons |
| --- | --- | --- | --- |

Companies are invited to express their view if any other overall comments or suggestions.

Any other comments or suggestions?

| Company | Comments |
| --- | --- |

Summary:

TBD.

Conclusions

Based on the email discussion, we give the below proposals.

TBD

References

- R2-2104764	Paging Collision Avoidance	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2104970	Paging collision avoidance for MUSIM device	Asia Pacific Telecom, FGI	discussion

- R2-2104991	On Paging Collision Avoidance Solution	Samsung	discussion

- R2-2105075	Definition and solution for paging collision, RRC Inactive, SI change	Lenovo, Motorola Mobility	discussion	LTE_NR_MUSIM-Core

- R2-2105084	MUSIM Page Collision Avoidance	Apple	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2105164	Consideration on the Paging Collision	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2105194	Further Consideration on Paging Collision Avoidance	CATT	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2105227	RAN Impacts for paging collision avoidance solutions for Multi-SIM	Nokia, Nokia Shanghai Bells	discussion	Rel-17

- R2-2105258	Options for paging collision avoidance	Qualcomm Incorporated	discussion

- R2-2105269	Paging Collision avoidance	vivo	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2105374	UE indication of paging collision for Multi-SIM	ASUSTeK	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2105682	Discussion on paging collision avoidance in Multi-SIM	Sony	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2105899	Paging Collision Avoidance for Multi-SIM	Charter Communications, Inc	discussion

- R2-2105917	Paging Collision Avoidance Open Issues	Huawei, HiSilicon	discussion	Rel-17

- R2-2105978	Paging collision avoidance	Ericsson	discussion

- R2-2106101	36.304 change for SA2 agreed NAS based IMSI offset signaling in EPS	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2106102	5G-S-TMSI re-assignment is “enough” for paging collision avoidance in 5GS	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

- R2-2106109	Considerations on Paging Collision	LG Electronics	discussion	Rel-17	LTE_NR_MUSIM-Core	R2-2103572

- R2-2106343	Paging collision avoidance for MUSIM device	MediaTek Inc.	discussion	LTE_NR_MUSIM-Core	R2-2104151

- R2-2106398	Discussion of the paging collision problem in 5GS	Xiaomi Communications	discussion