3GPP TSG-RAN WG2 Meeting #114-electronic                                            	      R2-2106559

Online, May 19th – May 27th, 2021

Title:	[Draft] Reply LS to RAN1 on physical layer aspects of small data transmission

Response to:	R1-2102125

Release:	Release 17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	vivo [to be RAN2]

To:	RAN1

Contact Person:

#### Name:	Yitao Mo

#### E-mail Address:	yitao.mo@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

# 1. Overall Description:

RAN2 would like to thank RAN1 for their reply LS on physical layer aspects of small data transmission.

RAN2 has concluded that new common search space is supported for RA-SDT. The relevant agreement was made as follows,

Agreement: - The separate search space is common to the UEs performing RA-SDT. Inform RAN1 of this agreement

Furthermore, RAN2 discussed the details of both RA-SDT and CG-SDT leading to the following agreements with potential RAN1 impacts.

Agreement: For RA-SDT - CFRA is not supported for RA-SDT For CG-SDT - Working assumption: UE-specific search space is configured for UEs performing CG-SDT. RAN2 asks RAN1 whether this working assumption can be confirmed - CG-SDT resource can be configured on either initial BWP or separate SDT BWP. Ask RAN1 to confirm

For RA-SDT, in particular, RAN2 would like to request RAN1 to provide configuration paramters for the PRACH resource configuration when PRACH occasions are shared between SDT and non-SDT and when PRACH occasions are separately configured for SDT and non-SDT (for both 2-step RA and 4-step RA).

For CG-SDT, RAN2 respectfully requests RAN1 if the agreements 3/4 mentioned above can be confirmed.

# 2. Actions:

RAN2 respectfully asks RAN1 to provide feedback on the physical layer aspects of small data transmission as requested above.

# 3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #115-e		        16th – 27th August 2021		                E-meeting

TSG-RAN WG2 Meeting #116-e		        1st – 12th Novemebr 2021		        E-meeting