3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2106009

Online, May 19th – 27th, 2021                                               Resubmission of R2-2103200

Agenda item:		8.1.1

Source:		Futurewei

Title:		Protocol Architecture of MRB with Dynamic PTM/PTP Switch

Document for:		Discussion and Decision

# Introduction

The following aspects were discussed in RAN2#113e [1]:

The main two points that seems to need resolution/consolidation are the following

A.	L2 ARQ for PTM for normal data transfer

B.	Which layer anchors the PTM PTP switch, i.e. at PTM PTP switch which layer remains the same, (and might be responsible for service continuity).

Both point A and B are included here because several companies indicate that they are inter-dependent,

For A. there seems to be the following options on the table:

A1. No L2 ARQ for PTM

A2. L2 ARQ by PDCP for PTM

A3. L2 ARQ by RLC-AM for PTM

For B. There seems to be the following options on the table:

B1. PDCP anchored PTM/PTP switch

B2. RLC anchored PTM/PTP Switch

RAN2#113e Agreement and Chairman Notes:

For the case that both PTM and PTP are RLC-UM, configuration with No L2 ARQ and with PDCP anchored PTM – PTP switching shall be supported (e.g. for services that would typically be configured with RLC UM for unicast).

Chair understanding is that actually all the proposals on the table could support the high reliability requirement. Indeed RLC AM proposal would be expected to be the most efficient by using PTM to greater extent (at least greater than A1), and by retransmitting segments. TO be continued at a later meeting.

It was also agreed in RAN2#113bis-e that “For a given UE, if the MRB’s QoS requirements are not met via PTM, switching to PTP with RLC-AM shall be supported.”

# Discussion

Split bearer design was originated from DC framework, where the two RLC entities can reside on different (e/g)NBs, and tight coordination (which is only possible through ideal backhaul) is not required. Dynamic switch between PTM and PTP occurs in the same cell, and it is feasible to have certain cooperation between the two transmitting RLC entities. Hence, logical channel aggregation, as depicted in Figures 2 and 3, can be considered to reduce UE complexity and improve transmission efficiency in the split bearer like protocol framework for dynamic switch of PTM and PTP transmissions.

Figure 2: Logical Channel Aggregation: UM PTM + AM PTP

In Figure 2, a UE is configured/directed to process RLC PDUs received from logical channels MTCH and DTCH by a single RLC receiving entity associated with the DTCH. RLC PDUs transmitted on logical channels MTCH and DTCH are generated at a cell by separate RLC transmitting entities of logical channels MTCH and DTCH, respectively. After transmission, RLC PDUs of the logical channel MTCH are stored in the retransmission buffer of the RLC transmitting entity associated with the logical channel DTCH. RLC status report generated by the receiving RLC entity of the logical channel DTCH, after processing packets received from both logical channels MTCH and DTCH, is sent over UL to the RLC transmitting entity of the logical channel DTCH at the network. The RLC status report includes the reception status of the RLC PDUs generated by the RLC transmitting entities of both logical channels MTCH and DTCH. RLC control in the RLC transmitting entity of the logical channel DTCH manages the retransmission of packets stored in the retransmission buffer based on the received RLC status report.

Cooperation is needed between transmitting entities of logical channels MTCH and DTCH in assigning RLC SN. For every RLC SDU, RLC SN needs to be included in every RLC PDU of the logical channel which is aggregated. In Figure 2, RLC SN is included in RLC PDUs of the logical channels MTCH and DTCH, when they are generated at the RLC transmitting entities at a cell. That is, an AM like PDU format (with SN for non-segmented SDU) should be used for UM MTCH as well. More generally speaking, RLC PDUs of aggregated logical channels should use the same RLC PDU format. And RLC SN are assigned jointly in the RLC transmitting entities of the aggregated logical channels.

Figure 3 shows logical channel aggregation of MTCH and DTCH, which are both in AM mode. In Figure 2, retransmission of an RLC SDU can be conducted only over logical channel DTCH. In Figure 3, RLC status report from DTCH can be shared between the RLC transmitting entities of MTCH and DTCH, and retransmission of an RLC SDU can be done flexibly over either MTCH or DTCH, on a segment by segment decision based on the need of  retransmission of a segment of RLC SDU either individually to a UE through DTCH or collectively to a group of UEs through MTCH.

Figure 3: Logical Channel Aggregation: AM PTM + AM PTP

Observation 1: Logical channel aggregation reduces UE complexity in supporting dynamic switch between PTM and PTP transmissions, as only one RLC receiving entity is needed.

Observation 2: Logical channel aggregation improves transmission efficiency in PTM and PTP connections, as there is no need of duplicate transmission over two RLC transmitting entities for independent ARQ operations.

Proposal: Logical channel aggregation is applied to the split bearer like framework for dynamic switch between PTM and PTP transmissions.

# Conclusions

Considering that dynamic switch between PTM and PTP transmissions occurs in the same cell, this paper discusses the use of logical channel aggregation in the split bearer like protocol framework, which leads to the following observations and proposal:

Observation 1: Logical channel aggregation reduces UE complexity in supporting dynamic switch between PTM and PTP transmissions, as only one RLC receiving entity is needed.

Observation 2: Logical channel aggregation improves transmission efficiency in PTM and PTP connections, as there is no need of duplicate transmission over two RLC transmitting entities for independent ARQ operations.

Proposal: Logical channel aggregation is applied to the split bearer like framework for dynamic switch between PTM and PTP transmissions.

# References

R2-2102313, [AT113-e][038][MBS] UP architecture decisions.