3GPP TSG-RAN WG2 Meeting #114-e	R2-2106027

# Electronic Meeting, 19th – 27th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.340 | CR | 0018 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the handling of unknown, unforeseen, and erroneous protocol data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to Rel.16 BAP specification, if an IAB node receives a BAP PDU with a BAP header containing reserved or invalid values, it will discard the BAP PDU. In case RAN2 decides in a future release to support new features that imply enhancements to the BAP header, it might happen that a “new” IAB node uses such new features that are not supported by a peer receiving “old” IAB node. The “old” IAB node would consider the BAP header of the received BAP packet as containing reserved or invalid values. Hence, upon reception, the “old” IAB node would discard such BAP PDU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The specification is changed such that IAB node does not discard a received BAP PDU if it contains unknown/reserved values, i.e. it ignores the reserved or invalid values included in the BAP header, unless the IAB node is an IAB access node for the BAP PDU. Impact analysis Impacted functionality: Handling of unknown, unforeseen, and erroneous protocol data Inter-operability: If the network is implemented according to the CR while the IAB-MT is not, there is no interoperability issue, since the CR only affects the IAB-MT receptions functionalities. If the IAB-MT is implemented according to the CR while the network is not, there is no interoperability issue, since the CR only affects the IAB-MT receptions functionalities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If in future releases the BAP header is enhanced to support new features, a Rel.16 IAB node would always discard a received BAP PDU having a BAP header containing reserved or invalid values. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

## 5.5	Handling of unknown, unforeseen, and erroneous protocol data

When a BAP PDU that contains a BAP address which is not included in the configured BH Routing Configuration and is not the BAP address of this node is received; or when a BAP PDU that contains reserved or invalid values and that contains a BAP address which is the BAP address of this node is received, the BAP entity shall:

-	discard the received BAP PDU.

When a BAP PDU that contains reserved or invalid values is received, the BAP entity shall:

-	ignore the reserved or invalid values and process the received BAP PDU as in clause 5.2.2.

END OF CHANGE