3GPP TSG-RAN WG2 Meeting #114	R2-2106414

Electronic, 19th May – 27th May, 2021

Agenda item:	5.4.1.1

Source: 	Huawei, HiSilicon

Title: 	Clarification on leftover issues for NR-DC

Document for:	Discussion and Decision

- Introduction

In RAN2 #113bis-e meeting, some issues were raised for NR-DC clarification and non-consensus was achieved during the meeting. Therefore, they were postponed to this RAN2 meeting.

[005] Postponed discussion: whether in the case of HO without SCG change, if SCG reconfigurationWithSync is not included, the UE continues the transmission on SG during the handover or not or whether this can be left to UE implementation, and whether there is a need for TS clarification.

[005] Postponed: CRs for UE timing at NR-DC handover. Majority view seems to be that UE should apply the target PCell timing as the PSCell SMTC timing reference during the NR-DC handover

In this contribution, we will provide our understanding and related proposals and CRs will be provided.

- Discussion

- Issue 1: UE handling in case of HO without SCG change if SCG reconfigurationWithSync is not included

In TS 38.331 v15.13.0, the condition of reconfigurationWithSync is specified as following:

| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup at change of AS security key derived from KgNB, - in the secondaryCellGroup at: - PSCell addition, - update of required SI for PSCell, - change of AS security key derived from S-KgNB while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message. Otherwise it is optionally present, need M. The field is absent in RRCResume or RRCSetup messages. |
| --- | --- |

As listed above, the conditions where the SCG reconfigurationWithSync field should be mandatory present are provided. Excluding the mandatory present cases, one typical scenario where SCG reconfigurationWithSync is not included in case of HO without SCG change should be that the intra-CU handover without SCG change. In addition, this KgNB is unchanged, which is allowed according to NR security framework.

Observation 1: In case of intra-CU handover without SCG change, the SCG reconfigurationWithSync is not included and the KgNB is unchanged.

Since the KgNB is unchanged, SCG transmission for MN terminated bearer can be performed without interruption. Similarly, since the KgNB is unchanged, the S-KgNB is not updated as well. Therefore, the SCG transmission for SN terminated bearer can be performed without interruption.

Observation 2: In case the KgNB is unchanged, the SCG transmission for both MN terminated bearer and SN terminated bearer can be performed without interruption during the HO without SCG change.

In NR RRC framework, the UE L2 handling during handover procedure is totally indicated by dedicated signalling from the network, instead of following the specified procedure as captured in LTE RRC specification. In this case, since the network does not signal any indication for PDCP/RLC/MAC handling, it is quite straightforward that the UE continues the SCG transmission during the handover without SCG change if SCG reconfigurationWithSync is not included.

Proposal 1: The UE can continue SCG transmission during handover without SCG change if SCG reconfigurationWithSync is not included.

- Issue 2: PSCell SMTC timing reference during the NR-DC handover

In EN-DC, considering different UE implementations at that moment, only a NOTE was added to clarify that explicit SMTC configuration is only supported when the source EUTRA PCell and target EUTRA PCell of the handover are SNF/subframe-synchronized.

TS 36.331 NOTE 2: It is not specified whether the timing reference for the SMTC configuration is the source EUTRA PCell or the target EUTRA PCell in case the NR PSCell addition or SN change takes place simultaneously with handover. As a consequence, explicit SMTC configuration is only supported when the source EUTRA PCell and the target EUTRA PCell of the handover are SFN/subframe-synchronized.

In 38.331, for MR-DC, the SMTC configuration is provided in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync, and it was specified in v15.10.0:

ReconfigurationWithSync ::= SEQUENCE { spCellConfigCommon ServingCellConfigCommon OPTIONAL, -- Need M newUE-Identity RNTI-Value, t304 ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000}, rach-ConfigDedicated CHOICE { uplink RACH-ConfigDedicated, supplementaryUplink RACH-ConfigDedicated } OPTIONAL, -- Need N ..., [[ smtc SSB-MTC OPTIONAL -- Need S ]] }

In RAN2#110e, for NR-DC, it was discussed how to provide SMTC configuration based on [2], and it was agreed:

Option 2 for PSCell addition for R15 Solution for R16 postponed If smtc configuration is absent for PSCell addition, the UE uses the smtc in measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

And option 2 was captured into TS 38.331 v15.11.0.

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change, NR PCell change, and NR PSCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change and NR PSCell addition, the smtc is based on the timing reference of source PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

The case of PSCell addition was addressed for Rel-15, but the case of PSCell change (SN change) was not concluded.

Observation 3: It was concluded that for Rel-15 NR-DC, SMTC configuration for PSCell upon PSCell addition was concluded, i.e. based on NR PCell. But the case of PSCell change (SN change) was not concluded for NR-DC for Rel-15.

In RAN2#111e meeting, for Rel-16 NR-DC, the same issue was discussed and RAN2 agreed:

R2-2007117 SMTC Configuration for PSCell Addition and SN Change in NR-DC Apple, MediaTek Inc., Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, ZTE Corporation, Sanechips, CATT discussion Rel-16 NR_newRAT-Core [040] Noted, Agree Option 1 for SMTC configuration in R16 for PSCell Addition and SN Change in NR-DC. (i.e. add the nfew parameter for SMTC configuration under RRCReconfiguration for PSCell addition and SN change)

A new SMTC configuration has been added for Rel-16 NR-DC:

Observation 4: For Rel-16 NR-DC, a new SMTC configuration has been agreed for both PSCell addition and PSCell change (SN change) cases.

For NR-DC, in RAN2#113bis meeting, majority view is that the UE should apply the target PCell timing as the PSCell SMTC timing reference during the NR handover with PSCell addition and PSCell change (SN change). It is proposed to specify the target PCell timing as the timing reference for PSCell SMTC upon PSCell addition/change for Rel-15 NR-DC.

Proposal 2: The UE should apply the target PCell timing as the PSCell SMTC timing reference during the NR handover with PSCell addition and PSCell change (including SN change).

- Conclusion

In this contribution, we have a discussion on remaining issues of NR-DC and the following observations and proposals were provided.

Observation 1: In case of intra-CU handover without SCG change, the SCG reconfigurationWithSync is not included and the KgNB is unchanged.

Observation 2: In case the KgNB is unchanged, the SCG transmission for both MN terminated bearer and SN terminated bearer can be performed without interruption during the HO without SCG change.

Observation 3: It was concluded that for Rel-15 NR-DC, SMTC configuration for PSCell upon PSCell addition was concluded, i.e. based on NR PCell. But the case of PSCell change (SN change) was not concluded for NR-DC for Rel-15.

Observation 4: For Rel-16 NR-DC, a new SMTC configuration has been agreed for both PSCell addition and PSCell change (SN change) cases.

Proposal 1: The UE can continue SCG transmission during handover without SCG change if SCG reconfigurationWithSync is not included.

Proposal 2: The UE should apply the target PCell timing as the PSCell SMTC timing reference during the NR handover with PSCell addition and PSCell change (including SN change).

- Reference

R2-2104633, Summary [AT113bis-e][005][NR15] Connection Control I, offline discussion rapporteur (ZTE Corporation).

R2-2006186	Summary of [AT110e][007][NR15] DC Configuration (Apple)	Apple	discussion	NR_HST