3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2107137

Online, Aug 16 – Aug 27, 2021

Source:	CATT, CAICT

Title:	Summary of Introduction of B3I signal in BDS system

Agenda Item:	8.11.6

Document for:	Discussion and Decision

- Introduction

China Satellite Navigation Office published “Beidou Navigation Satellite System Signal in Space Interface Control Document—Open Service Signal B3I (Version 1.0)” [1] in December, 2017. http://www.beidou.gov.cn/xt/gfxz/201802/P020180209623601401189.pdf

The B3I signal is transmitted by the BDS-2 and BDS-3 satellites including Medium Earth Orbit (MEO) satellites, Inclined Geo Synchronous Orbit (IGSO) satellites and Geostationary Earth Orbit (GEO) satellites for providing open services.

So RP-210903 NR Positioning Enhancements [2] was discussed in 3GPP TSG RAN meeting #91e. Approved the decision to specify the support for the BDS B3I signal in the BDS system..

- Impact analysis

  - Summary of impact on introduction of signal B3I

The impacted assistance data of B3I signal in the network-assisted BDS should be introduced as a part of A-GNSS positioning methods in both LTE and NR.

Network-assisted B3I of BDS system positioning method can reuse the procedures of network-assisted BDS positioning method.

The impact of B3I signal in BDS system on Stage-2 and Stage-3 for LTE and NR is summarized here:

- Impact of B3I signal in BDS on Stage-2 36.305 for LTE

- BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I as the reference file [1] should be involved in this spec.

- Impact of B3I signal in BDS on Stage-2 38.305 for NR

- BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I as the reference file [1] should be involved in this spec.

- Impact of B3I signal in BDS on Stage-3 37.355 for LTE and NR

  - BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I as the reference file [1] is added into section 2 as reference.

  - All impacted IEs have been pointed out and the main parts have been changed for introduction of B3I signal in 6.5.2.

Table 1   Summary of impact on Specs

| TS 36.305 | TS38.305 | TS 37.355 |
| --- | --- | --- |
| The reference file should be added in section 2 References | The reference file should be added in section 2 References | The reference file should be added in section 2 References |
| The reference file [1] should be mentioned in section 8.1 GNSS positioning methods. | The reference file [1] should be mentioned in section 8.1 GNSS positioning methods. | All impacted IEs have been pointed out and the main parts have been changed for introduction of B3I signal in 6.5.2 of TS 37.355. |

  - Impact analysis on 36.305

Network-assisted B3I of BDS positioning method can reuse the procedures of network-assisted BDS positioning method. So the impact on 36.305 is about the B3I BDS ICD file as reference.

- BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (ICD) should be added in section 2 References.

- The B3I BDS ICD file [1] should be mentioned in section 8.1 GNSS positioning methods.

  - Impact analysis on 38.305

The impact on 38.305 is the same as on 36.305 above.

  - Impact analysis on 37.355

The current working procedures can be reused for B3I signal in BDS, but new BDS-ID, some necessary information and related description shall be introduced.

    - GNSS Assistance Data (Section 6.5.2.1)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideAssistanceData | No impact |
| GNSS-CommonAssistData | No impact |
| GNSS-GenericAssistData | No impact |
| GNSS-PeriodicAssistData | No impact |

    - GNSS Assistance Data Elements (Section 6.5.2.2)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-ReferenceTime | No impact |
| GNSS-SystemTime | No impact |
| GPS-TOW-Assist | No impact. |
| NetworkTime | No impact |
| GNSS-ReferenceLocation | No impact |
| GNSS-IonosphericModel | No impact |
| KlobucharModelParameter | KlobucharModelParameter can be reused for BDS B3I. The reference ICD file of B3I BDS shall be added in description. |
| KlobucharModel2Parameter | No impact |
| NeQuickModelParameter | No impact |
| GNSS-EarthOrientationParameters | No impact |
| GNSS-RTK-ReferenceStationInfo | No impact |
| GNSS-RTK-CommonObservationInfo | No impact. |
| GNSS-RTK-AuxiliaryStationData | No impact |
| GNSS-TimeModelList | No impact |
| GNSS-DifferentialCorrections | No impact. |
| GNSS-NavigationModel > NavModelSatelliteList >>GNSS- NavModelSatelliteElement >>> svHealth >>> iod | - B3I health state is added in ‘GNSS to svHealth Bit String(8) relation’ table. - IOD of B3I is updated in ‘GNSS to iod Bit String(11) relation’ table. |
| StandardClockModelList | No impact |
| NAV-ClockModel | No impact |
| CNAV-ClockModel | No impact |
| GLONASS-ClockModel | No impact |
| SBAS-ClockModel | No impact |
| BDS-ClockModel | BDS-ClockModel can be reused for BDS B3I. The reference ICD file of B3I BDS shall be added in description. |
| BDS-ClockModel2 | No impact |
| NavIC-ClockModel | No impact |
| NavModelKeplerianSet | No impact |
| NavModelNAV-KeplerianSet | No impact |
| NavModelCNAV-KeplerianSet | No impact |
| NavModel-GLONASS-ECEF | No impact |
| NavModel-SBAS-ECEF | No impact |
| NavModel-BDS-KeplerianSet | NavModel-BDS-KeplerianSet can be reused for BDS B3I. The reference ICD file of B3I BDS shall be added in description. |
| NavModel-BDS-KeplerianSet2 | No impact |
| NavModel-NavIC-KeplerianSet | No impact |
| GNSS-RealTimeIntegrity | No impact |
| GNSS-DataBitAssistance > GNSS-DataBitsSatList >> GNSS-DataBitsSatElement >>> GNSS-DataBitsSgnList >>>> GNSS-DataBitsSgnElement >>>>> gnss-DataBits | gnss-DataBits of B3I is updated in ‘GNSS-DataBitAssistance fied descripeions’ table. |
| GNSS-AcquisitionAssistance | No impact |
| GNSS-Almanac | No impact |
| AlmanacKeplerianSet | No impact |
| AlmanacNAV-KeplerianSet | No impact |
| AlmanacReducedKeplerianSet | No impact |
| AlmanacMidiAlmanacSet | No impact |
| AlmanacGLONASS-AlmanacSet | No impact |
| AlmanacECEF-SBAS-AlmanacSet | No impact |
| AlmanacBDS-AlmanacSet | No impact |
| AlmanacNavIC-AlmanacSet | No impact |
| GNSS-UTC-Model | No impact |
| UTC-ModelSet1 | No impact |
| UTC-ModelSet2 | No impact |
| UTC-ModelSet3 | No impact |
| UTC-ModelSet4 | No impact |
| UTC-ModelSet5 | No impact |
| GNSS-AuxiliaryInformation | No impact |
| BDS-DifferentialCorrections | BDS-DifferentialCorrections can be reused for BDS B3I. The reference ICD file of B3I BDS shall be added in description. |
| BDS-GridModelParameter | BDS-GridModelParameter can be reused for BDS B3I. The reference ICD file of B3I BDS shall be added in description. |
| GNSS-RTK-Observations | No impact |
| GLO-RTK-BiasInformation | No impact |
| GNSS-RTK-MAC-CorrectionDifferences | No impact |
| GNSS-RTK-Residuals | No impact |
| GNSS-RTK-FKP-Gradients | No impact |
| GNSS-SSR-OrbitCorrections | No impact |
| GNSS-SSR-ClockCorrections | No impact |
| GNSS-SSR-CodeBias | No impact |
| GNSS-SSR-URA | No impact |
| GNSS-SSR-PhaseBias | No impact |
| GNSS-SSR-STEC-Correctio | No impact |
| GNSS-SSR-GriddedCorrection | No impact |
| NavIC-DifferentialCorrections | No impact |
| NavIC-GridModelParameter | No impact |

    - GNSS Assistance Data Request (Section 6.5.2.3)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestAssistanceData | No impact |
| GNSS-CommonAssistDataReq | No impact |
| GNSS-GenericAssistDataReq | No impact |
| GNSS-PeriodicAssistDataReq | No impact |

    - GNSS Assistance Data Request Elements (Section 6.5.2.4)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-ReferenceTimeReq | No impact |
| GNSS-ReferenceLocationReq | No impact |
| GNSS-IonosphericModelReq | No impact |
| GNSS-EarthOrientationParametersReq | No impact |
| GNSS-RTK-ReferenceStationInfoReq | No impact |
| GNSS-RTK-AuxiliaryStationDataReq | No impact |
| GNSS-SSR-CorrectionPointsReq | No impact |
| GNSS-TimeModelListReq | No impact |
| GNSS-DifferentialCorrectionsReq | GNSS-DifferentialCorrectionsReq can be reused for BDS B3I. BDS B3I signal shall be added in description. |
| GNSS-NavigationModelReq | No impact |
| GNSS-RealTimeIntegrityReq | No impact |
| GNSS-DataBitAssistanceReq | No impact |
| GNSS-AcquisitionAssistanceReq | No impact |
| GNSS-AlmanacReq | No impact |
| GNSS-UTC-ModelReq | No impact |
| GNSS-AuxiliaryInformationReq | No impact |
| BDS-DifferentialCorrectionsReq | No impact |
| BDS-GridModelReq | No impact |
| GNSS-RTK-ObservationsReq | No impact |
| GLO-RTK-BiasInformationReq | No impact |
| GNSS-RTK-MAC-CorrectionDifferencesReq | No impact |
| GNSS-RTK-ResidualsReq | No impact |
| GNSS-RTK-FKP-GradientsReq | No impact |
| GNSS-SSR-OrbitCorrectionsReq | No impact |
| GNSS-SSR-ClockCorrectionsReq | No impact |
| GNSS-SSR-CodeBiasReq | No impact |
| GNSS-SSR-URA-Req | No impact |
| GNSS-SSR-PhaseBiasReq | No impact |
| GNSS-SSR-STEC-CorrectionReq | No impact |
| GNSS-SSR-GriddedCorrectionReq | No impact |
| NavIC-DifferentialCorrectionsReq | No impact |
| NavIC-GridModelReq | No impact |

    - GNSS Location Information (Section 6.5.2.5)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideLocationInformation | No impact |

    - GNSS Location Information Elements (Section 6.5.2.6)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-SignalMeasurementInformation | No impact |
| MeasurementReferenceTime | No impact |
| GNSS-MeasurementList | No impact |
| GNSS-LocationInformation | No impact |

    - GNSS Location Information Request (Section 6.5.2.7)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestLocationInformation | No impact |

    - GNSS Location Information Request Elements (Section 6.5.2.8)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-PositioningInstructions | No impact |

    - GNSS Capability Information (Section 6.5.2.9)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideCapabilities | No impact |

    - GNSS Capability Information Elements (Section 6.5.2.10)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-CommonAssistanceDataSupport | No impact |
| GNSS-ReferenceTimeSupport | No impact |
| GNSS-ReferenceLocationSupport | No impact |
| GNSS-IonosphericModelSupport | No impact |
| GNSS-EarthOrientationParametersSupport | No impact |
| GNSS-RTK-ReferenceStationInfoSupport | No impact |
| GNSS-RTK-AuxiliaryStationDataSupport | No impact |
| GNSS-GenericAssistanceDataSupport | No impact |
| GNSS-TimeModelListSupport | No impact |
| GNSS-DifferentialCorrectionSupport | No impact |
| GNSS-NavigationModelSupport | No impact |
| GNSS-RealTimeIntegritySupport | No impact |
| GNSS-DataBitAssistanceSupport | No impact |
| GNSS-AcquisitionAssistanceSupport | No impact |
| GNSS-AlmanacSupport | No impact |
| GNSS-UTC-ModelSupport | No impact |
| GNSS-AuxiliaryInformationSupport | No impact |
| BDS-DifferentialCorrectionsSupport | No impact |
| BDS-GridModelSupport | No impact |
| GNSS-RTK-ObservationsSupport | No impact |
| GLO-RTK-BiasInformationSupport | No impact |
| GNSS-RTK-MAC-CorrectionDifferencesSupport | No impact |
| GNSS-RTK-ResidualsSupport | No impact |
| GNSS-RTK-FKP-GradientsSupport | No impact |
| GNSS-SSR-OrbitCorrectionsSupport | No impact |
| GNSS-SSR-ClockCorrectionsSupport | No impact |
| GNSS-SSR-CodeBiasSupport | No impact |
| GNSS-SSR-URA-Support | No impact |
| GNSS-SSR-PhaseBiasSupport | No impact |
| GNSS-SSR-STEC-CorrectionSupport | No impact |
| GNSS-SSR-GriddedCorrectionSupport | No impact |
| NavIC-DifferentialCorrectionsSupport | No impact |
| NavIC-GridModelSupport | No impact |

    - GNSS Capability Information Request (Section 6.5.2.11)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-RequestCapabilities | No impact |

    - GNSS Error Elements (Section 6.5.2.12)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-Error | No impact |
| GNSS-LocationServerErrorCauses | No impact |
| GNSS-TargetDeviceErrorCauses | No impact |

    - Common GNSS Information Elements (Section 6.5.2.13)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-FrequencyID | No impact |
| GNSS-ID | No impact |
| GNSS-ID-Bitmap | No impact |
| GNSS-Link-CombinationsList | No impact |
| GNSS-NavListInfo | No impact |
| GNSS-NetworkID | No impact |
| GNSS-PeriodicControlParam | No impact |
| GNSS-ReferenceStationID | No impact |
| GNSS-SignalID | No impact |
| GNSS-SignalIDs | No impact |
| GNSS-SubNetworkID | No impact |
| SBAS-ID | No impact |
| SBAS-IDs | No impact |
| SV-ID | No impact |

Proposals

In this contribution, we summarized the impacts on introduction of signal B3I, especially the update in 37.355 and we get the following proposals:

Proposal 1: The global B3I signal in BDS should be supported in 3GPP both LTE and NR in Rel-17.

Proposal 2: RAN2 is kindly requested to agree CRs in [2] [3] [4].

Reference

BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", December, 2017.

RP-210903 NR Positioning Enhancements, Intel Corporation, CATT

R2-2107138 Introduction of B2a and B3I signal in BDS system in A-GNSS (36.305 CR)	CATT, CAICT

R2-2107141 Introduction of B3I signal in BDS system in A-GNSS (37.355 CR)	CATT, CAICT

R2-2107193 Introduction of B2a and B3I signal in BDS system in A-GNSS (38.305 CR)	CATT, CAICT