3GPP TSG-RAN WG2 #115e	R2-2107471

Electronic meeting, 2021-08-16 - 2021-08-27

Agenda Item:	8.7.2.4

Source:	Ericsson

Title:	Aspects for QoS management with SL relay

Document for:	Discussion, Decision

# 1	Introduction

One objective of the SL relay WI is to specify mechanisms for E2E, i.e. PC5 and Uu, QoS management for L2 relay [1]. In this paper, we discuss QoS aspects for L2 relay.

# 2	Discussions

The general QoS handling framework for L2 UE-to-Network Relay has been studied in SI phase [2]. The conclusion is that the gNB implementation can handle the QoS breakdown over Uu and PC5 for end-to-end QoS enforcement, and this breakdown can be flexibly tailored to the AS conditions on SL and Uu. Remote UE operates using the configuration provided in SIB or dedicated RRC signaling with overall better QoS performance than using pre-configuration. QoS can be enforced for each E2E bearer as the gNB can decide whether an E2E bearer is admitted or not depending on the current radio situation.

The QoS framework for L2 UE-to-Network Relay has been studied in SI phase.

In our view, we also think gNB should have full control on QoS configuration such as QoS breakdown over Uu and PC5 and bearer to Uu/PC5 RLC channel mapping in L2 relay scenario, how to do the QoS configuration should be up to gNB implementation. Therefore, we propose to confirm the SI conclusion.

Confirm that how to do the QoS configuration in L2 relay scenario is up to gNB implementation.

One open issue is the handling in case PC5 RLC channels with different E2E QoS are mapped to the same Uu RLC channel. The concern here is that this may result in loss of QoS differentiation for the PC5 RLC channels on the Uu link especially for UL where the UE determines which Uu RLC channel to transmit w/o caring the PC5 RLC channels mapped to that Uu RLC channel. In our understanding, this can be resolved by proper gNB implementation, e.g. gNB may only map PC5 RLC channels with different E2E QoS to the same Uu RLC channel if the QoS breakdown over Uu is the same for these PC5 RLC channels so that they can be treated equally over Uu.

Mapping PC5 RLC channels with different E2E QoS to the same Uu RLC channel may result in loss of QoS differentiation on the Uu link.

In case PC5 RLC channels with different end-to-end QoS are mapped to the same Uu RLC channel, proper gNB implementation can avoid the impact from loss of QoS differentiation on the Uu link.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	The general QoS handling framework for L2 UE-to-Network Relay has been studied in SI phase.

Observation 2	Mapping PC5 RLC channels with different E2E QoS to the same Uu RLC channel may result in loss of QoS differentiation on the Uu link.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm that how to do the QoS configuration in L2 relay scenario is up to gNB implementation.

Proposal 2	In case PC5 RLC channels with different end-to-end QoS are mapped to the same Uu RLC channel, proper gNB implementation can avoid the impact from loss of QoS differentiation on the Uu link.

# References

[1] RP-210893, “New WID on NR Sidelink Relay”

[2] TR 38.836, v17.0.0, “Study on NR sidelink relay; (Release 17)”