3GPP TSG RAN WG2 #115-e	R2-2107502

Electronic, 9th – 27th August, 2021

Title:	[DRAFT] LS on positioning for the UE in RRC_INACTIVE

Response to:

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN2

To:	SA2

Cc:

Contact Person:

Name:                   Yinghao Guo

E-mail Address:   yinghaoguo at huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

The Rel-17 work item in RAN comprises the following objective:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]: - DL NR positioning methods and RAT-independent positioning methods - Support of UE positioning measurements for UEs in RRC_INACTIVE state - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state Note: this work will be coordinated with the SDT WI. - As 2nd priority: - UL and DL+UL NR positioning methods - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

RAN2 has discussed the positioning techniques for the UE in RRC_INACTIVE state and the following agreements have been reached:

RAN2#113bis-e

Agreements: WA: Any uplink LCS or LPP message can be transported in RRC_INACTIVE from RAN2 perspective, subject to the data volume supported by AS layers. I.e. RAN2 do not specify a restriction on message type. FFS if LPP needs to select transport, i.e. if the message is just submitted to lower layers which decide how to deliver it (SDT, change state, etc.). FFS if RRC state is exposed to LPP.

RAN2#114-e

Agreements: Any uplink LCS or LPP message can be transported in RRC_INACTIVE from RAN2 perspective. Follow Rel-17 SDT framework for INACTIVE UL and DL positioning:  If the UE initiated data transmission using UL SDT, the network can send DL LCS, LPP message and RRC message (e.g. to configure SRS (TBD on what message is used), if UL positioning supported) to the UE.  Otherwise, if UE did not initiate UL SDT, rely on legacy operation, i.e. the network shall transition the UE to RRC_CONNECTED, e.g. based on RAN paging.

RAN2 would like to ask SA2 to take the above information into considerations and evaluate the impacts on SA2 specifications.

2. Actions:

To SA2 group.

ACTION: 	RAN2 kindly asks SA2 to take the above information into account.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #116e		1 – 12 November 2021			Electronic Meeting