3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2107526

Elbonia, 16 – 27 of August 2021

Agenda item:	6.1.4.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On supporting CHO with SCG configuration

WID/SID:	NR_Mob_enh-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

During the Rel-16 MobEnh WI the following has been agreed for CHO with SCG configuration:

2 We will not preclude SCG configuration in RRC Reconfiguration with conditional reconfiguration. Limit to cases without RAN3 impact.

At RAN2#113bis (April 2021) it has been decided to check with RAN3 whether such support is ensured in the specification. The LS in [1] has been sent. RAN3 has responded to our LS in [2]. This paper provides our further view concerning the CHO with SCG configuration.

# 2	Discussion

Including SCG configuration in RRC Reconfiguration providing CHO could be desired in order to enable multi-connectivity at the same time (or soon after) when PCell mobility is commanded. Thanks to this, additional signalling over Uu interface is avoided and the UE may setup its SCG soon after it completes the CHO execution.

Observation 1: Enabling SCG configuration in CHO command will lead to reduced signalling over Uu and faster setup of multi connectivity.

It is worth noting that HO with PSCell addition/change is supported since Rel-15, so extending that to CHO shall not be regarded as introducing a completely new functionality.

Observation 2: HO with PSCell change or addition is supported since Rel-15, so applying a similar mechanism to CHO should be an obvious follow-up decision.

In Rel-16 discussions some were arguing that the execution time of CHO is uncertain, so how can the network predict which SCG to configure. However, it should be underlined that CHO may be also configured with a single candidate cell only and the network usually knows which SCG matches well the configured candidate PSCell. Thus, such combination of CHO and SCG shall be feasible in many network deployments.

Observation 3: The network knows which SCG is to be configured with candidate CHO PCell. A single candidate CHO PCell may be configured to the UE.

RAN3 in their LS [2] has indicated there is no suitable signalling available to support the CHO with SCG configuration cases outlined by RAN2 in the initial LS [1]. Nevertheless, the Action in [2] clearly indicates RAN3 is able to specify the support for these scenarios in Rel-16. All that is needed is an explicit indication from RAN2 that such support would be required.

Observation 4: RAN3 in their LS (R3-212848) indicates it is feasible to introduce signalling for supporting CHO with SCG configuration in Rel-16.

In our understanding, the required changes are rather straightforward and shall be limited to informing the Secondary Node that UE will not immediately access the configured PSCell, as this is subject to CHO execution condition fulfilment. The exact scope of the changes is obviously up to RAN3, but we think it is fair to conclude such change should be pursued for the completeness of Rel-16 specification.

Proposal 1: CHO with SCG configuration is supported in Rel-16 despite minor RAN3 impacts.

RAN2 is therefore asked to inform RAN3 about the conclusion reached and kindly request RAN3 to introduce required specification changes. A corresponding response LS has been prepared in [3].

Proposal 2: Send the response LS to RAN3. Draft is available in [3].

# 3	Conclusion

This paper considered CHO with SCG configuration for 3GPP Rel-16. The following observations and proposals have been made:

Observation 1: Enabling SCG configuration in CHO command will lead to reduced signalling over Uu and faster setup of multi connectivity.

Observation 2: HO with PSCell change or addition is supported since Rel-15, so applying a similar mechanism to CHO should be an obvious follow-up decision.

Observation 3: The network knows which SCG is to be configured with candidate CHO PCell. A single candidate CHO PCell may be configured to the UE.

Observation 4: RAN3 in their LS (R3-212848) indicates it is feasible to introduce signalling for supporting CHO with SCG configuration in Rel-16.

Proposal 1: CHO with SCG configuration is supported in Rel-16 despite minor RAN3 impacts.

Proposal 2: Send the response LS to RAN3. Draft is available in [3].

# References

[1] R2-2104348	LS on Conditional Handover with SCG configuration scenarios	3GPP TSG-RAN WG2 Meeting #113bis electronic, 12 – 20th of April 2021

[2] R3-212848 Response LS on Conditional Handover with SCG configuration scenarios 3GPP TSG-RAN WG3 Meeting #112 electronic, 17 – 27th of May 2021

[3] R2-2107527 Response LS on CHO with SCG 3GPP TSG-RAN WG2 Meeting #115 electronic, 16 – 27th of August 2021