3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2107542

Online, August 9th – 27th, 2021

Agenda item:		8.21.2.2

Source:		Futurewei

Title:		Adaptation of QoS Flow to DRB Mapping for MDBV Enforcement

Document for:		Discussion and Decision

# Introduction

Enhancement to enforce MDBV in RAN was deferred in Rel-15 and Rel-16, while SA2 has increased MDBV of signaled 5QI by a factor of 500 in Rel-16 for some IIoT use cases, such as “discrete automation” and “intelligent transportation”, etc.

This contribution discusses the limitation of LCP in handling QoS flow based, non-static, but time- or data-volume varying QoS requirement, and proposes dynamic adaptation of QoS flow to DRB mapping for MDBV enforcement.

# Discussions

Limitations of existing LCP mechanism have been identified in [1 – 3] to support service of time/volume-varying traffic requirement:

- LCP does not allow shorter latency techniques to be used only during the slow start phase of TCP operation; and

- in scenarios where LCP restrictions cannot isolate Delay Critical GBR services from each other and/or from other services, MDBV cannot be enforced, resulting in lower capacity for all GBR bearers.

The proposed enhancements are to dynamically change, either by network signaling or configured rule at UE, parameters that control the operation of LCP, such as priority, PBR, BSD, etc., of a logic channel. However, some shortcomings were pointed out in RAN2#107bis meeting as follows:

- Adjusting operating parameters dynamically can be challenging, as LCP task is performed under tight processing timeline; and

- These enhancements work only for the special case when a single QoS flow is mapped to a DRB, but not in general scenarios when multiple QoS flows are mapped onto a DRB.

Figure 1 shows the 5GS QoS framework, illustrating the steps to apply QoS control on an application’s traffic data:

- First a service data flow (i.e., an application’s traffic data) is mapped into a QoS flow at NAS layer, according to a NAS QoS rule, where multiple service data flows may be mapped into the same QoS flow;

- Then a QoS flow is mapped into a data radio bearer (DRB) at AS layer, according to QoS flow to DRB mapping rule, where multiple QoS flows may be mapped into the same DRB;

- Finally Logical Channel Prioritization (LCP) is performed to allocate the available radio resources among the logical channels of DRBs.

QoS flow and DRB are the finest granularity of QoS differentiation at the NAS and AS layers, respectively. That is, packets carried in the same QoS flow receive the same forwarding treatment at NAS layer [4], and packets carried in the same DRB receive the same forwarding treatment at AS layer [5]. Different forwarding treatments can be enforced for distinct requirements on scheduling priority, data rate, and packet delay budget.

At AS layer, separate DRBs may be established for QoS flows requiring different packet forwarding treatment, and LCP mechanism allocates radio resources accordingly among the logical channels of DRBs to meet their respective needs of forwarding treatment. Hence, QoS flow to DRB mapping is the tool to transport requirements of a QoS flow to the corresponding AS support. For example, if a QoS flow has certain requirements on data rate and delay budget, it should be mapped onto a corresponding DRB supporting the required data rate and packet delay budget (PDB).

Observation 1: QoS flow to DRB mapping is the tool to transport requirements of a QoS flow to the corresponding AS support.

Figure 1: 5GS QoS Framework

MDBV (Maximum Data Burst Volume) denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB (i.e. 5G-AN part of the PDB) [4]. The introduction of MDBV requirement makes the support of the involved QoS flow time/volume-varying, depending on whether MDBV is already met or not – before MDBV is met, a DRB with higher priority and/or data rate needs to be used to carry packets of this QoS flow; after MDBV is met, a DRB with lower priority and/or data rate is sufficient. Hence, dynamically adjusting QoS flow to DRB mapping can achieve MDBV requirement with efficient utilization of radio resources.

Observation 2: Dynamically adjusting QoS flow to DRB mapping can achieve MDBV requirement with efficient utilization of radio resources:

- before MDBV requirement is met, the involved QoS flow is mapped to a DRB with higher priority and/or data rate;

- after MDBV requirement is met, the involved QoS flow is mapped to a DRB with lower priority and/or data rate.

A UE may be configured with QoS flow to DRB mapping by RRC [6] as,

SDAP-Config ::=                     SEQUENCE {

pdu-Session                         PDU-SessionID,

sdap-HeaderDL                       ENUMERATED {present, absent},

sdap-HeaderUL                       ENUMERATED {present, absent},

defaultDRB                          BOOLEAN,

mappedQoS-FlowsToAdd                SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI                                 OPTIONAL, -- Need N

mappedQoS-FlowsToRelease            SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI                                 OPTIONAL, -- Need N

...

}

Dynamic adaptation of QoS flow to DRB mapping may be configured in RRC by providing a QoS flow with mappings to 2 DRB, and MDBV related parameters, such as DBV (Data Burst Volume) and PDB (Packet Delay Budget), used as switching condition. For example, DBV and PDB parameters can be added into SDAP-Config of a DRB for a QoS flow to indicate that this is the DRB for the QoS flow to fallback once MDBV requirement is achieved.

Observation 3: Dynamic adaptation of QoS flow to DRB mapping may be configured in RRC by providing a QoS flow with mappings to 2 DRB, and MDBV related parameters, such as DBV and PDB, used as switching condition.

Once configured, UE performs QoS flow to DRB mapping at SDAP layer [7], as

“At the reception of an SDAP SDU from upper layer for a QoS flow, the transmitting SDAP entity shall:

-	if there is no stored QoS flow to DRB mapping rule for the QoS flow as specified in the subclause 5.3:

-	map the SDAP SDU to the default DRB;

-	else:

-	map the SDAP SDU to the DRB according to the stored QoS flow to DRB mapping rule;”

Hence, if configured with mappings to two DRBs for a QoS flow and the corresponding MDBV switching condition, SDAP can perform dynamic switching between two DRBs for a QoS flow, based on the amount of data (related to DBV) delivered in certain period of time (related to PDB) to the DRB with the data rate matched to MDBV requirement.

Observation 4: SDAP can perform dynamic switching between two DRBs for a QoS flow, based on the amount of data (related to DBV) delivered in certain period of time (related to PDB) to the DRB with the data rate matched to MDBV requirement.

Figure 2 illustrates the dynamic switch in delivery of SDAP PDUs of a QoS flow to two DRBs, based on burst data volume.

Figure 2: Adaptation of QoS Flow to DRB Mapping

Proposal 1: Dynamic adaptation of QoS flow to DRB mapping is specified to enforce MDBV requirement through

- RRC configuration of a QoS flow with mappings to 2 DRB, and a switching condition based on MDBV related parameters, such as DBV and PDB; and

- SDAP performing dynamic switching between two DRBs for a QoS flow, based on the amount of data (determined by the DBV) delivered in certain period of time (determined by the PDB) to a DRB (with the data rate matched to MDBV requirement).

# Conclusions

This contribution discusses the limitation of LCP in handling QoS flow based, non-static, but time- or data-volume varying QoS requirement, and proposes dynamic adaptation of QoS flow to DRB mapping for MDBV enforcement, with the observations and proposals as follows:

Observation 1: QoS flow to DRB mapping is the tool to transport requirements of a QoS flow to the corresponding AS support.

Observation 2: Dynamically adjusting QoS flow to DRB mapping can achieve MDBV requirement with efficient utilization of radio resources:

- before MDBV requirement is met, the involved QoS flow is mapped to a DRB with higher priority and/or data rate;

- after MDBV requirement is met, the involved QoS flow is mapped to a DRB with lower priority and/or data rate.

Observation 3: Dynamic adaptation of QoS flow to DRB mapping may be configured in RRC by providing a QoS flow with mappings to 2 DRB, and MDBV related parameters, such as DBV and PDB, used as switching condition.

Observation 4: SDAP can perform dynamic switching between two DRBs for a QoS flow, based on the amount of data (related to DBV) delivered in certain period of time (related to PDB) to the DRB with the data rate matched to MDBV requirement.

Proposal 1: Dynamic adaptation of QoS flow to DRB mapping is specified to enforce MDBV requirement through

- RRC configuration of a QoS flow with mappings to 2 DRB, and a switching condition based on MDBV related parameters, such as DBV and PDB; and

- SDAP performing dynamic switching between two DRBs for a QoS flow, based on the amount of data (determined by the DBV) delivered in certain period of time (determined by the PDB) to a DRB (with the data rate matched to MDBV requirement).

# References

R2-1912845, “Fixed LCP Restrictions”, Nokia, Ericsson, Fujitsu, Nokia Shanghai Bell.

R2-1916007, “Open issue of uplink MDBV enforcement”, Huawei, HiSilicon.

R2-2002743, “MDBV Enforcement”, Nokia, Nokia Shanghai Bell.

3GPP TS 23.501, “System Architecture for the 5G System; Stage 2”.

3GPP TS 38.300, “NR; NR and NG-RAN Overall Description; Stage 2”.

3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”.

3GPP TS 37.324, “E-UTRA and NR; Service Data Adaptation Protocol (SDAP) specification”.