3GPP TSG-RAN WG2 Meeting #115-e	R2-210xxxx

# Online, Aug 16th – 27th, 2021

Agenda Item: 	6.1.4.3

Source: 			Huawei, HiSilicon

Title: 	Summary of [AT115-e][028][NR16] UE capabilities I

Document for: 	Discussion and decision

- Introduction

This document summarizes the following offline discussion.

[AT115-e][028][NR16] UE capabilities I (Huawei)

Scope: Determine agreeable parts and agree CRs, Treat R2-2108480, R2-2107342, R2-2108641, R2-2108468, R2-2108585, R2-2108586, R2-2108651, R2-2106952, R2-2108618, R2-2108619, R2-2108735, R2-2108736

Intended outcome: Report, Agreed CRs.

Deadline: Schedule 1

- Contact from companies

| Company | Email |
| --- | --- |

- Discussion

  - Part 1: Intended to determine agreeable parts

Misc Corrections

R2-2108480	Miscellaneous corrections to UE capability descriptions	Lenovo, Motorola Mobility	CR	Rel-16	38.306	16.5.0	0626	-	F	NR_unlic-Core, TEI16

The proposed changes in above CR include:

1.	offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16: The description of the conditional support of pdcch-MonitoringAnyOccasions with value withDCI-Gap (FG 3-5a) has been added.

2.	searchSpaceSetGroupSwitchingwWithDCI-r16: The capability name has been replaced by searchSpaceSwitchWithDCI-r16.

3.	extendedSearchSpaceSwitchWithDCI-r16: It has been clarified that UE indicating support of this feature shall indicate support of searchSpaceSwitchWithDCI-r16.

4.	Number of editorial issues have been fixed (missing suffices, misalignment of parameter names with TS 38.331 etc.).

Q1 Do companies agree with the intention of the CR above?

| Company | Yes or No | Comments |
| --- | --- | --- |

DAPS

R2-2107342	Correction on the capability field DiffSCS-DAPS	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

R2-2108641	Correction on the capability field DiffSCS-DAPS	Huawei, HiSilicon	CR	Rel-16	38.306	16.5.0	0636	-	F	NR_Mob_enh-Core

The proposal in above discussion paper is listed below. The above CR includes the corresponding change.

Proposal 1: for the two capability fields, i.e. intraFreqDiffSCS-DAPS-r16 and interFreqDiffSCS-DAPS-r16, add clarification that “In this release the UE shall not report this UE capability”.

Q2 Do companies agree with the intention of Proposal 1 and CR above?

| Company | Yes or No | Comments |
| --- | --- | --- |

eMIMO

R2-2108468	Correction to ul-FullPwrMode capability	Sequans Communications	CR	Rel-16	38.306	16.5.0	0625	-	F	NR_eMIMO-Core

The proposed change in above CR includes: removing the misleading reference “fullpower as specified in clause 6.1.1.1 of TS.38.214 [12]” for ul-FullPwrMode-r16.

Q3 Do companies agree with the intention of the CR above?

| Company | Yes or No | Comments |
| --- | --- | --- |

IIOT

R2-2108585	Correction on PDCCH Blind Detection in CA	Huawei, HiSilicon	CR	Rel-16	38.331	16.5.0	2781	-	F	NR_IIOT-Core

R2-2108586	Correction on PDCCH Blind Detection in CA	Huawei, HiSilicon	CR	Rel-16	38.306	16.5.0	0634	-	F	NR_IIOT-Core

The proposed changes in above CRs include: To allow UE to report more than one combinations of pdcch-BlindDetectionCA1-r16 capability and pdcch-BlindDetectionCA2-r16 capability, add a list of elements of SEQUENCE type except for pdcch-BlindDetectionCA-Mixed-r16 IE and pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16 IE.

Q4 Do companies agree with the intention of the CRs above?

| Company | Yes or No | Comments |
| --- | --- | --- |

UL Skipping

R2-2108651	FR1FR2 differentiation for enhanced UL grant skipping capabilities	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell	discussion	Rel-16	TEI16

The proposal in above discussion paper is listed below. The above CR includes the corresponding change.

Proposal 1:	RAN2 to discuss and decide on an approach to modify the current specification to allow FR differentiation

Proposal 2:	RAN2 to introduce a new capability IE to allow frequency range differentiation. 2 options are proposed:

•	Option-A: new capability is defined per nr-bands, to allow full flexibility for the UE to indicate the supported duplex mode and frequency range combination.

•	Option-B: new TDD only capability with frequency range differentiation. Both, current and new IE may be used by the UE to convey its capability to the network.

Proposal 3: to have one of the proposed changes in section 4 &5 or 6&7 agreed.

Q5 Do companies agree to modify the current specification to allow FR differentiation for enhancedSkipUplinkTxConfigured-r16 and enhancedSkipUplinkTxDynamic-r16? If yes, which option above do companies prefer?

| Company | Yes or No | Option-A or B? | Comments |
| --- | --- | --- | --- |

UL TX Switching

R2-2106952	LS on UL MIMO coherence for Tx switching between two carriers (R4-2107765; contact: China Telecom)	RAN4	LS in	Rel-16	NR_RF_FR1-Core	To:RAN2, RAN1

R2-2108618	Adding UE capability of UL MIMO coherence for UL Tx switching	Huawei, HiSilicon, China Telecom, Apple	CR	Rel-16	38.306	16.5.0	0635	-	F	NR_RF_FR1-Core

R2-2108619	Adding UE capability of UL MIMO coherence for UL Tx switching	Huawei, HiSilicon, China Telecom, Apple	CR	Rel-16	38.331	16.5.0	2786	-	F	NR_RF_FR1-Core

R2-2108735	Introducing UL MIMO coherence capability for Tx switching	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.5.0	0638	-	F	NR_RF_FR1-Core

R2-2108736	Introducing UL MIMO coherence capability for Tx switching	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2796	-	F	NR_RF_FR1-Core

The intention of CRs in [9][10] and [11][12] is the same, i.e. adding new per-BC capability uplinkTxSwitching-PUSCH-TransCoherence-r16 based on LS R2-2106952. The main difference between CRs in [9][10] and [11][12] is the value of uplinkTxSwitching-PUSCH-TransCoherence-r16.

Q6-1 Do companies agree with the intention of the CRs above, i.e. adding a new per-BC UE capability uplinkTxSwitching-PUSCH-TransCoherence-r16?

| Company | Yes or No | Comments |
| --- | --- | --- |

Option 1: The value set is: ENUMERATED {nonCoherent, partialCoherent, fullCoherent}

Option 2: The value set is: ENUMERATED {nonCoherent, coherent}

Q6-2 if the answer for Q6-1 is yes, which option above do companies prefer?

| Company | Option1 or 2? | Comments |
| --- | --- | --- |

- Conclusions

To be added…

- References

R2-2108480	Miscellaneous corrections to UE capability descriptions	Lenovo, Motorola Mobility	CR	Rel-16	38.306	16.5.0	0626	-	F	NR_unlic-Core, TEI16

R2-2107342	Correction on the capability field DiffSCS-DAPS	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

R2-2108641	Correction on the capability field DiffSCS-DAPS	Huawei, HiSilicon	CR	Rel-16	38.306	16.5.0	0636	-	F	NR_Mob_enh-Core

R2-2108468	Correction to ul-FullPwrMode capability	Sequans Communications	CR	Rel-16	38.306	16.5.0	0625	-	F	NR_eMIMO-Core

R2-2108585	Correction on PDCCH Blind Detection in CA	Huawei, HiSilicon	CR	Rel-16	38.331	16.5.0	2781	-	F	NR_IIOT-Core

R2-2108586	Correction on PDCCH Blind Detection in CA	Huawei, HiSilicon	CR	Rel-16	38.306	16.5.0	0634	-	F	NR_IIOT-Core

R2-2108651	FR1FR2 differentiation for enhanced UL grant skipping capabilities	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell	discussion	Rel-16	TEI16

R2-2106952	LS on UL MIMO coherence for Tx switching between two carriers (R4-2107765; contact: China Telecom)	RAN4	LS in	Rel-16	NR_RF_FR1-Core	To:RAN2, RAN1

R2-2108618	Adding UE capability of UL MIMO coherence for UL Tx switching	Huawei, HiSilicon, China Telecom, Apple	CR	Rel-16	38.306	16.5.0	0635	-	F	NR_RF_FR1-Core

R2-2108619	Adding UE capability of UL MIMO coherence for UL Tx switching	Huawei, HiSilicon, China Telecom, Apple	CR	Rel-16	38.331	16.5.0	2786	-	F	NR_RF_FR1-Core

R2-2108735	Introducing UL MIMO coherence capability for Tx switching	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.5.0	0638	-	F	NR_RF_FR1-Core

R2-2108736	Introducing UL MIMO coherence capability for Tx switching	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2796	-	F	NR_RF_FR1-Core