3GPP TSG-RAN WG2 Meeting #115-e	R2-210xxxx

# Online, Aug 16th – 27th, 2021

Agenda Item: 	6.1.4.3

Source: 			Huawei, HiSilicon

Title: 	Summary of [AT115-e][028][NR16] UE capabilities I

Document for: 	Discussion and decision

# Introduction

This document summarizes the following offline discussion.

[AT115-e][028][NR16] UE capabilities I (Huawei)

Scope: Determine agreeable parts and agree CRs, Treat R2-2108480, R2-2107342, R2-2108641, R2-2108468, R2-2108585, R2-2108586, R2-2108651, R2-2106952, R2-2108618, R2-2108619, R2-2108735, R2-2108736

Intended outcome: Report, Agreed CRs.

Deadline: Schedule 1

# Contact from companies

| Company | Email |
| --- | --- |
| Huawei, HiSilicon | Yiru Kuang, kuangyiru@huawei.com |
| ZTE | liu.jing30@zte.com.cn li.wenting@zte.com.cn zhang.mengjie@zte.com.cn |
| Qualcomm Incorporated | mkitazoe@qti.qualcomm.com |
| MediaTek | chun-fan.tsai@mediatek.com |
| Nokia | amaanat.ali@nokia.com |
| Ericsson | Helka-Liina Määttänen, Helka-liina.maattanen@ericsson.com |
| China Telecom | linp@chinatelecom.cn |
| Apple | naveen.palle@apple.com |

# Discussion

## Part 1: Intended to determine agreeable parts

### Misc Corrections

R2-2108480	Miscellaneous corrections to UE capability descriptions	Lenovo, Motorola Mobility	CR	Rel-16	38.306	16.5.0	0626	-	F	NR_unlic-Core, TEI16

The proposed changes in above CR include:

1.	offsetSRS-CB-PUSCH-PDCCH-MonitorAnyOccWithGap-fr1-r16: The description of the conditional support of pdcch-MonitoringAnyOccasions with value withDCI-Gap (FG 3-5a) has been added.

2.	searchSpaceSetGroupSwitchingwWithDCI-r16: The capability name has been replaced by searchSpaceSwitchWithDCI-r16.

3.	extendedSearchSpaceSwitchWithDCI-r16: It has been clarified that UE indicating support of this feature shall indicate support of searchSpaceSwitchWithDCI-r16.

4.	Number of editorial issues have been fixed (missing suffices, misalignment of parameter names with TS 38.331 etc.).

Q1 Do companies agree with the intention of the CR above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm Incorporated | Yes |  |
| MediaTek | Yes |  |
| ZTE(Wenting) | Yes |  |
| Nokia | Yes |  |
| Ericsson | Yes |  |
| China Telecom | Yes |  |
| Apple | Yes |  |

### DAPS

R2-2107342	Correction on the capability field DiffSCS-DAPS	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

R2-2108641	Correction on the capability field DiffSCS-DAPS	Huawei, HiSilicon	CR	Rel-16	38.306	16.5.0	0636	-	F	NR_Mob_enh-Core

The proposal in above discussion paper is listed below. The above CR includes the corresponding change.

Proposal 1: for the two capability fields, i.e. intraFreqDiffSCS-DAPS-r16 and interFreqDiffSCS-DAPS-r16, add clarification that “In this release the UE shall not report this UE capability”.

Q2 Do companies agree with the intention of Proposal 1 and CR above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Proponent. |
| Qualcomm Incorporated | No | Fine with the intention of the CRs. But we think making them dummy fields which is ignored by the network is better approach for backward compatibility. |
| MediaTek | Intention OK, CR not | Same view as QC. We prefer to dummify those IEs in new ASN.1 and NW just ignore those fields if included |
| ZTE(Mengjie) | No | Agree with the intention. But we also think that dummying the fileds is a better solution for backward compatibility. |
| Nokia | Intention yes, CR no | Agree with the intention but maybe the dummify approach is more logical |
| Ericsson | No | This capability was intentionally added and removing it would not be according to agreements/intention. Example: Two UEs (one simple UE and one advanced UE) may support the same SCSs: they support SCS 15 kHZ and 60 kHZ in the source band they support SCS 15 kHZ and 60 kHZ in the target band. The simple UE may support DAPS between source and target, but only if 15 kHz was used in the source and 15 kHz is also used in the target, or only if 60 kHz was used in source and 60 kHz is also used in the target. The more advanced UE may support DAPS between source and target, even if source SCS is 15 kHz and target SCS is 60 kHz is also used in the target. The capability bit is used to distinguish these two types of UEs and we don’t think we can dummify it. At least it is not up to RAN2 to make this obsolete. |
| China Telecom | Intention yes, CR no | Same view with others. |
| Apple | No | We do not need to dummify the two capabilities. |

### eMIMO

R2-2108468	Correction to ul-FullPwrMode capability	Sequans Communications	CR	Rel-16	38.306	16.5.0	0625	-	F	NR_eMIMO-Core

The proposed change in above CR includes: removing the misleading reference “fullpower as specified in clause 6.1.1.1 of TS.38.214 [12]” for ul-FullPwrMode-r16.

Q3 Do companies agree with the intention of the CR above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Editorial change. Minor change, can be merged to e.g. Misc Corrections in [1]. |
| Qualcomm Incorporated | No | Why not correct the specification reference, instead of removing it? |
| MediaTek | Yes, but | As QC commented, maybe we could use correct reference section? |
| ZTE(Wenting) | Yes with comments | Editorial change, and can merge the correct reference to Misc Corrections in [1]. |
| Nokia | Yes with comment | Good to find that the reference was not fully correct, but instead of removing it we should add the correct reference. |
| Ericsson | Yes | We are fine to update the reference as well. |
| China Telecom | Yes | Same view with others. We also support to update the reference instead of removing it. |
| Apple | Yes with comment | Ok with correct reference. |

### IIOT

R2-2108585	Correction on PDCCH Blind Detection in CA	Huawei, HiSilicon	CR	Rel-16	38.331	16.5.0	2781	-	F	NR_IIOT-Core

R2-2108586	Correction on PDCCH Blind Detection in CA	Huawei, HiSilicon	CR	Rel-16	38.306	16.5.0	0634	-	F	NR_IIOT-Core

The proposed changes in above CRs include: To allow UE to report more than one combinations of pdcch-BlindDetectionCA1-r16 capability and pdcch-BlindDetectionCA2-r16 capability, add a list of elements of SEQUENCE type except for pdcch-BlindDetectionCA-Mixed-r16 IE and pdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16 IE.

Q4 Do companies agree with the intention of the CRs above?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Proponent. |
| Qualcomm Incorporated | No | Agree to the intention, but the 38.331 CR is not complete in that it is FFS how many combinations the ASN.1 will support. We also wonder if we have the same problem with FG11-2e? |
| ZTE |  | We agree with the intention and the modification to the Asn.1 coding, bu we also agree with Q that we need to confirm the number of the supported combinations. |
| Nokia | No | We basically agree with Qualcomm and ZTE – the CR cannot be implemented before it is clarified how many combinations are we going to support. |
| Apple | No | Please find our comments below. - We agree with the intention of this CR, but we wonder whether a RAN1 FG is to be added? This may need to be clarified with RAN1. - The work item code in the CR should rather be NR_L1enh_URLLC-Core. - We think the ASN.1 changes in yellow may be needed in addition, depending on the solution approach that is taken. CA-ParametersNR-v16xy ::= SEQUENCE { -- R1 11-2h: add the replicated FGs... pdcch-BlindDetectionCA-MixedList-r16 SEQUENCE(SIZE(1..maxNrofPdcch-BlindDetectionCA-Mixed-r16-1)) OF Pdcch-BlindDetectionCA-MixedExt-r16 OPTIONAL, -- R1 11-2i: add the replicated FGs... pdcch-BlindDetectoinCA-MixedList-NonAlignedSpan-r16 SEQUECNCE(SIZE(1..maxNrofPdcch-BlindDetectionCA-Mixed-NonAlignedSpan-r16-1)) OF Pdcch-BlindDetectionCA-Mixed-NonAlignedSpanExt-r16 OPTIONAL } pdcch-BlindDetectionCA-MixedExt-r16 SEQUENCE { pdcch-BlindDetectionCA1-r16 INTEGER (1..15), pdcch-BlindDetectionCA2-r16 INTEGER (1..15), supportedSpanArrangement-r16 ENUMERATED {alignedOnly, alignedAndNonAligned} } Pdcch-BlindDetectionCA-Mixed-NonAlignedSpanExt-r16 ::= SEQUENCE { pdcch-BlindDetectionCA1-r16 INTEGER (1..15), pdcch-BlindDetectionCA2-r16 INTEGER (1..15) } |

### UL Skipping

R2-2108651	FR1FR2 differentiation for enhanced UL grant skipping capabilities	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell	discussion	Rel-16	TEI16

The proposal in above discussion paper is listed below. The above CR includes the corresponding change.

Proposal 1:	RAN2 to discuss and decide on an approach to modify the current specification to allow FR differentiation

Proposal 2:	RAN2 to introduce a new capability IE to allow frequency range differentiation. 2 options are proposed:

•	Option-A: new capability is defined per nr-bands, to allow full flexibility for the UE to indicate the supported duplex mode and frequency range combination.

•	Option-B: new TDD only capability with frequency range differentiation. Both, current and new IE may be used by the UE to convey its capability to the network.

Proposal 3: to have one of the proposed changes in section 4 &5 or 6&7 agreed.

Q5 Do companies agree to modify the current specification to allow FR differentiation for enhancedSkipUplinkTxConfigured-r16 and enhancedSkipUplinkTxDynamic-r16? If yes, which option above do companies prefer?

| Company | Yes or No | Option-A or B? | Comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | No | Option-A if CR is agreed to pursue | In RAN2#113 meeting, this issue has been discussed in offline 019 and the conclusion is no need of FR1/FR2 differentiation, we don’t see strong motivation to revert the previous conclusion. If majority of companies prefer to introduce this now, we would prefer Option-A which is more aligned with our Rel-16 principle for FRx/xDD capability design. |
| Qualcomm Incorporated | Yes |  | Proponent. Fine with either solution. |
| MediaTek | No strong view | Option A | We see no strong need to have FR1/FR2 differentiation but fine to have this if majority prefer. If agreed, prefer to follow previous agreement to avoid any misunderstanding. |
| ZTE |  | Option A | If the intention is supported by majority companies, then we prefer option A. |
| Nokia | Yes |  | Fine to go with the consensus here. |
| Ericsson | No | Option-A if CR is agreed to pursue | Agree with Huawei. |
| Apple | Yes | Option A | We are also OK with option B, but option A is our preference. |

### UL TX Switching

R2-2106952	LS on UL MIMO coherence for Tx switching between two carriers (R4-2107765; contact: China Telecom)	RAN4	LS in	Rel-16	NR_RF_FR1-Core	To:RAN2, RAN1

R2-2108618	Adding UE capability of UL MIMO coherence for UL Tx switching	Huawei, HiSilicon, China Telecom, Apple	CR	Rel-16	38.306	16.5.0	0635	-	F	NR_RF_FR1-Core

R2-2108619	Adding UE capability of UL MIMO coherence for UL Tx switching	Huawei, HiSilicon, China Telecom, Apple	CR	Rel-16	38.331	16.5.0	2786	-	F	NR_RF_FR1-Core

R2-2108735	Introducing UL MIMO coherence capability for Tx switching	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.5.0	0638	-	F	NR_RF_FR1-Core

R2-2108736	Introducing UL MIMO coherence capability for Tx switching	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2796	-	F	NR_RF_FR1-Core

The intention of CRs in [9][10] and [11][12] is the same, i.e. adding new per-BC capability uplinkTxSwitching-PUSCH-TransCoherence-r16 based on LS R2-2106952. The main difference between CRs in [9][10] and [11][12] is the value of uplinkTxSwitching-PUSCH-TransCoherence-r16.

Q6-1 Do companies agree with the intention of the CRs above, i.e. adding a new per-BC UE capability uplinkTxSwitching-PUSCH-TransCoherence-r16?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Proponent of CRs [9][10]. |
| ZTE | Yes | Proponent of CRs [11][12]. |
| Qualcomm Incorporated | No | RAN4 LS was also sent to RAN1. The UL coherence depends on the UE transmitter architecture. Given typical UE implementation would support multiple bands with common Tx chains in CA and DC, the coherence should be per-band and per-band combination capability. Our view therefore is that we should change the existing UE capability from per RF band to per band per band combination. Then it can cover non-Tx-switching case and Tx-switching case in different band combinations. We submitted a paper in RAN1. We should at least wait for RAN1 discussion to avoid out of sync. |
| MediaTek | Yes, but | Also fine to wait RAN1 progress |
| Nokia |  | Fine to wait for RAN1 to complete their discussions. |
| Ericsson |  | We are also fine to wait for RAN1. |
| China Telecom | Yes | Proponent of CRs [9][10]. We are also fine to wait for RAN1 progress. |
| Apple | Yes | Proponent of CRs [9][10]. Regarding QC’s comment, we feel it’s hard to say if the change from per band to per band per BC would be agreed/decided in RAN1 shortly thus we don’t need to postpone this issue in RAN2. |

Option 1: The value set is: ENUMERATED {nonCoherent, partialCoherent, fullCoherent}

Option 2: The value set is: ENUMERATED {nonCoherent, coherent}

Q6-2 if the answer for Q6-1 is yes, which option above do companies prefer?

| Company | Option1 or 2? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option1 | The basic idea of option1 is to copy the value set of existing MIMO capability pusch-TransCoherence without change, in order to make sure the RAN1 spec related to the handling of pusch-TransCoherence can also applied to this new UE capability without any change. Regarding ‘fullCoherent’ in option1 v.s. ‘coherent’ in option2, the thing to be clarified is in RAN4 CR R4-2109582 the value of ‘coherent’ is used, but after checking with RAN4 colleague it actually meant ‘fullcoherent’, and RAN4 CR can be updated based on RAN2 agreed value set. Regarding ‘partialCoherent’, although we notified the value of partialCoherent will not be used for 2Tx UE, considering future-proof and spec maintenance we copied it here. But no strong view, we can follow majority views on whether to include partialCoherent or not. |
| ZTE | Option 2 | We think option 2 should be adopted, because: - “partialCoherent” is not applicable to 2Tx UE, so for both Rel-16 and Rel-17 UL Tx switching, this value will not be used. Even if in Rel-18, UL Tx switching will be enhanced to support more than 2Tx UEs (e.g. 3Tx, 4Tx), we understand several new capabilities will be defined, if needed, any extension (like partialCoherent) can be introduced at that time. In Rel-16 spec, we don’t have to reserve something for Rel-18+ use. - Option 2 is aligned with RAN4 agreed CR. And we see no need to update RAN4’s spec because using “Coherent” does not cause any confusion in RAN4. - If Option 1 is adopted, we are not sure how RAN4 spec describes the value of “partialCoherent” (as it is invalid right now), and also we need to discuss whether to add some restriction in RAN2 spec, like saying “UE is not allowed to report partialCoherent in this release”. So we think option 2 is simple and more accurate. Besides the value range of new capability, the description in TS 38.306 CRs are also different. We prefer the version in R2-2108735[11], because R2-2108618[9] describes the “support of partialCoherent” which is technically incorrect for Rel-16 and Rel-17 UL Tx switching. |
| MediaTek |  | It seems that RAN4 (and RAN1) should tell us what the correct enum value is in UE capability (as they did in UE feature table). |
| Ericsson |  | We can just wait for RAN1 discussion to conclude. |
| China Telecom | Option 1 | Proponent of CRs [9][10]. We have no strong view and also fine to go for the majority views on whether “partialCoherent” is needed or not. |
| Apple | Option 1 | Though we see the point raised by ZTE that partialCoherent is not applicable for 2Tx UE, however in RAN4 LS R4-2107765, it also mentions if this new UE capability for UL Tx switching is absent, the existing per band UE capability is applicable for UL Tx switching. In another word, NW would not have trouble in understanding the UE capability for UL Tx switching with receiving the legacy field pusch-TransCoherence. Thus, it might be not that critical to change the value set. |

# Conclusions

To be added…

# References

R2-2108480	Miscellaneous corrections to UE capability descriptions	Lenovo, Motorola Mobility	CR	Rel-16	38.306	16.5.0	0626	-	F	NR_unlic-Core, TEI16

R2-2107342	Correction on the capability field DiffSCS-DAPS	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

R2-2108641	Correction on the capability field DiffSCS-DAPS	Huawei, HiSilicon	CR	Rel-16	38.306	16.5.0	0636	-	F	NR_Mob_enh-Core

R2-2108468	Correction to ul-FullPwrMode capability	Sequans Communications	CR	Rel-16	38.306	16.5.0	0625	-	F	NR_eMIMO-Core

R2-2108585	Correction on PDCCH Blind Detection in CA	Huawei, HiSilicon	CR	Rel-16	38.331	16.5.0	2781	-	F	NR_IIOT-Core

R2-2108586	Correction on PDCCH Blind Detection in CA	Huawei, HiSilicon	CR	Rel-16	38.306	16.5.0	0634	-	F	NR_IIOT-Core

R2-2108651	FR1FR2 differentiation for enhanced UL grant skipping capabilities	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell	discussion	Rel-16	TEI16

R2-2106952	LS on UL MIMO coherence for Tx switching between two carriers (R4-2107765; contact: China Telecom)	RAN4	LS in	Rel-16	NR_RF_FR1-Core	To:RAN2, RAN1

R2-2108618	Adding UE capability of UL MIMO coherence for UL Tx switching	Huawei, HiSilicon, China Telecom, Apple	CR	Rel-16	38.306	16.5.0	0635	-	F	NR_RF_FR1-Core

R2-2108619	Adding UE capability of UL MIMO coherence for UL Tx switching	Huawei, HiSilicon, China Telecom, Apple	CR	Rel-16	38.331	16.5.0	2786	-	F	NR_RF_FR1-Core

R2-2108735	Introducing UL MIMO coherence capability for Tx switching	ZTE Corporation, Sanechips	CR	Rel-16	38.306	16.5.0	0638	-	F	NR_RF_FR1-Core

R2-2108736	Introducing UL MIMO coherence capability for Tx switching	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.5.0	2796	-	F	NR_RF_FR1-Core