3GPP TSG-RAN WG2 Meeting #115-e	R2-21XXXX

Online, 9-27 August, 2021

Title:	[Draft] LS on limited service availability of an SNPN

Release:	Release 17

Work Item:	NG_RAN_PRN_enh-Core

Source:	CMCC [to be RAN2]

To:	CT1, SA2

Cc:

Contact Person:

Name:	Chaili

E-mail Address:	chaili@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

During RAN2#113_e meeting, RAN2 had achieved the following agreement relevant to support of emergency services for SNPN:

- R17 UEs in SNPN Access Mode can camp on an acceptable SNPN cell supporting emergency services to obtain emergency services.

In RAN2#115_e meeting, RAN2 agreed to:

- Introduce a new IE/field to indicate the support of IMS emergency service for SNPN.

And also, RAN2 discussed the availability of emergency services for R17/R16/R15 UEs.

Regarding the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs. RAN2 tends to reach the following conclusion:

- The R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN.

RAN2 would like to ask CT1/SA2 to confirm RAN2’s understanding.

Question:

Can CT1/SA2 to confirm that the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN.

-	if the NG-RAN is shared by more than one network […], the broadcast indicator is related to those networks that supports Emergency Services.

# 2. Actions:

RAN2 respectfully asks CT1/SA2 to answer the above question.

# 3. Date of Next RAN WG2 Meetings:

RAN2#116-e                        1st November - 11th November 2021	Online