3GPP TSG-RAN2 Meeting #115e	Draft of R2-2108851

Online, 9th – 27th, August 2021

Agenda Item:	4.5, 7.1.1, 7.4

Source: 	Samsung

Title:  	[AT115-e][201][LTE] Miscellaneous LTE CRs

Document for:	Discussion and Decision

# Introduction

This report gives a summary of this offline email discussion.

[AT115-e][201][LTE] Miscellaneous LTE CRs (Samsung)

Scope:

    - Discuss LTE CRs marked for this discussion (if needed)

Intended outcome:

    - Discussion report in R2-2108851

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Thu, UTC 1700

    - Deadline for CR finalization: 2nd week Wed, UTC 0900

The deadline for the first round is Thu, UTC 0900.

# Contact information

| Company | Email |
| --- | --- |
| Samsung | seungri.jin@samsung.com |

# Discussion

## On T330 resetting

R2-2108312	On T330 resetting	Ericsson, ZTE Corporation, Sanechips	CR	Rel-15	36.331	15.14.0	4712	-	F	LTE_5GCN_connect-Core

T330 (logged MDT DurationTimer) is used to indicate the effectiveness period of a logged MDT configuration which shall keep running irrespective of state transition or RAT type.

However, according current specs T330 will be stopped when UE enters RRC_INACTIVE or when UE HO to another RAT, as a consequence, UE cannot continue logging MDT results when transiting to idle or connected (if configured to log MBSFN measurements) or coming back to LTE since T330 is stopped and not restarted.

Question 1)
Do companies agree on this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |

## LTE RRC Rapporteur CRs

R2-2108634	Minor changes collected by Rapporteur for Rel-15	Samsung	CR	Rel-15	36.331	15.14.0	4718	-	F	LTE_eMTC4-Core, LTE_sTTIandPT, LTE-L23

R2-2108635	Minor changes collected by Rapporteur for Rel-16	Samsung	CR	Rel-16	36.331	16.5.0	4719	-	A	LTE_eMTC4-Core, LTE_sTTIandPT, LTE-L23

This CRs include the minor changes, especially removing the redundant editorial Notes which are reamined in the specifications.

- In PDSCH-ConfigDedicated IE, below Editor’s note for eMTC is removed.

-- eNote (ToDo): Clarify that eMTC fields (i.e. fields starting with ce-) do not apply

-- for SCell (merging issue)

- In SPS-ConfigUL IE, below Editor’s note for STTI is removed.

-- eNote (TBC) that no separate STTI field is required (alike in merged CR)

- Editor’s note in Annex A is removed.

Editor's note	No agreements have been reached concerning the extension of RRC PDUs so far. Any statements in this clause about the protocol extension mechanism should be considered as FFS.

- In A.4.3.1, some below two TBD parts are removed.

<TBD: ref to seperate example>

Question 2)
Do companies agree on this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |

## Early security reactivation upon reception of RRCConnectionReject

R2-2107774	Correction on early security reactivation upon reception of RRCConnectionReject	NEC	CR	Rel-16	36.331	16.5.0	4696	-	F	TEI16, LTE_eMTC5-Core

According to 5.3.3.18, early security reactivation includes UP transmission using PUR and resuming a suspended RRC connection in 5GC. However, in 5.3.3.8, early security reactivation, transmission using PUR and RRC connection resume in 5GC are listed as separate cases.

This misalignment would be caused by two overlapping approved CRs in RAN#87, where one was introducing 5.3.3.18 for TEI16 (CR4167r2/ RP-200357) and the other was adding PUR and RRC connection resume in 5GC for eMTC (CR4191r1/ RP-200360).

This CR proposed to delete “or for transmission using PUR or for resuming a suspended RRC connection in 5GC”, and add a reference to 5.3.3.18.

Question 3)
Do companies agree on this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |

## ReportConfigEUTRA for CHO/CPAC

R2-2108701	36.331 Correction on ReportConfigEUTRA for CHO/CPAC	CATT	CR	Rel-16	36.331	16.5.0	4720	-	F	LTE_feMob-Core

In the current specification, the field of triggerType is mandatory present in IE ReportConfigEUTRA. However, if condReconfigurationTriggerEUTRA-r16 is configured for conditional reconfiguration (CHO/CPC), both the fields of triggerType and condReconfigurationTriggerEUTRA-r16 will be present in the IE of ReportConfigEUTRA. It is not clear which trigger type should be applied for the associated reprot configuration. Considering that for CHO/CPC, the field of triggerType is not applicable, and the UE should ignore the field of triggerType when condReconfigurationTriggerEUTRA-r16 is configured.

Moreover, the fields of triggerQuantity/ reportQuantity/ maxReportCells/ reportInterval/ reportAmount are all mandatory present fields. Except the field of triggerQuantity, all the other fields are related with measurement report configuration which is not applicable for CHO/CPC. Hence if the condReconfigurationTriggerEUTRA-r16 is configured, the UE should ignore them.

Question 4)
Do companies agree on this CR?

| Company | Yes/No | Comments |
| --- | --- | --- |

# Summary of email discussion

TBD

# Conclusions

TBD

# References