3GPP TSG-RAN WG2 Meeting #115e                       R2-21XXXX

E-Meeting, 9th - 27th Aug 2021

Source:	Xiaomi

Title:	Report of [Offline-616]

Agenda Item:	8.7.2.1

Document for:	Discussion and Decision

# Introduction

This contribution is to discuss following offline discussion.

[AT115-e][616][Relay] Proposals from control plane summary (Xiaomi)

Scope: Briefly discuss P1/P4/P5 and P8/P9/P10 of R2-2108824 and attempt to reach consensus. Also confirm if P18 is agreeable.

Intended outcome: Report to comeback session, in R2-2108948

Deadline:  Tuesday 2021-08-24 2000 UTC

# Discussion

## 2.1 Uu RLC configuration for remote UE’s SRB0/1

Regarding Uu RLC configuration for remote UE’s SRB0 message, companies’ views are observed as following,

Option 1,	Fixed/specified 	[1], [6], [33], [36]

Option 2,	Default			[7], [9], [13]

Option 3,	NW configured	[4], [7], [9], [13], [15], [20], [26]

Proponents of option 1 prefer the same principle as PC-5 RLC configuration should be reused for Uu. Proponents of option 3 believes the NW is able to provide Uu RLC configuration since relay UE shall enter CONNECTED to relay remote UE’s SRB0 message. Meanwhile, [7], [9] and [13] also supports default configuration in case NW doesn’t provide dedicated configuration. There is no clear majority view. P1 in [39] propose RAN2 to discuss following question,

Question 1: which Uu RLC configuration is used for remote UE’s SRB0 message.

Option 1,	Fixed/specified.

Option 2,	Default,

Option 3,	NW configured.

| Company | Option | Comments |
| --- | --- | --- |
| Apple | Option 1 | We think it is simple for relay UE implementation to just use fixed/specified option for SRB0 in both PC5 and Uu hop. |

Regarding Uu RLC configuration for remote UE’s SRB1 message, companies’ views are observed as following,

Option 1, Fixed/specified	[33]

Option 2,	Default			[1], [6], [7], [9], [13], [36]

Option 3,	NW configured	[4], [7], [9], [13], [15], [20], [26], [36]

Only one company prefers fixed/specified configuration. 6 companies prefer to use default configuration. 8 companies prefer to use NW configured configuration. RAN2 had agreed Uu RLC configuration for remote UE’s SRB1 message such as RRCResume and RRCReestablishment message could be (re-)configured by NW via dedicated signalling. It’s still FFS whether default configuration is supported. P4 in [39] propose RAN2 to discuss following question,

Question 2: Do you agree default Uu RLC configuration for remote UE’s SRB1 message is supported.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes | It can be overridden by network configuration |

[7] further discuss which RLC configuration is used for RRCReconfigurationComplete in path switch to indirect path. P5 in [39] propose RAN2 to discuss following question,

Question 3: Do you agree dedicated signalling is used for the PC5 RLC and Uu RLC configuration of remote UE SRB1 for RRCReconfigurationComplete in path switch to indirect path.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes | This can be part of RRCReconfiguration message to the remote UE. |

## 2.3 Sharing of ID/DRX information for paging forward

In RAN2 114 meeting, it’s agreed that When Relay UE in RRC IDLE/RRC INACTVE and Remote UE in RRC IDLE/RRC INACTIVE, the Relay UE monitors paging occasions of its PC5-RRC connected Remote UE(s). However, it’s FFS how relay UE obtain remote UE’s paging occasions. Many companies support remote UE provides UE ID, i.e. 5G-S-TMSI/I-RNTI to relay UE. Relay UE could decode paging message to derive the UE ID and forward the paging message accordingly. P8/P9 in [39] propose RAN2 to discuss following question,

Question 4: Do you agree IDLE/INACTIVE remote UE provides 5G-S-TMSI/I-RNTI to IDLE/INACTIVE relay UE.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes | As SA3 has confirmed there is no security concern on exposing the 5G-S-TMSI/I-RNTI, it is OK to share those with relay UE. |

Question 5: Do you agree IDLE/INACTIVE Relay UE decodes received paging message to derive the 5G-S-TSMI/I-RNTI and forward the paging message accordingly.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes |  |

Relay UE shall be aware of remote UE’s PO to perform monitoring. [2], [8], [12], [27] propose to provide remote UE’s DRX cycle to relay UE. [21] propose to provide remote UE’s PO to relay UE. [30] believes the PO of remote UE is configured via SIB. However, according to 38.304 specification, the PO is calculated in MAC and the essential parameters, e.g. DRX cycle, may come from SIB or upper layer. It’s unclear how SIB could configure remote UE’s PO. P10 in [39] propose RAN2 to discuss following question,

Question 6: Do you agree IDLE/INACTIVE remote UE provide its Uu DRX cycle T to IDLE/INACTIVE relay UE.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes |  |

## 2.4 Connection establishment for relay and remote UE

If both remote UE and relay UE are not in CONNECTED, both UEs need to enter CONNECTED for service transmission. [9] assumes remote UE and relay UE performs connection establishment/resume independently. [33] propose to discuss combined relay UE and remote UE’s RRC connection establishment/assumption. Rapporteur understands second option may require additional impact in RAN1, since current msg3 is not enough for two RRC messages. Furthermore, in TR, it’s agreed if the Relay UE had not started in RRC_CONNECTED, it would need to do its own connection establishment upon reception of a message on the default L2 configuration on PC5. P10 in [39] propose RAN2 to discuss following question,

Question 7: Do you agree, as baseline, Remote UE and relay UE performs connection establishment/resume independently, i.e. relay UE shall enter CONNECTED to be able to forward remote UE’s initial RRC messages.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Apple | Yes |  |

# Report

Following proposals are made,

# Reference

[1] R2-2106989	Control Plane Procedures of L2 Relay	CATT

[2] R2-2107039	Discussion on Control Plane Aspects for L2 Relay	OPPO

[3] R2-2107044	Stage 2 level procedure for Connection Establishment	MediaTek Inc.

[4] R2-2107103	Further discussion on RRC connection management of L2 U2N relay	Qualcomm Incorporated

[5] R2-2107104	Further discussion on paging and SIB forwarding in L2 U2N relay	Qualcomm Incorporated

[6] R2-2107176	Remaining issues on RRC connection management	Samsung Electronics GmbH

[7] R2-2107231	Discussion on RRC connection management for L2 sidelink relay	Huawei, HiSilicon

[8] R2-2107232	SI forwarding and paging for L2 sidelink relay	Huawei, HiSilicon	discussion

[9] R2-2107273	Connection Establishment Procedure for L2 UE to NW Relays	InterDigital

[10] R2-2107274	Paging Procedures for L2 UE to NW Relays	InterDigital	discussion

[11] R2-2107275	SI Forwarding for L2 UE to NW Relays	InterDigital	discussion

[12] R2-2107304	Discussion on paging forwarding for a remote UE	SHARP Corporation	discussion

[13] R2-2107306	Remaining issues of L2 Relay connection management	Intel Corporation

[14] R2-2107367	Discussion on control plane procedures for L2 U2N relay	Spreadtrum Communications

[15] R2-2107541	Configuration of Uu Interface for Sidelink Relay	Futurewei

[16] R2-2107622	Remaining issues on SIB forwarding for IDLE/INACTIVE remote UE	Apple

[17] R2-2107623	Unified Access Control on Relay UE	Apple

[18] R2-2107625	RNA Update via L2 UE-to-NW relay	Apple

[19] R2-2107709	Paging delivery via L2 Relay in RRC_CONNECTED	Samsung

[20] R2-2107757	Way forward for L2 U2N Remote UE SRB0 SRB1 configuration	vivo

[21] R2-2107966	Discussion on SI and paging delivery	Xiaomi communications

[22] R2-2107967	Discussion on connection control	Xiaomi communications

[23] R2-2108007	SI acquisition, CN Registration and RNAU	Lenovo Mobile Com. Technology

[24] R2-2108008	Monitoring Paging by a U2N Relay	Lenovo Mobile Com. Technology

[25] R2-2108060	L2 relay control plane procedures	Sony	discussion

[26] R2-2108145	Consideration on the connection management of SL relay	ZTE, Sanechips

[27] R2-2108146	Consideration on the system information acquisition and paging in SL relay	ZTE, Sanechips

[28] R2-2108153	SIB Delivery & Paging for Remote UE 	LG Electronics Inc

[29] R2-2108154	Connection Establishment	LG Electronics Inc

[30] R2-2108192	Discussion on paging and SIB handling for L2 sidelink relay	Ericsson

[31] R2-2108195	Discussion on RRC connection management procedures for L2 SL relay	Ericsson

[32] R2-2108414	Discussion on SI and paging forwarding

[33] R2-2108458	Discussion on RRC connection establishment of remote UE in L2 U2N relay	Nokia, Nokia Shanghai Bell

[34] R2-2108462	Support of idle mode mobility for remote-UE in SL UE-to-Nwk relay	Nokia, Nokia Shanghai Bell

[35] R2-2108510	Control plane procedure	CMCC

[36] R2-2108250	Sidelink Relay Uu RLC for Remote UE and Adaptation Layer Design	Beijing Xiaomi Mobile Software

[37] R2-2108156	Relay reselection when Relay UE performs HO 	LG Electronics Inc

[38] R2-2108144 Further discussion on Relay selection 	ZTE, Sanechips

[39] R2-2108824 Summary of AI 8.7.2.1 Xiaomi Technology