3GPP TSG-RAN WG2 Meeting #115e	R2-2107180

Electronic, 9 Aug– 27 Aug 2021

Source: 			China Telecom

Title: 	Further discussion on introduction of paging cause

Agenda item:		8.3.4

Document for: 	Discussion and Decision

- Introduction

In RAN2 113bise meeting, the following conclusions are agreed for paging cause:

- 1: RAN2 works to support the MUSIM paging cause feature that SA2 is working on and also addresses the paging cause issue raised by SA2 LS. - 2: RAN2 attempts to reply LS to SA2 once we progress on solution and agree on CR(s) that support/address the above feature/issue. - 5: If RAN2 agrees to add a paging cause value (or any other information that could lead to a specific paging cause) in Uu paging message, RAN2 specifies the relevant UE behavior (i.e. inform or passing to the upper layer) upon its reception in both LTE and NR specifications. - RAN2 does not intend to introduce alternative paging IDs for MUSIM paging (unless requested by SA2).

As referred by the above agreements, SA2 LS [1] notify the necessity of discriminating paging for non-voice service from paging from network not supporting the paging cause.

SA2 has concluded that the UE shall be able to discriminate the case where it is being paged for non-voice service from the case where it is being paged (for any service) by a RAN node not supporting the Paging Cause, either because the RAN node does not support the Paging Cause feature, or because in case of RAN sharing it is configured to operate without the Paging Cause feature for some of the connected CNs.

In this paper, we give discussion on remaining issues related to paging cause.

- Discussion

The SA2 has concluded that UE shall be able to discriminate the case where it is being paged for non-voice service from the case where it is being paged (for any service) by a RAN node not supporting the Paging Cause. We will analyse different scenarios for this requirement from UE and RAN perspective.

1) Legacy RAN node

The paging message to all UEs will not contain paging cause IE or Rel 17 paging record extension. The UE will perceive paging message in a legacy way as for any service. So there is no additional requirement for UE under legacy RAN node.

Observation 1: UE will perceive paging message in a legacy way under RAN node not supporting the Paging Cause.

2) RAN node supporting paging cause

UE under RAN node supporting paging cause should be able to handle three kinds of paging message corresponding to Scenario1/2/3.

RAN node supporting paging cause should be able to handle 4 scenarios as below.

Table 1: 4 different paging scenarios under RAN node supporting paging cause

|  | Scenario1: CN support paging cause Voice paging to UE with MUSIM capabilities | Scenario2: CN support paging cause Non voice paging to UE with MUSIM capabilities | Scenario3: CN don’t support paging cause Voice paging | Scenario 4: CN support paging cause Paging to UE without MUSIM capabilities |
| --- | --- | --- | --- | --- |
| Requirement for RAN node | RAN node should include the Voice Service indication as the paging cause in the paging message to the UE | RAN node should indicate UE that the paging is a non-voice call | RAN node should indicate UE this is a legacy paging message. | RAN node should indicate UE this is a legacy paging message. |
| Requirement for UE with MUSIM capabilities | Voice paging cause should be recognized | Paging should be recognized as non-voice call | Paging should be recognized as a legacy call (for any service) | N/A |

As agreed by SA2#145E meeting for the Introduction of Paging Cause feature [3]:

If the UE indicates support for the Paging Cause Indication for Voice Service feature in the Registration Request message and if the network supports and intends to apply the Paging Cause Indication for Voice Service feature for the UE, the AMF includes an indication that the UE supports the Paging Cause Indication for Voice Service feature in the N2 message carrying the Registration Accept message

For idle mode:

If the network supports the Paging Cause Indication for Voice Service feature and if the UE context in the AMF indicates that the UE supports the Paging Cause Indication for Voice Service feature, the AMF should provide the Voice Service Indication in the NGAP Paging message only when the AMF detects that the downlink data which triggers the Paging message is related to voice service, as specified in clause 5.X.Y in TS 23.501 [2]. If the NG RAN supporting the Paging Cause Indication for Voice Service feature receives the Voice Service Indication, it provides the Voice Service Indication in the Paging message and sends the Paging message to the UE.

For inactive mode:

If the NG RAN supports the Paging Cause Indication for Voice Service feature and if the UE context in NG-RAN indicates that the UE supports the Paging Cause Indication for Voice Service feature, the NG RAN shall provide the Voice Service Indication in the RAN Paging message for the UE when it detects that the downlink data which triggers the RAN Paging message is related to voice service, as specified in clause 5.X.Y in TS 23.501 [2].

We can see that only in Scenario1 and Scenario 2 AMF will indicate the UE support Paging Cause Indication for Voice Service feature by which the RAN can discriminate scenario 1&2 from scenario 3&4. Only in Scenario 1 AMF will include Voice Service Indication in NG-AP paging message.

Observation 2: RAN node can discriminate non-voice paging from paging of legacy CN or paging to legacy UE from the Paging Cause Indication for Voice Service feature from AMF.

For RAN node supporting paging cause, paging message of these four scenarios listed in Table 1 may be included in one paging record list. For scenario1, the paging record should indicate Voice paging cause. For scenario2 the paging record should indicate it is a non-voice call. For Scenario 3 and Scenario 4 the paging record should indicate it is a legacy paging message.

As discussed in RAN 113bise meeting [2], both Solution Direction A (impact only LTE/NR RRC Uu paging message) and Solution Direction B (LTE/NR RRC Uu paging message+ additional signalling e.g. via SIB, paging DCI, NAS) will be considered.

As agreed by SA2#145E meeting for the Introduction of MUSIM capability exchange [4]:

If the Multi-USIM UE has indicated support for one or more Multi-USIM Specific Capabilities in the UE 5GMM Core Network Capability in step 1, the AMF shall indicate to the Multi-USIM UE whether the corresponding one or more Multi-USIM specific features described in clause  5.x of TS 23.501 [2] are supported, based on  network capability and preference by the network (i.e. based on local network policy), by providing one or more of the Connection Release Supported, Paging Cause Supported, Paging Restriction Supported and Reject Paging  Supported indications.

CN will provide Paging Cause Supported indicator through NAS message from which the UE can know whether the core network support paging cause or not. Along with the presence of extended paging record list the UE can discriminate non-voice call from legacy call. For this solution, only 1 bit is needed for each record to represent voice call or non-voice call. This solution save signaling overhead at the expense of UE complexity as it need AS layer and NAS layer coordination.

If RAN2 agree to go with this direction, we prefer the solution below:

Introduce a parallel pagingRecordList-v17xy with the same number of entries as PagingRecordList (without suffix). For each record within PagingRecordList-v17xy:

1) 1 bit voice paging indication can represent voice call.

If a parallel pagingRecordList-v17xy is included, the UE can deduce that the RAN node support Paging Cause feature. Voice indication present for intended UE means it is a voice paging. If voice indication absence, UE can further distinguish the non-voice paging with paging from CN don’t support paging cause via NAS Paging Cause Supported indicator.

Proposal 1: RAN2 consider the solution of voice paging indication in Uu paging message combined with Paging Cause Supported indicator in NAS message to discriminate voice paging, non-voice paging and legacy paging.

Proposal 2: If Solution of Uu paging message combined with NAS indicator is adopted, a parallel pagingRecordList-v17xy is applied with 1 bit voice paging indication for each record.

If RAN2 agree to go with direction A, we prefer the solution below:

Introduce a parallel pagingRecordList-v17xy with the same number of entries as PagingRecordList (without suffix). For each record within PagingRecordList-v17xy:

1) 1 bit paging cause present indication can be used to indicate whether this record is for a Rel 17 extended paging message or legacy paging message (for any service).

2) 1 bit paging cause value can represent voice call or non-voice call

The proposed ASN.1 text proposal is shown in Appendix.

Proposal 3: If Solution Direction A (impact only LTE/NR RRC Uu paging message) is adopted, a parallel pagingRecordList-v17xy is applied with 1 bit paging cause present indication for each record to discriminate Rel 17 extended paging message from legacy paging message (for any service) and 1 bit paging cause value represent voice call or non-voice call.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: UE will perceive paging message in a legacy way under RAN node not supporting the Paging Cause.

Observation 2: RAN node can discriminate non-voice paging from paging of legacy CN or paging to legacy UE from the Paging Cause Indication for Voice Service feature from AMF.

Proposal 1: RAN2 consider the solution of voice paging indication in Uu paging message combined with Paging Cause Supported indicator in NAS message to discriminate voice paging, non-voice paging and legacy paging.

Proposal 2: If Solution of Uu paging message combined with NAS indicator is adopted, a parallel pagingRecordList-v17xy is applied with 1 bit voice paging indication for each record.

Proposal 3: If Solution Direction A (impact only LTE/NR RRC Uu paging message) is adopted, a parallel pagingRecordList-v17xy is applied with 1 bit paging cause present indication for each record to discriminate Rel 17 extended paging message from legacy paging message (for any service) and 1 bit paging cause value represent voice call or non-voice call.

- References

- S2-2102039 LS on System support for Multi-USIM devices

- R2-2104331 Summary of [AT113bis-e][230][MUSIM] Reply LS to SA2 on paging cause, Intel

- S2-2105120 Introduction of Paging Cause feature

- S2-2105118 Introduction of MUSIM capability exchage

Appendix: Text proposal

38.331

#### –	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: Network to UE

Paging message

-- ASN1START

-- TAG-PAGING-START

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v17xy-IEs                                                              OPTIONAL

}

Paging-v17xy-IEs ::=			SEQUENCE {

pagingRecordList-v17xy			PagingRecordList-v17xy				OPTIONAL,	-- Need N

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecordList-v17xy ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-v17xy

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

PagingRecord-v17xy ::=				SEQUENCE {

pagingCause-r17					ENUMERATED { voice, nonvoice}				OPTIONAL	-- Need N

}

PagingUE-Identity ::=               CHOICE {

ng-5G-S-TMSI                        NG-5G-S-TMSI,

fullI-RNTI                          I-RNTI-Value,

...

}

-- TAG-PAGING-STOP

-- ASN1STOP

PagingRecord field descriptions

accessType Indicates whether the Paging message is originated due to the PDU sessions from the non-3GPP access.

pagingCause It indicates whether the paging is triggered by a voice service or not. This indication applies only to UEs report Multi-USIM Mode Indication as specified in TS 23.501[32].

36.331

–	Paging

The Paging message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: E-UTRAN to UE

Paging message

-- ASN1START

Paging ::=					SEQUENCE {

pagingRecordList				PagingRecordList					OPTIONAL,	-- Need ON

systemInfoModification			ENUMERATED {true}					OPTIONAL,	-- Need ON

etws-Indication					ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v890-IEs						OPTIONAL

}

Paging-v890-IEs ::=			SEQUENCE {

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			Paging-v920-IEs						OPTIONAL

}

Paging-v920-IEs ::=			SEQUENCE {

cmas-Indication-r9				ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1130-IEs					OPTIONAL

}

Paging-v1130-IEs ::=			SEQUENCE {

eab-ParamModification-r11		ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1310-IEs					OPTIONAL

}

Paging-v1310-IEs ::=			SEQUENCE {

redistributionIndication-r13	ENUMERATED {true}					OPTIONAL,	-- Need ON

systemInfoModification-eDRX-r13	ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1530-IEs					OPTIONAL

}

Paging-v1530-IEs ::=			SEQUENCE {

accessType						ENUMERATED {non3GPP}				OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v1610-IEs					OPTIONAL

}

Paging-v1610-IEs ::=			SEQUENCE {

pagingRecordList-v1610			PagingRecordList-v1610				OPTIONAL,	-- Need ON

uac-ParamModification-r16		ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension			Paging-v17xy-IEs							OPTIONAL

}

Paging-v17xy-IEs ::=			SEQUENCE {

pagingRecordList-v17xy			PagingRecordList-v17xy				OPTIONAL,	-- Need ON

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

PagingRecordList ::=				SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord

PagingRecordList-v1610 ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-v1610

PagingRecord ::=					SEQUENCE {

ue-Identity							PagingUE-Identity,

cn-Domain							ENUMERATED	{ps, cs},

...

}

PagingRecordList-v17xy ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-v17xy

PagingRecord-v1610 ::=				SEQUENCE {

accessType-r16						ENUMERATED {non3GPP}			OPTIONAL,		-- Need ON

mt-EDT-r16							ENUMERATED {true}				OPTIONAL		-- Need ON

}

PagingRecord-v17xy ::=				SEQUENCE {

pagingCause-r17					ENUMERATED { voice, nonvoice}				OPTIONAL	-- Need ON

}

PagingUE-Identity ::=				CHOICE {

s-TMSI								S-TMSI,

imsi								IMSI,

...,

ng-5G-S-TMSI-r15					NG-5G-S-TMSI-r15,

fullI-RNTI-r15						I-RNTI-r15

}

IMSI ::=							SEQUENCE (SIZE (6..21)) OF IMSI-Digit

IMSI-Digit ::=						INTEGER (0..9)

-- ASN1STOP

Paging field descriptions

accessType It indicates whether Paging is originated due to the PDU sessions from the non-3GPP access when E-UTRA is connected to 5GC. E-UTRAN does not include both accessType (i.e., without suffix) and accessType-r16 in a single paging message.

cmas-Indication If present: indication of a CMAS notification.

cn-Domain Indicates the origin of paging.

eab-ParamModification If present: indication of an EAB parameters (SIB14) modification.

etws-Indication If present: indication of an ETWS primary notification and/ or ETWS secondary notification.

imsi The International Mobile Subscriber Identity, a globally unique permanent subscriber identity, see TS 23.003 [27]. The first element contains the first IMSI digit, the second element contains the second IMSI digit and so on.

mt-EDT Indication of mobile terminating EDT.

pagingRecordList If E-UTRAN includes pagingRecordList-v1610, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix).

redistributionIndication If present: indication to trigger E-UTRAN inter-frequency redistribution procedure as specified in TS 36.304 [4], clause 5.2.4.10.

systemInfoModification If present: indication of a BCCH modification other than SIB10, SIB11, SIB12 and SIB14. This indication does not apply to UEs using eDRX cycle longer than the BCCH modification period.

systemInfoModification-eDRX If present: indication of a BCCH modification other than SIB10, SIB11, SIB12 and SIB14. This indication applies only to UEs using eDRX cycle longer than the BCCH modification period.

uac-ParamModification If present: indication of UAC parameters (SIB25) modification.

ue-Identity Provides the NAS identity of the UE that is being paged. The IMSI is not applicable for E-UTRA/5GC.

pagingCause It indicates whether the paging is triggered by a voice service or not. This indication applies only to UEs report Multi-USIM Mode Indication as specified in 23.401[41].