3GPP TSG-RAN WG2 Meeting #115-e	R2-2107552

# Electronic meeting, August 16 – 27, 2021

Agenda item: 	8.18

Source:		Intel Corporation

Title:	Common aspects of RACH partitioning

Document for:	Discussion and Decision

# Introduction

RACH partitioning is being proposed or agreed in multiple Wis: Slicing, RedCap, SDT and coverage enhancement. In RAN2#114, it is agreed to discuss this topic in a common session.

In this contribution, the status related to RACH is reviewed for each topic to understand how far RACH partitioning is agreed and any common areas that can be discussed in this session. In addition, we provide our view on how to discuss the common RACH issues from the multiple WIs.

# RACH related status in different WIs

The following shows the status of the RACH agreements in various Wis from the RAN1 and RAN2 Chair’s notes [1]:

| WI name | Status |
| --- | --- |
| Slicing enhancements | RAN2#113bis-e Agreements: 1 RAN2 aims to support both RO partition and preambles partition. 2 scalingFactorBI and powerRampingStepHighPriority can be configured at least in SIB (FFS for dedicated RRC signalling). 3 Network can configure slices with 4-step or 2-step (or both) RA resources. 4 Legacy 2-step RA fallback mechanism is supported. RAN2#114-e Agreements: - RAN2 confirm for a slice group, separated RO and/or separate preamble can be configured within the existing RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA - Same as NR Rel-15 conclusion, RAN2 conclude that there is no RA-RNTI collision between slice specific RACH and legacy RACH in shared RO - Same as NR Rel-15 conclusion, RAN2 conclude that the RA-RNTI collision between slice specific RACH and legacy RACH may happen in separate RO. - Working assumption: this can be left to network implementation to resolve it (e.g. network configure RO in different time) - FFS how many slice groups we can have and how they are indicated. There is an email discussion (252) discussing RACH type selection and fallback mechanism to Msg1 |
| RedCap | RAN2#113-e agreements: RAN2 discussed early identification, and agreed it is as one of option for option 4 as below Agreements via email - from offline [108] - Capture following text in 11.1 in description of Option 4: “Option 4: During MsgA transmission - E.g., via separate initial UL BWP or in MsgA preamble part via separate PRACH resource or PRACH preamble partitioning, or in MsgA PUSCH part.” Also as captured in the TR (R2-2102056), separate PRACH resources or PRACH preamble partitioning are candidate solutions for option 1 - Option 1: During Msg1 transmission - E.g., via separate initial UL BWP, separate PRACH resource, or PRACH preamble partitioning The further down selection is expected during WI phase (RP-210918) - Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1] RAN2#114-e agreements: - Either Msg1 and/or Msg3 early identification will be supported RAN1 Working assumption and agreements: Working assumption: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled - FFS How to support enable/disable the early indication - FFS details e.g.: - separate initial UL BWP - separate PRACH resource - PRACH preamble partitioning - FFS the possibility of supporting Msg3 for the early indication Agreement: - Support 2-step RACH for RedCap UEs as an optional feature - FFS details of early indication in MsgA, e.g.: - Separation of 2-step RACH resources or MsgA preambles - Separation of initial UL BWP - Using a new indication in MsgA PUSCH part - Note: Discussion on 4-step RACH for early indication should be prioritised |
| SDT | RAN2#112-e Agreement: As a baseline, the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT If ROs for SDT and non SDT are different, preamble partitioning between SDT and non SDT is not needed. If ROs for SDT and non SDT are same, preamble partitioning is needed FFS if common configuration should be allowed RAN2#113-e Agreement: RAN2 continues to progress the work based the separate RACH resources for SDT (i.e. explicit mechanisms to support common resources won’t be pursued unless there is sufficient support for this. However, use of common RACH resources will not be precluded if possible via implementation. RAN2#114-e Agreement: The separate search space is common to the UEs performing RA-SDT. Inform RAN1 of this agreement RAN2 LS to RAN1: R2-2106561: For RA-SDT (for both 2-step RA and 4-step RA), in particular, RAN2 would like to request RAN1 to provide configuration parameters for the PRACH resource configuration when PRACH occasions are shared between SDT and non-SDT and when PRACH occasions are separately configured for SDT and non-SDT. FFS were captured in relation to the RACH fallback scenarios in email discussion [POST114-e][506]: FFS Random Access Preamble group selection for RA type switching from RA-SDT to non-RA-SDT and from 2-stepRA-SDT to 4-stepRA-SDT. |
| Coverage Enhancement | The following was agreed in RAN1#104bis-e: For Msg3 PUSCH repetition, support the following modified Option 2-1. - Option 2-1: For UE requested triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - A UE can request trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.). - Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission. In RAN1#105-e - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission. |

Based on the table above, the followings are some observations on the RAN2 RACH related agreements:

Observation#1: For Slicing and SDT WIs, RACH partitioning with shared RO and separate RO is agreed (See green highlight). For Coverage enhancement WI, only RACH partitioning with shared RO is agreed (See blue highlight). However, for RedCap Wi, it is still with FFS (See yellow highlight)

Observation#2: For Slicing WI, the number of partitioning (i.e. to differentiate just between critical slice group and normal slice group or among multiple slice group) is also FFS

Observation#3: For Slicing WI, fallback mechanism to Msg1 and RACH type selection are being discussed within the WI. For SDT WI, the fallback mechanism to Msg1 is left as FFS in the email disc [506].

For Observation#2, the number of partitioning for slicing WI should still be discussed in the individual WI since the requirement on whether there is a need for it lies in those Wis.  As for Observation#3, whether to fallback from slice specific RACH to common RACH or whether the RACH type selection can be between slice specific RACH and common RACH can be further discussed in the individual WIs. On the other hand, for Observation#1, if RACH partitioning is to be done for a WI, it should be discussed in the common session so that the approach to RACH partitioning is done the same and coordinated.

Another thing to discuss in the common session is on the combinations of WIs/features for the RACH partitioning.  There are some features/WI that may need to be supported at the same time (e.g. RedCap with SDT, SDT with slicing etc.). Such cross WIs interaction should be discussed in the common session.

Proposal#1: RAN2 can start with discussing the RACH partitioning and its supporting combinations of WIs/features for the RACH partitioning in the common session.

In the following sections, RACH partitioning and its supporting combinations of WIs/features for the RACH partitioning is further discussed.

# Existing RACH preamble partitioning

RACH preamble partitioning is already supported in Re-15 for the UE to indicate to network that it can use Group A or Group B Msg3 size and in Rel-16 to differentiate 4-step RACH and 2-step RACH in the case of shared RO. For 2-step RACH, it also allows the differentiation with 4-step RACH via using separate PRACH configuration. The following is how current RACH preamble partitioning is configured for GroupA/B and 2-step RACH when 2-step RACH and 4-step RACH shared the ROs:

Figure 1: Existing preamble partitioning in shared RO

The above can be further partitioned to take into consideration of the partitioning required by slicing, SDT, Coverage enhancement, RedCap and their combinations in the case of shared RO among the different WI by partitioning further on the CF preambles space in each of the SSB as follow:

Figure 2: Existing preamble partitioning in shared RO extended to consider SDT and other WI

As agreed for slicing and SDT, it is also possible to use separate RO for the differentiation between SDT and non-SDT. In this approach, a new prach-ConfigurationIndex is used for separate PRACH resources of other WIs/features in order to be able to separate the RO from the common 2-step and 4-step RACH. An illustration using slicing is as follow for differentiating between critical slice group and other slice groups using separate RO:

Figure 3: Separate RO between the RO of the critical slices and the common 2 step and 4 step RACH

How to structure in the RRC signalling for the extending of the shared RO and separate RO for the different WI and their combinations can be discussed further in this common session.

Proposal#2: Similar to the Rel-16 2-step RACH, for the shared RO case, the different features and their combinations can be included into the preamble space reserved for CFRA. For the separate RO case, a new prach-ConfigurationIndex can be used. How to structure in the RRC signalling for the extending of the shared RO and separate RO for the applicable/different WIs and their combinations can be discussed further in this common session.

# Combinations of Rel-17 features/WI for RACH partitioning

Other than partitioning on a per Rel-17 features basis, some of the Rel-17 features may make logical sense in combinations. For example, it is logical to indicate RedCap and SDT in combination as RedCap UE may benefit from using small data transmission. Similarly, it may also be useful to indicate the slice group for RedCap UE or for UE performing SDT for faster access in IIoT use case. As in eMTC, RedCap UE used in MTC use case may require support of coverage enhancement. On the other hand, SDT requires that the UE is in good coverage in order to send small data during RA procedure and thus does not combine with coverage enhancement. The following is our initial view of the combinations of Rel-17 features/WI which RACH partitioning maybe possible in our view.

|  | Slicing | SDT | RedCap | CovEnh |
| --- | --- | --- | --- | --- |
| Slicing | N/A | Yes | Yes | Yes |
| SDT | Yes | N/A | Yes | No |
| RedCap | Yes | Yes | N/A | Yes |
| CovEnh | Yes | No | Yes | N/A |

Proposal#3: The RACH partitioning design can consider the following combinations of Rel-17 features as starting point for discussion and further down-select the combinations:

|  | Slicing | SDT | RedCap | CovEnh |
| --- | --- | --- | --- | --- |
| Slicing | N/A | Yes | Yes | Yes |
| SDT | Yes | N/A | Yes | No |
| RedCap | Yes | Yes | N/A | Yes |
| CovEnh | Yes | No | Yes | N/A |

# Conclusion

It is requested that RAN2 agree to the following proposals and observations:

Observation#1: For Slicing and SDT WIs, RACH partitioning with shared RO and separate RO is agreed (See green highlight). For Coverage enhancement WI, only RACH partitioning with shared RO is agreed (See blue highlight). However, for RedCap Wi, it is still with FFS (See yellow highlight)

Observation#2: For Slicing WI, the number of partitioning (i.e. to differentiate just between critical slice group and normal slice group or among multiple slice group) is also FFS

Observation#3: For Slicing WI, fallback mechanism to Msg1 and RACH type selection are being discussed within the WI. For SDT WI, the fallback mechanism to Msg1 is left as FFS in the email disc [506].

Proposal#1: RAN2 can start with discussing the RACH partitioning and its supporting combinations of WIs/features for the RACH partitioning in the common session.

Proposal#2: Similar to the Rel-16 2-step RACH, for the shared RO case, the different features and their combinations can be included into the preamble space reserved for CFRA. For the separate RO case, a new prach-ConfigurationIndex can be used. How to structure in the RRC signalling for the extending of the shared RO and separate RO for the applicable/different WIs and their combinations can be discussed further in this common session.

Proposal#3: The RACH partitioning design can consider the following combinations of Rel-17 features as starting point for discussion and further down-select the combinations:

|  | Slicing | SDT | RedCap | CovEnh |
| --- | --- | --- | --- | --- |
| Slicing | N/A | Yes | Yes | Yes |
| SDT | Yes | N/A | Yes | No |
| RedCap | Yes | Yes | N/A | Yes |
| CovEnh | Yes | No | Yes | N/A |

# References

[1]  RAN1 and RAN2 Chair’s notes