3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2107718

Online, 16th – 27th August 2021

Agenda Item:	8.13.2.2

Source: 	vivo

Title:         	Discussion on remaining issues of 2-step RACH report

Document for: 	Discussion and Decision

- Introduction

RAN2#113b-e [1]and 114-e meeting [2] discussed the contents of 2-step RA and made the following agreements:

113b-e Agreements: 1 The RA report includes an explicit indication per RA attempt that enables the network to know that the fallback from 2-step RA to 4-step RA was performed by the UE. 2 RAN2 already agreed “UE includes the measured RSRP of DL pathloss reference obtained just ‎before performing RACH ‎procedure in 2step RA report. FFS how to reduce the report ‎overhead.‎” 114-e Agreements: 1 If a RA procedure switching from 2-step RA to 4-step RA occurs, one RA report entry is used to convey RA information for 2-step RA and 4-step RA attempts. 2 To introduce 2-step RACH related information in RACH report: enhance the legacy field ra-InformationCommon to include 2-step RA related information. FFS the detailed information.

In the offline email [3], several FFSs are left for further discussions:

Proposal 2A: FFS whether RA type ‎per RA procedure is known ‎implicitly by NW or indicated ‎explicitly in RA report. Proposal 3A: FFS how network knows (i.e., explicitly or impliclty) whether ‎switching from 2-step RA to 4-step ‎RA is performed by UE due to ‎reaching a configured MSGA ‎transmission times.‎

In this contribution, we would like to address these remaining issues of 2-step RACH report.

- Discussion

  - Indication on RA type ‎per RA procedure

It is FFS that whether RA type ‎per RA procedure is known ‎implicitly by NW or indicated ‎explicitly in RA report. Note that RAN2 agreed the following parameters will be included in RA report:

- msgA-FrequencyStart-r17 - msgA-FrequencyStartCFRA-r17 - msgA-SubcarrierSpacing-r17 - msgA-SubcarrierSpacingCFRA-r17 - msgA-FDM-r17 - -msgA-FDMCFRA-r17 - UE includes the measured RSRP of DL pathloss reference obtained just before performing RACH ‎procedure in 2step RA report

The above parameters can be used to distinguish between the 2-step RA procedure and 4-step RA procedure, because the inclusion of such parameters are 2-step specific. Thus RA type ‎per RA procedure can be known ‎implicitly by NW in RA report.

- RAN2 introduced 2-step RA specific parameters which can be used to indicate the RA type.

With the inclusion of 2-step RA specific parameters, RA type ‎per RA procedure is known ‎implicitly by NW in RA report.

  - Switch indication (explicit vs. implicit)

There are only the following two cases where 2-step RA will terminate and be changed to 4-step RA process:

- Fallback mechanism: UE performs MsgA transmission while receives MsgB containing fallback RAR, subsequently UE falls back to 4-step RA by performing Msg3 transmission;

- Switch mechanism: UE performs MsgA transmission but receives no response from gNB, or receives fallback RAR but ultimately fails to receive msg4, then UE will try to perform 2-step RA again, once the UE achieves the maximum number of MsgA preamble transmissions (msgA-TransMax), it will switch to 4-step random access.

It was agreed to include an explicit flag in RA report to indicate UE has experiecend a fallback, however, whether an explicit flag is also needed for the switch case has left as an FFS. The key point here is whether network can derive whether UE has switched to 4-step RA from any exisiting knowledge. Note that UE will switch to 4-step random access once it achieves the maximum number of MsgA preamble transmissions (msgA-TransMax), while the the parameter msgA-TransMax is configured by NW.

Nevetheless this IE can be either cell-specific (BWP-UplinkCommon-> MsgA-ConfigCommon-> RACH-ConfigCommonTwoStepRA -> msgA-TransMax) or UE-specific (via RACH-ConfigDedicated). In the following, we will discuss the issue case by case.

- msgA-TransMax can be configured as a cell-specific or UE-specific parameter.

- Cell-specific case

The ID of the cell where the RA is performed will be included in the report, NW can be aware of the configured msgA-TransMax once it receives the RA report. Consequently, NW may derive that whether the UE had switched to 4-step RA according to the PerRAInfo entries recorded within the report. For instance, NW only needs to compare the the number of PerRAInfo entries and the configured msgA-TransMax, if the former is not greater than the latter, then it means UE didn’t switch to 4-step RA during the whole procedure.

However, whether the NW can be able to derive the situation or not actually depends on the stage-3 design of PerRAInfo for 2-step RA. At last meeting it was agreed that to enhance the legacy field ra-InformationCommon to include 2-step RA related information, this implies that perRAInfoList would be extended to accommodate both 2-step and 4-step relevant PerRAInfo.

Enhance the legacy field ra-InformationCommon

Currently no fields in PerRAInfo can be used to differentiate whether this entry is created by 2-step or 4-step RACH attempt (see ASN.1 code attached below). Note that we agreed ‘The reporting granularity of whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB is per-RA-attempt.’, but whether this is indicated by reusing the legacy field dlRSRPAboveThreshold or a new field has not been decided. If a new field is used to indicate the 2-step RA case, then the network can understand that this entry is created due to a 2-step RACH attempt, otherwise it remains an impossible task for network to derive.

PerRAInfoList-r16 ::= SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16 PerRAInfo-r16 ::= CHOICE { perRASSBInfoList-r16 PerRASSBInfo-r16, perRACSI-RSInfoList-r16 PerRACSI-RSInfo-r16 } PerRASSBInfo-r16 ::= SEQUENCE { ssb-Index-r16 SSB-Index, numberOfPreamblesSentOnSSB-r16 INTEGER (1..200), perRAAttemptInfoList-r16 PerRAAttemptInfoList-r16 } PerRACSI-RSInfo-r16 ::= SEQUENCE { csi-RS-Index-r16 CSI-RS-Index, numberOfPreamblesSentOnCSI-RS-r16 INTEGER (1..200) } PerRAAttemptInfoList-r16 ::= SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16 PerRAAttemptInfo-r16 ::= SEQUENCE { contentionDetected-r16 BOOLEAN OPTIONAL, dlRSRPAboveThreshold-r16 BOOLEAN OPTIONAL, ... }

- If msgA-TransMax is configured as a cell-specific parameter, NW may derive that whether the UE had switched to 4-step RA according to the PerRAInfo entries recorded within the report. But the premise is that the field(s) in PerRAInfo can be used to differentiate whether this entry is created by 2-step or 4-step RACH attempt (which depends on the signalling design at stage-3).

- A new field (similar to the legacy field dlRSRPAboveThreshold) can be used to indicate whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB per-RA-attempt, so that the entry could be identified as a 2-step RA attempt.

- UE-specific case

Since the parameter msgA-TransMax is not configured on the basis of cell, network is not in a position to always have the dedicated value of msgA-TransMax. Thus it is straight-forward for the UE to report assistant information to help network identify whether switch was performed or not. Basically there are two options available, one option is to report the value of msgA-TransMax, another is to directly carry a flag, which is similar to the fallback situation, indicating a swtich has occurred to UE. The latter option obviously requires less overhead (1 bit is enough) compared to the former one (need 4 bits to include 10 candidate values).

- If msgA-TransMax is configured as a UE-specific parameter, network is not in a position to have the dedicated value of msgA-TransMax. UE should report assistant information to help network identify whether switch was performed or not.

- The assistance information can be either the value of msgA-TransMax or a flag (similar to the fallback situation), but the latter one requires less overhead.

- Summary

However, it also occurs to us that maybe no explicit indicatin is needed for both cases, the rationale behind is:

- there are only two cases, i.e., fallback and switch situations, where 4-step and 2-step frequency-related parameters coexist;

- it was agreed to explicitly indicate the fallback case;

- thus it can be derived whether the switch was performed based on the coexistence of 4-step and 2-step frequency-related parameters, and the explicit indication for fallback. For instance, if network receives a RA report with both 4-step and 2-step parameters, and there is no indication for fallback, then network could deduce that a switch has occurred.

- There are only two cases, i.e., fallback and switch situations, where 4-step and 2-step frequency-related parameters coexist.

- It can be derived whether the switch was performed based on the coexistence of 4-step and 2-step frequency-related parameters, and the explicit indication for fallback. For instance, if network receives a RA report with both 4-step and 2-step parameters, and there is no indication for fallback, then network could deduce that a switch has occurred.

Switch indication is not needed for 2-step RA report as network impliclty knows whether ‎switching from 2-step RA to 4-step ‎RA is performed based on the coexistence of 4-step and 2-step frequency-related parameters and the explicit indication for fallback.

- Conclusion

In this paper, the following observations and proposal are given:

Observation 1	RAN2 introduced 2-step RA specific parameters which can be used to indicate the RA type.

Observation 2	msgA-TransMax can be configured as a cell-specific or UE-specific parameter.

Observation 3	If msgA-TransMax is configured as a cell-specific parameter, NW may derive that whether the UE had switched to 4-step RA according to the PerRAInfo entries recorded within the report. But the premise is that the field(s) in PerRAInfo can be used to differentiate whether this entry is created by 2-step or 4-step RACH attempt (which depends on the signalling design at stage-3).

Observation 4	A new field (similar to the legacy field dlRSRPAboveThreshold) can be used to indicate whether the DL beam quality, associated to the used 2 step RA resource, is above or below the msgA-RSRP-ThresholdSSB per-RA-attempt, so that the entry could be identified as a 2-step RA attempt.

Observation 5	If msgA-TransMax is configured as a UE-specific parameter,

Observation 6	The assistance information can be either the value of msgA-TransMax or a flag (similar to the fallback situation), but the latter one requires less overhead.

Observation 7	There are only two cases, i.e., fallback and switch situations, where 4-step and 2-step frequency-related parameters coexist.

Observation 8	It can be derived whether the switch was performed based on the coexistence of 4-step and 2-step frequency-related parameters, and the explicit indication for fallback. For instance, if network receives a RA report with both 4-step and 2-step parameters, and there is no indication for fallback, then network could deduce that a switch has occurred.

Proposal 1	With the inclusion of 2-step RA specific parameters, RA type ‎per RA procedure is known ‎implicitly by NW in RA report.

Proposal 2	Switch indication is not needed for 2-step RA report as network impliclty knows whether ‎switching from 2-step RA to 4-step ‎RA is performed based on the coexistence of 4-step and 2-step frequency-related parameters and the explicit indication for fallback.

- References

- R2-2104306	Report from SON/MDT session, Report, Session chair (CMCC) WG2 #113b-e, Electronic meeting, April 12th – April 20th 2021

- R2-2106476,	Report from SON/MDT session, Report, Session chair (CMCC) WG2 #114-e, Electronic meeting, May 17th – May 27th 2021

- R2-2104536	Report of [AT113b-e][802][NR/R17 SON/MDT]  2 step RA and other SON enhancements (CATT)	CATT