3GPP TSG-RAN WG2 Meeting #115-e	R2-2107885

Online, 16 – 27 August 2021

Agenda item:	 8.13.2.1

Source:		Lenovo, Motorola Mobility

Title:		SON Enhancements for SHR

Document for:		Discussion and Decision

1	Introduction

In RAN2#113 bis-e [1], it agreed that:

4	At least the following triggering conditions are applied for generating a HO Success Report in the case that the HO succeeds:

a.	The UE logs the HO success report if, while doing HO, T310 value exceeds a threshold

b.	The UE logs the HO success report if, while doing HO, T312 value exceeds a threshold

c.	The UE logs the HO success report if, while doing HO, T304 exceeds a threshold

d.	In case of DAPS, if the UE gets an RLF in the source while doing DAPS.

Agreements:

1	RAN2 to focus on the following scenarios for HO Success Report:

a.	Scenario 1 (ordinary HO): 1a, 1b

b.	Scenario 2 (CHO): 2a, 2b

c.	Scenario 3 (DAPS): 3a

2	RAN2 for further discuss whether the following scenarios should be considered under the RLF report or under the HO success report:

a.	Scenario 2c

b.	Scenario 3b

3	The following radio related measurements are as part of the successful HO report:

a.	Latest radio measurement results of the candidate target cells in the case of conditional HO. FFS best cell(s) should be included in.

b.	Flag to indicate RLF issues in source cell during DAPS HO

4	The following time-related measurements are as part of the successful HO report:

a.	Time elapsed between the CHO execution towards the target cell and the corresponding latest CHO configuration received for the selected target cell

5	Location information is included as part of the successful HO report.

In RAN2#114-e [2], more were achieved:

Agreements:

31	The UE does not log SHR if no triggering conditions are configured.

32	The UE generates Successful HO report upon exceeding thresholds on T310, T312 and T304 exceed also for CHO case (in addition to regular HO)

34	The UE indicates in the SHR which triggering conditions for generating the SHR were fulfilled, e.g. flag for T310, T304, T312 indications.

35	Include in the SHR, the latest radio link quality of neighbour cells before HO execution for all HO types.

36	For location config/reports for SHR, location info for RLF report can be reused.

38	UE logs successful HO report in case prior configuration is received for successful HO report (interested trigger and corresponding configuration), otherwise UE doesn’t store successful HO report.

39	The varSuccHOReport is introduced to store the parameters for successful HO report.

40	The UE includes the availability of successful HO report to NW in each completed message send in RRC procedure, i.e., RRCReconfigurationComplete, RRCReestablishmentComplete, RRCSetupComplete, RRCResumeComplete message if it has available successful HO report to be reported.

41	UEInformationRequest/UEInformationResponse message is used for successful HO report request and report.

42	The UE only stores the latest SHR entry.

43	The SHR scenario 3b, i.e. “Successful HO completion, but RLF in source during DAPS HO” is part of the SHR.

44	The SHR scenario 2c, i.e. “Successful CHO recovery while initial failure” is part of the RLF-Report.

In this paper, we would further discuss SON Enhancements for SHR.

2	Discussion

In previous RAN2 meetings, some triggering conditions for HO Success Report for normal handover/DAPS HO/CHO were agreed as below:

a.	The UE logs the HO success report if, while doing HO, T310 value exceeds a threshold

b.	The UE logs the HO success report if, while doing HO, T312 value exceeds a threshold

c.	The UE logs the HO success report if, while doing HO, T304 exceeds a threshold

d.	In case of DAPS, if the UE gets an RLF in the source while doing DAPS.

Besides T310 or T312 for radio link of the source cell, the UE may also monitor target link after the UE successfully handovers to the target cell but finally no RLF occurs in the target cell, e.g. the UE starts T310 or T312 for the target cell, or the UE detects the consecutive "out-of-sync" indications from target cell. Therefore, the conditions e.g. RLF does not happen in target cell but T310 in target cell is started within a period after successful DAPS handover, RLF does not happen in target cell but T312 in target cell is started within a period after successful DAPS handover, RLF does not happen in target cell but the number of consecutive "out-of-sync" indications from target cell is greater than one threshold, or the number of preamble attempt towards the target cell is greater than one threshold should also be considered.

Proposal 1: At least the following triggering conditions are applied for generating a HO Success Report for normal handover/DAPS HO/CHO:

- RLF does not happen in target cell but T310 in target cell is started within a period after successful handover;

- RLF does not happen in target cell but T312 in target cell is started within a period after successful handover;

- RLF does not happen in target cell but the number of consecutive "out-of-sync" indications from target cell is greater than one threshold;

- The number of preamble attempt towards the target cell is greater than one threshold.

Based on the triggering conditions, additional information should be reported in the HO successful report:

- T310 value in target cell when T310 stops.

- T312 value in target cell when T312 stops.

- The number of consecutive "out-of-sync" indications from target cell.

- The RACH related information e.g. the number of the transmitted preamble.

Proposal 2: At least the following information can be reported in the HO Success Report for normal handover/DAPS HO/CHO:

- T310 value in target cell when T310 stops.

- T312 value in target cell when T312 stops.

- The number of consecutive "out-of-sync" indications from target cell.

- The RACH related information e.g. the number of the transmitted preamble.

Considering that NR-U system is also involved for SON, there is the case that consistent LBT failures for at least one UL BWP are detected in the MAC layer of the source cell before successful handover, it shows that the unlicensed spectrum for the source cell are not available enough, even though final handover is successful, RLF may happen if consistent LBT failures occur in all the configured UL BWPs in the source cell. Therefore, consistent LBT failures in at least one UL BWP on the source cell can be considered as a triggering condition for generating a HO Success Report.

Proposal 3: For NR-U, consistent LBT failures in at least one UL BWP on the source cell can be considered as a triggering condition for generating a HO Success Report.

When the triggering condition that consistent LBT failures in at least one UL BWP on the source cell is satisfied, the UE would set the contents in the HO Success Report, e.g. the flag for this triggering condition, the identifier of the UL BWP where consistent LBT failure occurs, thus the source node may modify the LBT configurations for the source cell when it receives the successful HO related information from the target node.

Proposal 4: The identifier of the UL BWP where consistent LBT failure occurs can be reported in the HO Success Report.

In DAPS HO, after successfully accessing the target cell, UE stops the timer T304 and transmits the RRCReconfigurationComplete message to target cell. There are two states of source link when T304 stops upon successful random access, i.e. source link fails when T304 is running, or source link is available when T304 stops. For MRO purpose, the UE can report the state of source link in the HO Success Report for DAPS HO.

Proposal 5: The state of source link can be reported in the HO Success Report for DAPS HO.

For normal handover/DAPS HO/CHO, upon successful RACH towards the target cell, if the triggering condition for generating a HO Success Report is met, the UE may store successful handover related information in the HO Success Report. There is a case that a RLF occurs shortly after the successful RACH towards the target cell, thus the UE may set/store failure related information to generate the RLF Report. In this case, both HO Success Report and RLF Report are available at UE, since finally RLF happens in the target cell, RLF Report is more important for MRO than HO Success Report, the UE can release the stored HO Success Report and only send the RLF Report to the network.

Proposal 6: When a RLF Report is generated upon RLF occurring shortly after the successful handover, the UE can release the stored HO Success Report if any, and only report the RLF Report.

3	Conclusion

In this contribution, we further discuss the SON enhancements for SHR. We have the following proposals:

Proposal 1: At least the following triggering conditions are applied for generating a HO Success Report for normal handover/DAPS HO/CHO:

- RLF does not happen in target cell but T310 in target cell is started within a period after successful handover;

- RLF does not happen in target cell but T312 in target cell is started within a period after successful handover;

- RLF does not happen in target cell but the number of consecutive "out-of-sync" indications from target cell is greater than one threshold;

- The number of preamble attempt towards the target cell is greater than one threshold.

Proposal 2: At least the following information can be reported in the HO Success Report for normal handover/DAPS HO/CHO:

- T310 value in target cell when T310 stops.

- T312 value in target cell when T312 stops.

- The number of consecutive "out-of-sync" indications from target cell.

- The RACH related information e.g. the number of the transmitted preamble.

Proposal 3: For NR-U, consistent LBT failures in at least one UL BWP on the source cell can be considered as a triggering condition for generating a HO Success Report.

Proposal 4: The identifier of the UL BWP where consistent LBT failure occurs can be reported in the HO Success Report.

Proposal 5: The state of source link can be reported in the HO Success Report for DAPS HO.

Proposal 6: When a RLF Report is generated upon RLF occurring shortly after the successful handover, the UE can release the stored HO Success Report if any, and only report the RLF Report.

Reference

[1] Chairman notes for RAN2#113bis-e meeting

[2] Chairman notes for RAN2#114-e meeting