3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2108003

Online, August 09 – August 27, 2021

Source:	CATT

Title:	On support of Type A PUSCH repetitions for Msg3 ‎

Agenda Item:	8.19.2

Document for:	Discussion and Decision

Introduction

In RAN#92-e meeting, WID on NR coverage enhancements was agreed [1]. One of the objectives is to address the repetition of MSG3.

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1, RAN2]

In this document, we analyze the impacts brought by repetitions for MSG3 except for common aspects related to RACH indication (in MSG1) / RACH partitioning. Our proposals are summarized in section 3.

Discussion

  - RAR format

In RAN1 previoius meeting, it has been agreed that [2]

RAN#104bis-e meeting Agreement: For Msg3 PUSCH repetition, support the following modified Option 2-1. - Option 2-1: For UE requested triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - A UE can request trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.). - Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission. - FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual - FFS details if any.

In accordance to the agreement, it can be seen that at least preambles can be used to inidicate to the network that the UE requests MSG3 PUSCH repetiton. After sending preamble requesting MSG3 repetition, the UE will monitor the corresponding RAR during the RAR window. For indication of the number of repetitions for Msg3 initial transmission, there are two options provided in RAN1’s discussion:

- Option 1: Using an information field from the existing information fields in RAR UL grant;

- Option 2: Using MAC RAR for indication

In RAN1#105-e meeting, it was agreed that [3]

Working assumption: - Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission. - TDRA information field with introducing a new TDRA table including the repetition factors. - MCS information field - TPC information field - CSI request information field - FDRA information field

It can be seen that, option 1 has been covered by RAN1 according to the working assumption. Details are fully in RAN1’s scope. We only concentrate on option 2 in RAN2. Considering the current RAR format which is illustrated in Figure 1[4], there are two sub-options addressing the issue for option 2.

Introducing more bits in RAR

From our perspective, introducing more bits in RAR will bring great complexity to specification. For instance, new subhead is needed for the UE to distiguish the MSG3 repetition RAR from the legacy RAR, how to make sure that the new RARs do not impact the legacy UEs and ect..

Using the reserve bit

As shown in Figure 1, only one reserve bit can be used to indicate the repetition number. Apparently, the repetition number will be greatly limited and which can satisfy the flexibility of repetiton of MSG3.

Therefore, from RAN2’s perspective, to extend/reuse the existing RAR MAC CE seems not very straightforward. But on the other hand, to define new MAC CE for MAC RAR in the case of MSG3 repetition requires more discussions and specification effort in RAN2. Hence, considering the progress so far in RAN1 and also the above analysis, we propose that:

Proposal 1: No enhancements on MAC RAR are needed for MSG3 repetition.

Figure 1 MAC RAR

  - Start of ra-ContentionResolutionTimer

Based on current specification, MAC will start ra-ContentionResolutionTimer at each HARQ (re)transmission in the first symbol after the end of the Msg3 transmission. Under the case of MSG3 repetition, i.e. MSG3 is transmitted multiple times, how to start ra-ContentionResolutionTimer should be studied.

In RAN1, similar discussions were raised in [5] and two options were discussed [2], [6].

- Option 1: the MAC entity only starts the ra-ContentionResolutionTimer for the last transmission of the MSG3 repetitions.

- Option 2: the MAC entity starts and restarts the ra-ContentionResolutionTimer after each transmission of Msg3 PUSCH repetition.

In option 2, the UE starts to monitor PDCCH after the first transmission of MSG3 in the repetition, i.e. ra-ContentionResolutionTimer is running. And early termination of MSG3 repetition may be achieved in option 2 if the network could decode the MSG3 by the former MSG3 transmissions. Therefore, the UE could stop transmission of the subsequent MSG3 repetitions. It is beneficial to reduce some uplink resources and RA latency. This is especially helpful for TDD operation.

However, MSG3 PUSCH repetition is typically enabled when the coverage is pretty bad. It is most likely that gNB decodes the MSG3 after receiving all the MSG3 repetitions in order to decode MSG3 successfully. If the ra-ContentionResolutionTimer is (re)started after each transmission of MSG3 PUSCH, UE has to monitor PDCCH unnecessarily even if the gNB doesn’t decode individual MSG3 transmission correctly.

Therefore, we propose that:

Proposal 2: ra-ContentionResolutionTimer is started for the last transmission of the MSG3 repetitions.

  - Extension of RA window

In LTE, for NB-IoT UEs, BL UEs or UEs in enhanced coverage, it is expected to receive multiple repetition of physical control channel before it can decode the scheduling information, for example, RAR [7]. Therefore, the UE needs to monitor physical control channel with larger duration. And that is why the duration of RAR window and contention resolution timer is extended in LTE for NB-IoT UEs, BL UEs or UEs in enhanced coverage. However, for MSG3 repetition, there is no enhancements on PDCCH/PDSCH for MSG2/4 repetition based on the previous discussions in RAN1. Hence, we propose that:

Proposal 3: No extension is needed for ra-ResponseWindow and ra-ContentionResolutionTimer for MSG3 repetition.

Conclusion

In this document, we analyse potential impacts brought by MSG3 repetition. And the following proposals are provided:

Proposal 1: No enhancements on MAC RAR are needed for MSG3 repetition.

Proposal 2: ra-ContentionResolutionTimer is started for the last transmission of the MSG3 repetitions.

Proposal 3: No extension is needed for ra-ResponseWindow and ra-ContentionResolutionTimer for MSG3 repetition.

Reference

- RP-211566, Revised WID on NR coverage enhancements;

- RAN1#104bis meeting report;

- RAN1#105 meeting report;

- TS 38.321, NR; Medium Access Control (MAC) protocol specification;

- R1-2104101, Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3;

- R1-2102646, Discussion on Type A PUSCH repetitions for Msg3;

- TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC);