3GPP TSG-RAN WG2 Meeting #115 Electronic	R2-2108164

Online, 09 -27 August 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on CHO with SCG configuration

Agenda item:		6.1.4.1.1

Document for:		Discussion and Decision

- Introduction

At RAN2#113bis-e meeting, we discussed the support scenarios for CHO with SCG configuration and send a LS [1] to RAN3 to check the spec impact. And we received the reply LS [2] from RAN3 after RAN2#114-e meeting. In this contribution, we discussed whether to support CHO with SCG configuration in Rel-16.

- Discussion

- At RAN2#109bis meeting, RAN2 made an agreement that “We will not preclude SCG configuration in RRC Reconfiguration with conditional reconfiguration. Limit to cases without RAN3 impact.”. However, there is no clear conclude at RAN2/RAN3 which scenarios can be supported for CHO with SCG configuration, and nothing is captured in the current spec. So we discussed this issue again at RAN2#113bis-e meeting, and sent a LS to RAN3 to request them check the spec impact on RAN3 for supporting CHO with SCG configuration:

1. Overall Description: At RAN2#109-bis the following agreement on Conditional Handover (CHO) with SCG configuration was made: 2 We will not preclude SCG configuration in RRC Reconfiguration with conditional reconfiguration. Limit to cases without RAN3 impact.. As per the aforementioned agreement, the SCG configuration is applied only when the conditional reconfiguration for a PCell is met and CHO is executed. This agreement neither was captured in the specification, nor consulted with RAN3 so far. RAN2 has identified the following potential scenarios for conditional reconfiguration with SCG configuration: - CHO with same SN: CHO from source PCell 1 with SCG in SN 1 to target PCell 2 with SCG in the same SN 1. - CHO with different SNs: CHO from source PCell 1 with SCG in SN 1 to target PCell 2 with SCG in SN 2. - CHO from single-connectivity to an (MR-)DC connection: CHO from source PCell 1 to target PCell 2 with SCG in SN. - Scenarios 1, 2, 3, listed above, with target MCG and SCG in the same network node. This corresponds to the case where the UE is connected to two gNB-DUs in NR-DC, one serving the target MCG and the other serving the target SCG, connected to the same gNB-CU acting both as MN and SN. RAN2 would like to ask the following questions: Question 1: Is there any RAN3 specification impact if in any of the scenarios listed above the conditional reconfiguration (CHO) with SCG configuration is supported? Question 2: Are there any other mobility scenarios, not included above, where conditional reconfiguration with SCG configuration would be possible? 2. Actions: To RAN WG3 group. ACTION: RAN2 respectfully asks RAN3 to answer the two questions formulated within this LS.

- And a reply LS[2] from RAN3 was received after RAN2#114-e meeting:

1. Overall Description: RAN3 would like to thank RAN2 for the LS on Conditional Handover with SCG configuration scenarios! RAN3 has discussed the matter and concluded as follows: Ad. 1) The scenarios assumed in RAN2 can’t be supported with the existing Rel.16 signalling. RAN3 would like to ask for guidance, if the support is needed in Rel.16, or is to be enabled in Rel.17. Ad. 2) RAN3 has not identified any other scenarios beyond the list included in the RAN2’s LS. 2. Actions: To RAN2: ACTION: RAN3 kindly asks RAN2 to consider the issues and provide guidance if RAN3 shall introduce the needed support in Rel.16.

- Based on the reply LS, we see that all scenarios for CHO with SCG configuration listed by RAN2 have RAN3 impact. And RAN3 has not identified any other scenarios beyond the RAN2 list which have no impact on RAN3.

- Observation 1: Based on the reply LS from RAN3, all identified scenarios for CHO with SCG configuration can’t be supported with the existing Rel-16 signalling, i.e. all scenarios have impact on RAN3.

- According to the agreement made at RAN2#109-bis, the scenarios with RAN3 impact shall be precluded in Rel-16 CHO. Namely, all scenarios for CHO with SCG configuration can not be supported in Rel-16.

- And RAN3 also asked RAN2 to consider whether RAN3 need to introduce some enhancement to support this in Rel-16. However, considering the Rel-16 CHO has been frozen for a long time and we have a similar CPAC agenda in Rel-17 which can handle with the CHO with SCG case as well, it’s preferred to follow the previous agreement, i.e. all scenarios for CHO with SCG configuration with RAN3 impact are precluded in Rel-16. Thus CHO with SCG configuration is not supported in Rel-16. And RAN3 is not required to introduce the needed support in Rel-16 as well.

- Proposal 1: CHO with SCG configuration is not supported in Rel-16.

- Proposal 2: RAN2 sends a reply LS to inform that RAN3 is not required to introduce the needed support for CHO with SCG configuration in Rel-16.

- And the corresponding text proposals to clarify no support of CHO with SCG configuration are shown in the Annex.

- Proposal 3: To introduce the text proposals in Annex in the specifications.

- Conclusion and proposals

- In this contribution, we discussed the support of CHO with SCG configuration and gave the following observations and proposals:

- Observation 1: Based on the reply LS from RAN3, all identified scenarios for CHO with SCG configuration can’t be supported with the existing Rel-16 signalling, i.e. all scenarios have impact on RAN3.

- Proposal 1: CHO with SCG configuration is not supported in Rel-16.

- Proposal 2: RAN2 sends a reply LS to inform that RAN3 is not required to introduce the needed support for CHO with SCG configuration in Rel-16.

- Proposal 3: To introduce the text proposals in Annex in the specifications.

- Reference

- R2-2104348	LS on Conditional Handover with SCG configuration scenarios	LS out	NR_Mob_enh-Core	To:RAN3

- R2-2107527_R3-212848 Response LS on Conditional Handover with SCG configuration scenarios LS out	NR_Mob_enh-Core	 LS out  To:RAN2

- Annex

  - TS 38.331 v16.5.0

CHANGE START

#### –	CondReconfigToAddModList

The IE CondReconfigToAddModList concerns a list of conditional reconfigurations to add or modify, with for each entry the condReconfigId and the associated condExecutionCond and condRRCReconfig.

CondReconfigToAddModList information element

-- ASN1START

-- TAG-CONDRECONFIGTOADDMODLIST-START

CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Cond condReconfigAdd

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...

}

-- TAG-CONDRECONFIGTOADDMODLIST-STOP

-- ASN1STOP

CondReconfigToAddMod field descriptions

condExecutionCond The execution condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration. When configuring 2 triggering events (Meas Ids) for a candidate cell, network ensures that both refer to the same measObject.

condRRCReconfig The RRCReconfiguration message to be applied when the condition(s) are fulfilled. The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration, the field daps-Config or the SCG configuration.

| Conditional Presence | Explanation |
| --- | --- |
| condReconfigAdd | The field is mandatory present when a condReconfigId is being added. Otherwise the field is optional, need M. |

CHANGE END

  - TS 36.331 v16.5.0

CHANGE START

#### –	CondReconfigurationToAddModList

The IE CondReconfigurationToAddModList concerns a list of conditional reconfigurations (i.e. conditional handover) to add or modify, for each entry the measId (associated to the triggering condition configuration) and the associated RRCConnectionReconfiguration.

CondReconfigurationToAddModList information element

-- ASN1START

CondReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16

CondReconfigurationAddMod-r16 ::= SEQUENCE {

condReconfigurationId-r16			CondReconfigurationId-r16,

triggerCondition-r16				SEQUENCE (SIZE (1..2)) OF MeasId

OPTIONAL,  -- Cond CondReconfigurationAdd

condReconfigurationToApply-r16	OCTET STRING (CONTAINING RRCConnectionReconfiguration)

OPTIONAL,-- Cond CondReconfigurationAdd

...

}

-- ASN1STOP

CondReconfigurationToAddMod field descriptions

condReconfigurationToApply The RRCConnectionReconfiguration message to be applied when the condition(s) are fulfilled. The RRCConnectionReconfiguration message contained in condReconfigurationToApply can not contain the SCG configuration.

triggerCondition The condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration. When configuring two triggering events (MeasIds) for a candidate cell, the network ensures that both refer to the same measObject.

| Conditional presence | Explanation |
| --- | --- |
| CondReconfigurationAdd | The field is mandatory present if a condReconfigurationId is being added. Otherwise it is optional, need ON. |

CHANGE END

  - TS 37.340 v16.6.0

CHANGE START

# 10	Multi-Connectivity operation related aspects

## 10.1	General

Similar procedures as defined under clause 10.1.2.8 (Dual Connectivity operation) in TS 36.300 [2] apply for MR-DC.

Similar CHO principles as defined in TS 36.300 [2] and TS 38.300 [3] apply for the Conditional PSCell Change in MR-DC.

Conditional PSCell Change is not supported for the MR-DC options NE-DC and NGEN-DC.

Configuration of CHO and CPC for simultaneous operation is not supported in this release.

In case MR-DC is configured, CHO is only supported in Master Node to eNB/gNB Change procedure in this release.

CHO is not supported in eNB/gNB to Master Node Change procedure in this release.

NEXT CHANGE

Annex B (informative):
Supported MR-DC Handover Scenarios

Table B-1 summarizes the supported handover scenarios involving MR-DC configurations.

Table B-1: Supported MR-DC handover scenarios.

| To (column) HO from (row) | E-UTRA with EPC | E-UTRA with 5GC | NR | GERAN or UTRAN | EN-DC | NGEN-DC | NE-DC | NR-DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA with EPC | YES | YES | YES | YES | YES | NO | NO | NO |
| E-UTRA with 5GC | YES | YES | YES | NO | NO | YES | NO | NO |
| NR | YES | YES | YES | NOTE 1 | YES | NO | YES | YES |
| GERAN or UTRAN | YES | NO | NO | YES | NO | NO | NO | NO |
| EN-DC | YES | YES | YES | YES | YES | NO | NO | NO |
| NGEN-DC | YES | YES | YES | NO | NO | YES | NO | NO |
| NE-DC | YES | YES | YES | NOTE 1 | NO | NO | YES | NO |
| NR-DC | YES | YES | YES | NOTE 1 | NO | NO | NO | YES |

NOTE 1:	Only SRVCC handover of IMS voice bearer to UTRAN is supported.

NOTE 2:	All handover scenarios according to Table B-1 that have a DC option in the column "from" are supported during fast MCG failure recovery.

NOTE 3:	CHO is only supported from E-UTRA with EPC/EN-DC to E-UTRA with EPC and from NR/NE-DC/NR-DC to NR.

NOTE 4:	DAPS handover is only supported from E-UTRA with EPC to E-UTRA with EPC and from NR to NR.

CHANGE END

  - TS 36.300 v16.6.0

CHANGE START

#### 10.1.2.1a	Conditional Handover

##### 10.1.2.1a.1	General

A Conditional Handover (CHO) is defined as a handover that is executed by the UE when one or more handover execution conditions are met. The UE starts evaluating the execution condition(s) for CHO candidate cells upon receiving the CHO configuration, and executes CHO once the execution condition(s) are met for a CHO candidate cell. UE stops evaluating the execution condition(s) once a handover is executed.

The following principles apply to CHO:

-	The CHO configuration contains the configuration of CHO candidate cell(s) generated by each CHO candidate cell and execution condition(s) generated by the source cell.

-	An execution condition may consist of one or two trigger condition(s) (CHO events A3/A5). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evaluation of CHO execution condition of a single candidate cell.

-	UE maintains connection with source eNB until UE determines a CHO execution condition is met for CHO candidate cell.

-	Before any CHO execution condition is satisfied, upon reception of HO command (without CHO configuration), the UE executes the HO procedure as described in clause 10.1.2.1, regardless of any previously received CHO configuration.

-	After source eNB sends CHO command to UE, the network is allowed to change source eNB configuration and network can add, modify or release a configured CHO configuration using RRC message (i.e. until UE starts executing CHO.

-	While executing CHO, i.e. from the time when the UE starts synchronization with target cell, UE does not monitor source cell.

NOTE 1:	CHO is not supported for S1 based handover in this release of the specification.

NOTE 2:	In case LTE-DC is configured, CHO is only supported in MeNB to eNB change procedure in this release of the specification.

NOTE 3:	CHO is not supported in eNB to MeNB change procedure in this release of the specification.

CHANGE END