3GPP TSG-RAN WG2 Meeting #115-e	R2-2108190

# Electronic Meeting, 9 – 27 August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2758 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ANS.1 misalignment for the SCGFailureInformationNR message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core, NR_unlic-Core, NR_IAB-Core, NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current TS 36.331 v16.5.0, in case the UE is in EN-DC or NGEN-DC and the UE experiences an NR SCG RLF, it sends the SCGFailureInformationNR message to the E-UTRA MCG. This message includes different failure causes that are referring to possible RLF use cases that has been identified during Rel-15 and Rel-16. The Rel-16 version of SCGFailureInformationNR message includes the following: FailureReportSCG-NR-r15 ::= SEQUENCE { failureType-r15 ENUMERATED { t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-reconfigFailure, srb3-IntegrityFailure, other-r16}, measResultFreqListNR-r15 MeasResultFreqListFailNR-r15 OPTIONAL, measResultSCG-r15 OCTET STRING OPTIONAL, ..., [[ locationInfo-r16 LocationInfo-r10 OPTIONAL, logMeasResultListBT-r16 LogMeasResultListBT-r15 OPTIONAL, logMeasResultListWLAN-r16 LogMeasResultListWLAN-r15 OPTIONAL, failureType-v1610 ENUMERATED {t312-Expiry, scg-lbtFailure, beamFailureRecoveryFailure, bh-RLF-r16, spare4, spare3, spare2, spare1} OPTIONAL ]] } The main problem with the Rel-16 version of the SCGFailureInformationNR message is that the field failureType-r15 has a difference number of codepoints with respect to the Rel-15 version of the same field. The Rel-15 version of SCGFailureInformationNR message includes the following: FailureReportSCG-NR-r15 ::= SEQUENCE { failureType-r15 ENUMERATED { t310-Expiry, randomAccessProblem, rlc-MaxNumRetx, synchReconfigFailureSCG, scg-reconfigFailure, srb3-IntegrityFailure}, measResultFreqListNR-r15 MeasResultFreqListFailNR-r15 OPTIONAL, measResultSCG-r15 OCTET STRING OPTIONAL, ... } As shown above, the field failureType-r15 has 7 entries in the Rel-16 version of TS 36.331, whereas in the Rel-15 version of TS 36.331 it has only 6. The reason for this is because in Rel-16 a new failure cause was added (i.e., other-r16) and this was done in a non-backward compatible way with respect the existing Rel-15. The result of this is that a Rel-15 eNB does not have any knowledge on the possibility for the UE to signal the field failureType-r15 with the value “other-r16”. In fact, in the Rel-15 version of the failureType-r15 field there are no spare values that are possible to use in later releases. Due to this ASN.1 misalignment, the consequence is that when a Rel-16 UE includes the field failureType-r15 with the value “other-r16” and sends it to a Rel-15 eNB, this Rel-15 eNB may consider the all SCGFailureInformationNR message as not valid and will trigger a ASN.1 syntax error. While this may be solved by implementation in intra-vendor deployment, for the case of inter-vendor deployment this may be more critical and may degrade the performance of the all MR-DC operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.7.3.3 - In the procedural text is clarified that the UE shall set the field failureTypeOther-r16 to other in when including a failure cause belonging to Rel-16. Impact Analysis Impacted 5G architecture options: (NG)EN-DC, NE-DC,NR-DC Impacted functionality: SCG failure information Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the network may consider the SCGFailureInformation message as invalid and trigger a RRC re-establishment procedure instead of performing a RRC connection recovery. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. This CR is considered mandatory to support the impacted functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the CR is not agreed, the network may consider the SCGFailureInformation message as invalid and trigger a RRC re-establishment procedure instead of performing a RRC connection recovery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR 4709 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.7.3.3	Failure type determination for (NG)EN-DC

The UE shall set the SCG failure type as follows:

1>	if the UE initiates transmission of the SCGFailureInformationNR message due to T310 expiry:

2>	set the failureType as t310-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to T312 expiry:

2>	set the failureTypeOther as other and set the failureType-v1610 as t312-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as synchReconfigFailureSCG;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide random access problem indication from SCG MAC:

2>	if the random access procedure was initiated for beam failure recovery:

3>	set the failureTypeOther as other and set the failureType-v1610 as beamFailureRecoveryFailure;

2>	else:

3>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to SRB3 integrity check failure:

2>	set the failureType as srb3-IntegrityFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to Reconfiguration failure of NR RRC reconfiguration message:

2>	set the failureType as scg-reconfigFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationNR message due to consistent uplink LBT failures:

2>	set the failureTypeOther as other and set the failureType-v1610 as scg-lbtFailure;

1> else if connected as an IAB-node and the SCGFailureInformationNR is initiated due to the reception of a BH RLF indication on BAP entity from the SCG:

2>	set the failureTypeOther as other and set failureType-v1610 as bh-RLF.

END OF CHANGES