3GPP TSG-RAN2 Meeting # 115-e	R2-21XXXXX

# Online, 16 - 27 Aug, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.300 running CR for Introduction of QoE measurements in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Unicom, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Running CR for introduction of QoE measurements in NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR captures the agreements made since RAN2#113b-e meeting. (detailed agreements are listed at the end of the CR) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | QoE measurements will not be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CRxxxx TS 38.306 CRxxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

MDBV	Maximum Data Burst Volume

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MT	Mobile Termination

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

TRP	Transmit/Receive Point

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

<Next modification>

# X	Application Layer Measurement Collection

## X.1	Overview

This function enables collection of application layer measurements from the UE. The supported service types are:

-	QoE Measurement Collection for streaming services;

-	QoE Measurement Collection for MTSI services.

## X.2	Configuration aspects

### X.2.1	General

The application layer measurement configuration and measurement reporting are supported in RRC_CONNECTED state only. Application layer measurement configuration received by the gNB from OAM or CN is encapsulated in a transparent container, which is forwarded to a UE in a downlink RRC message (there can be multiple configurations in the same message). Application layer measurements received from UE's higher layer are encapsulated in a transparent container and sent to the network in an uplink RRC message, as specified in TS 38.331 [12]. gNB can release multiple application layer measurement configurations from the UE in one RRC message at any time. At reception of QoE release command, the UE discards any unsent QoE reports corresponding to the released application layer configuration.

### X.2.2	Pause and resume

Application layer measurements can be paused by the gNB using a downlink RRC message that is used to temporarily stop application layer measurement reports from being sent from the UE to the network.

### X.2.3	Measurement handling in RRC_IDLE and RRC_INACTIVE

If the UE enters RRC_INACTIVE, the UE AS configuration for the QoE is stored in the UE Inactive AS context.

If the UE enters RRC_IDLE state, the UE releases all the QoE measurement configurations.

# Annex	RAN2 agreements on NR QoE

RAN2 agreements for information, which wlll be removed from final CR.

## A.1 Configuration architecture general aspect

RAN2#114-e Agreements

gNB can release a list of QoE measurement configurations in one RRCReconfiguration message.

If a QoE measurement configuration is released, RRC layer informs the upper layer to release the QoE measurement configuration. This could be revisited based on other issues’ progress.

If the UE enters IDLE state, UE should release all of the QoE measurement configurations.

QoE configuration and report are encapsulated in a transparent container in the RRC messages. It is FFS for RAN-visible QoE configuration and report (dep on R3).

At lease service type and RRC level ID (Reference ID or shorten ID) together with corresponding QMC configuration container should be included for each QoE configuration in RRCReconfiguration message when the network setups QoE measurement to the UE.

At least RRC level ID (Reference ID or shorten ID) together with corresponding QMC report container should be included in MeasReportAppLayer message for each QoE report.

RAN2 confirms logged MDT framework for QoE data retrieval and reporting is not supported in Rel-17.

RAN2 assumes that QoE configuration modification does not need to be supported from RAN2 signalling point of view (in RRC), and send LS to SA5/SA4 to confirm the assumption.

Send LS to SA4/SA5/RAN3 ask whether multiple QoE measurement configurations can be configured for a certain service type.

RAN2 assumes to re-use the maximum container size of 1000 bytes for QoE measurements configuration and send LS to SA4 to confirm the assumption.

Send LS to SA4 to check the necessity of the maximum container size of QoE measurements report beyond than 8000 bytes.

RAN2#113b-e Agreements

Configure QoE measurements for NR in RRCReconfiguration.

Add configuration of QoE measurements in OtherConfig in RRCReconfiguration.

Add the configuration of QoE measurements by means of list to enable configuration of multiple simultaneous measurements.

R2 assumes that for RRC an ID is required to identify a measurement, FFS whether this is the QoE reference ID or something else.

Define SRB4 for transmission of QoE reports in NR.

Define an RRC message MeasReportAppLayer for the transmission of QoE reports in NR.

RAN2 assumes that QoE support for NR includes (as the LTE framework): activation by Trace Function, both signalling and management-based configuration and RRC procedures supporting AppLayer config and report.

From RAN2 point of view, the UE shall follow gNB commands and, NG-RAN can in principle release by RRC the application layer measurement configuration towards the UE at any time, e.g. if required due to load or other reasons (Note that other WGs are responsible to define the normal system procedures for release and which nodes are responsible etc).

The UE Inactive AS context includes the UE AS configuration for the QoE (it is not released when UE goes to Inactive).

## A.2 start and stop

RAN2#114-e Agreements

At reception of QoE release, the UE shall discard any unsent QoE reports corresponding to the released QoE configuration.

FFS whether pause resume will affect all configurations or whether pause resume can act selectively per configuration.

On whether to store reports in the AS or the application layer at Pause, Send LS to SA4/SA5/SA3 to inform them about the options and their pros/cons (if possible) and ask them for feedback. RAN2 will continue work on this topic based on the feedback received.

RAN2#113b-e Agreements

“QoE pause” indication from the network is used to temporarily stop QoE reports from being sent from the UE to the network. Application layer behaviour upon UE receiving “pause/resume” indications is out of RAN2 scope.

The following are options considered by RAN2 for QoE report handling during RAN overload via “QoE report pause indication”:

Option 1: Application layer is responsible for storing QoE reports when the UE receives QoE pause indication.

Option 2: AS layer is responsible for storing QoE reports when the UE receives QoE pause indication.

Option 3: The QoE container received from application layer is discarded during pause.