3GPP TSG-RAN WG2 Meeting #115 Electronic 											R2-2108259

Elbonia, 16 – 27 August 2021

Agenda item:	8.12.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On RRM relaxations for REDCAP

WID/SID:	NR_redcap-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

RAN2 objectives for reduced capability (REDCAP) devices were revised in RP-211574:

In this contribution we discuss RRM relaxations as per the following revised work item objectives:

Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation. - Specify UE requirements for RRM measurement relaxation [RAN4] - No RRM measurement relaxations are specified for the serving cell.

The following was agreed for RRM relaxations in RAN2#113ebis meeting:

Agreements:

- Assuming there will be a stationary property based on subscription (which is FFS), we will not restrict to this and will continue to assume that a UE can use some RSRP/RSRQ based criteria (FFS whether reuse R16 thresholds or new ones. FFS also on the use of a beam based criteria)

Agreements:

1.	At least for RRC idle/inactive, a measurement-based R17 stationarity criterion can be configured separately from R16 low-mobility criterion for R17 UEs supporting the feature. FFS how the configuration is provided. FFS whether this stationarity criterion is based on:

-	the same algorithm used in R16 low-mobility criterion but with its own specific set of thresholds; and/or

-	a combination of R16 low-mobility criterion and/or beam-change based criterion. Exact details of beam change criterion are FFS.

Agreements - via email (from offline [102]):

- Network can configure R17 stationarity criterion/criteria together with a not-at-cell-edge criterion, to trigger RRM relaxations in RRC Idle/Inactive for R17 UEs supporting the feature. FFS whether the R16 not-at-cell-edge thresholds can be reused or separate R17 not-at-cell-edge thresholds are needed.

The following was agreed for RRM relaxations in RAN2#114 meeting:

Agreements:

- An RSRP/RSRQ based stationarity criterion (Working Assumption: the same as in idle/inactive) can be configured for UEs in RRC Connected. If the criterion is met, this is reported to the network (FFS how/when). It is FFS whether, based on this, besides possibly reconfiguring RRM measurements (up to network implementation), the network can enable RRM measurement relaxation (FFS whether same method as in Idle/Inactive)

Agreements:

- Subscription based relaxation criteria will not be considered in Rel-17 RRM relaxation

Agreements via email (from offline 111):

- Reuse R16 low mobility criterion, as part or whole of Rel-17 stationary criterion in RRC_IDLE/INACTIVE. When NW configures both Rel-17 stationary criterion and Rel-16 low mobility criterion, NW configures different Rel-17 thresholds (i.e., SSearchDeltaP_stationary/TSearchDeltaP_stationary) from Rel-16 (SSearchDeltaP / TSearchDeltaP). How to configure the criterion (e.g. more stringent) is left to NW implementation (i.e. no specification impact to RAN2).

- Postpone the following discussion until RAN4 defines RRM relaxation method for Rel-17:

When NW configures both R16/R17 relaxation criteria and the UE fulfills both, UE performs:

- Option 1) UE performs Rel-17 RRM relaxation method

- Option 2) It is up to UE implementation to select either Rel-16 or Rel-17 relaxation operation

Agreements online:

- Working Assumption: If beam-level criterion is adopted for Rel-17 stationary criterion in RRC_IDLE/INACTIVE, it is configured separately with Rel-16 low mobility criterion reused

- When NW configures Rel-17 RRM relaxation for RRC_IDLE/INACTIVE, Rel-17 stationary criterion is mandatory, and Rel-17 not-at-cell-edge criterion is optional configuration. FFS whether the same applies to RRC Connected

- Continue discussion on Rel-17 not-at-cell-edge criterion in RRC_IDLE/INACTIVE within two options:

- Option 1) Reuse Rel-16 not-at-cell-edge criterion with the same thresholds (i.e., SSearchThresholdP / SSearchThresholdQ)

- Option 2) Reuse Rel-16 not-at-cell-edge criterion with the different thresholds

# 2	Discussion

In REL16 RRM measurement relaxations were specified for IDLE/INACTIVE UEs. In REL-16 the following two criterions for allowing the UE to relax measurements in IDLE/INACTIVE were defined in TS 38.304 i.e. “low mobility” and “not at cell edge”.

##### 5.2.4.9.1	Relaxed measurement criterion for UE with low mobility

The relaxed measurement criterion for UE with low mobility is fulfilled when:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

##### 5.2.4.9.2	Relaxed measurement criterion for UE not at cell edge

The relaxed measurement criterion for UE not at cell edge is fulfilled when:

-	Srxlev > SSearchThresholdP, and,

-	Squal > SSearchThresholdQ, if SSearchThresholdQ is configured,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

RAN2 work item objective for RRM relaxations define that RRM relaxations shall be specified for all the RRC states:

Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation. - Specify UE requirements for RRM measurement relaxation [RAN4] No RRM measurement relaxations are specified for the serving cell

In REL16 no RRM relaxations were specified for CONNECTED. Considering the lack of RRM measurement relaxation in CONNECTED and work item objectives it seems straight forward to propose the following:

Proposal 1: RRM relaxation is supported for the non-serving cells only

Proposal 2: Measurement based (RSRP/RSRQ) not-at-cell-edge criterion is defined for RRC IDLE/INACTIVE CONNECTED

Proposal 3: Measurement based (RSRP) stationarity criterion is defined for RRC IDLE/INACTIVE CONNECTED

Proposal 4: Stationarity criterion in IDLE/INACTIVE reuses the Rel-16 low mobile criterion for IDLE/INACTIVE

The following was agreed in RAN2#113ebis meeting:

Agreements:

1.	At least for RRC idle/inactive, a measurement-based R17 stationarity criterion can be configured separately from R16 low-mobility criterion for R17 UEs supporting the feature. FFS how the configuration is provided. FFS whether this stationarity criterion is based on:

-	the same algorithm used in R16 low-mobility criterion but with its own specific set of thresholds; and/or

-	a combination of R16 low-mobility criterion and/or beam-change based criterion. Exact details of beam change criterion are FFS.

The following WA was agreed in RAN2#114 meeting:

Agreements online:

- Working Assumption: If beam-level criterion is adopted for Rel-17 stationary criterion in RRC_IDLE/INACTIVE, it is configured separately with Rel-16 low mobility criterion reused

In addition, to determine when UE can relax (or adapt) the RRM measurements, it is important to define an exit condition or criteria when UE has to cancel the adaptation. In NR the cell quality is derived based on the beam level measurements at RRC and single value is derived using multiple beams (depending on the NW configuration). Thus, in addition to cell quality, UE could determine the RRM relaxation enter/exit conditions based on individual beam measurements that were used for deriving the cell quality. For example, when UE obtains the reference value (SrxlevRef) it uses the beam (or beams) index/indices used for deriving the cell quality as reference beams for evaluating beam change condition. Using the beam measurements that are used for cell quality derivation, would not require additional measurements at UE.

Proposal 5: Define beam change based criterion that considers beams used for cell quality derivation for allowing/disallowing RRM measurement relaxation in IDLE/INACTIVE/CONNECTED.

Work item defines also objective to have support for enabling/disabling of RRM relaxation by both broadcast and dedicated signalling.

Specify support for the following RRM measurement relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected [RAN2, RAN4]: - Specify measurement (RSRP/RSRQ) based stationarity criterion and not-at-cell-edge criterion [RAN2] - Enabling/disabling of RRM measurement relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM measurement relaxation.

Broadcast configuration should be supported for RRM relaxation according to work item objective. Also network dedicated control for enabling RRM relaxations should be supported for all the RRC states as clearly indicated in the work item objectives. When releasing UE’s RRC connection, RAN can then enable RRM relaxation for the UE in the RRC Release message. The UE can apply the enabled RRM relaxation method once it is in RRC Idle/Inactive.

Proposal 6: RRM relaxation parameters are provided in system information

Proposal 7: Network can enable/disable RRM relaxation for IDLE/INACTIVE/CONNECTED UE with dedicated signaling

# 3	Conclusion

In this contribution RRM relaxations for REDCAP UE was discussed, and the following is proposed:

Proposal 1: RRM relaxation is supported for the non-serving cells only

Proposal 2: Measurement based (RSRP/RSRQ) not-at-cell-edge criterion is defined for RRC IDLE/INACTIVE CONNECTED

Proposal 3: Measurement based (RSRP) stationarity criterion is defined for RRC IDLE/INACTIVE CONNECTED

Proposal 4: Stationarity criterion in IDLE/INACTIVE reuses the Rel-16 low mobile criterion for IDLE/INACTIVE

Proposal 5: Define beam change based criterion that considers beams used for cell quality derivation for allowing/disallowing RRM measurement relaxation in IDLE/INACTIVE/CONNECTED.

Proposal 6: RRM relaxation parameters are provided in system information

Proposal 7: Network can enable/disable RRM relaxation for IDLE/INACTIVE/CONNECTED UE with dedicated signaling