3GPP TSG-RAN WG2 Meeting #115-e	R2-2108412

Online, August, 2021

Agenda item:       8.10.3.2 Idle/Inactive mode

Source: 	Convida Wireless

Title: 	NTN type and scenario indication

Document for:	Discussion and Decision

# 1  Introduction

For Rel-17, a work item for NR to support NTN (Non-Terrestrial Networks) [1] has been approved with some initial progress in RAN2. One of the primary RAN2 objectives for such enhancements is to address idle mode procedures as highlighted below:

The following control plane procedures enhancements should be specified (see TR 38.821) [2]

- Idle mode:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

  - Definition of NTN (satellite/HAPS) cell specific information in SIB

An essential aspect to the CP procedure enhancements is to identify the network type (i.e., Terrestrial Network or Non-Terrestrial Network) and RAN2 has made some initial agreements related to the identification or indication of an NTN network type. This is shown below with the following was incorporated into the endorsed running CR for 38.300 [3] from 16.x.4.1.1 Mobility in RRC_IDLE from RAN2#114e:

The UE is made aware that a cell is provided by NTN.

Editor’s note: FFS on whether to make the UE aware of NTN in an implicit or explicit way.

Furthermore, based on the previous email discussion outcomes, no consensus could be made as far as the need to introduce an implicit or explicit network type (e.g. NTN vs TN) indication or an implicit or explicit network scenario (e.g. earth moving vs earth fixed, GEO/ LEO /HAPS) for neighbor cells to assist cell reselection.

Based on the lack of further agreements on NTN indication, the objective of this paper is to progress this further. This discussion paper is a resubmission of R2-2106387.

# 2	 Discussion

#### 2.1 Rationale for Network Identification

In this section, we discuss the open issue regarding the network type to be known to the UE. For mixed network deployments that support both TN and NTN platforms (types), there may be UEs that are not NTN capable. These UEs should only search and select TN cells. Likewise, UEs supporting only NTNs should also search and select NTN cells. The scenario should be avoided where a UEs attempts access on a particular network/platform (type) that it is not capable of supporting. Differentiating between networks and network types from a UE perspective will be essential given NTNs/TNs may be deployed in the same bands and PLMN IDs may also not differentiate the network/platform type or scenario.

Additionally, UEs supporting both TNs and NTNs will be able to select between the two network types and potentially more than one NTN type. It is necessary to identify and select the appropriate platform/type and NTN scenario.

Based on these potential deployment scenarios, UE capabilities and preferences for a given network platform type, the awareness of a network type (NTN versus TN) will be helpful for the UE to find a suitable cell for cell search and cell (re)selection procedures. Latency reduction associated with scanning RF channels may also be beneficial given already long propagation delays. From the identification of the platform type and NTN scenario, prioritization and cell (re)selection can be aided. The UE may use preferences, such as UE capabilities, service requirements, and Quality of Service (e.g., latency requirements).

Observation 1: Consistent with the endorsed running CR for 38.300, the network type (i.e., TN or NTN) shall be known to UE.

Observation 2: The NTN scenario (e.g., LEO, MEO, earth moving/fixed) shall be known to UE.

#### 2.2 Potential solutions

#### 2.2.1 Explicit versus implicit indication

During the post RAN2#113bis-e [101] email discussion [4], the focus was on network/scenario type indication for neighbor cells for cell reselection in NTN. Unfortunately, there was no conclusion on whether there is need to indicate network type and/or scenarios explicitly for neighbor cells to assist cell reselection.

In summary, there are two general methodologies that have been proposed in RAN2 for NTN platform type indication as follows (or combination thereof):

- NTN platform indication may be signaled to the UE in an explicit way by the gNB to assist the UE in making decisions for cell (re)selection. For simplicity, this does not introduce any ambiguity from the UE perspective, but requires a definition of new indicators(s).

- NTN platform type indication may be implied from other, already available information that is already signaled from the gNB to the UE. This does not introduce any additional overhead as far as signaling a separate indication and does not require definition of a separate indicator.

One example of explicit indication of platform type could be a flag set in the existing MIB that is broadcasted by gNBs to UEs. However, there is only one spare bit available in the current Rel-16 MIB defined today, but this could be used to distinguish between TN versus NTN. This indicator could be used along with some additional system information to determine a particular type of NTN.

Observation 3: Explicit platform type indicator/indication may not be necessary as other already available information may be signalled from the gNB to the UE.

Observation 4: Implicit platform type indicator/indication does not require any additional overhead nor require additional definition of a separate indicator.

Based on the summary from [4], only 7 of 22 responding companies supported the introduction of an explicit network type (e.g., NTN vs TN) indication for neighbour cells to assist cell reselection. Only 4 companies support introduction of an explicit network scenario (e.g., earth moving vs earth fixed, GEO/ LEO /HAPS) indication for neighbor cells to assist cell reselection.

Observation 5: A majority of RAN2 companies do not support an explicit indication of network type (e.g. NTN vs TN) or network scenario.

Proposal 1: Based on the majority RAN2 view, RAN2 agrees to leverage an implicit platform type indicator/indication to the UE. The details of an implicit network type and scenario indication should be concluded in an email discussion at RAN2#115e.

#### Implied NTN Indicator(s) Options

#### 2.2.2.1 Frequency Layer Identification

One type of implied indication could be frequency layer identification to determine the platform type. In this case, the UE could be pre-configured to associate a frequency band with a particular platform (TN/NTN, or NTN scenario). However, it should be noted that TNs and NTNs may be deployed in the same bands.

Observation 6: Association of a frequency band with a particular platform (TN/NTN, or NTN scenario) may help identify a network type. However, TNs and NTNs may be deployed in the same bands, which likely requires some other indication.

#### 2.2.2.2 Timing Relationship Offset Identification

RAN1 has agreed the following regarding a timing relationship offset for initial access [5]:

•	For K_offset configured in system information and used in initial access, at least a cell specific K_offset configuration, which is used in all beams of a cell, should be supported.

With the assumption that a timing relationship offset is necessary and to be delivered to the UE from the network for initial access, the existence of this offset can be inferred by the UE to determine not only that the platform type of the cell is NTN, but also that the platform is of a specific NTN scenario.

This is possible given the relationship between the amount of timing relationship offset specific to a given NTN platform deployment. That is, low-orbiting SVs will require a smaller timing offset versus a high altitude-orbiting SV. Based on the offset (or offset range), an NTN-capable UE can determine not only the NTN platform type, but also distinguish between the various NTN scenarios. Alternatively, if there is no value or missing value for the timing relationship offset, the UE can assume that the cell is TN platform.

Furthermore, the determination of platform type as well as specific NTN scenario could be further leveraged for platform prioritization and cell (re)selection.

Observation 7: The existence of a timing relationship offset can be inferred by the UE to determine not only that the platform type of the cell is NTN, but also that the platform is of a specific NTN scenario.

#### 2.2.2.3 Presence of NTN-specific SI for Indication

There are two options or combination of options for this type of solution:

- NTN cells are determined by System Information Blocks (SIBs) either in an NTN-specific SIB or an existing SIB modified to support NTN system information. In this case, SIB1 would be broadcasted by the gNB and indicate that there exists NTN SIB(s) to be scheduled.

- Furthermore SI (if Satellite ephemeris is included) would also determine the scenario of NTN, e.g. GEO, LEO, HAPS, etc. For example, a Non-geostationary orbiting satellite would likely have additional parameters for trajectory, velocity, etc. or formatted/encoded differently based on platform scenario.

In the simplest form, the mere existence of NTN System Information can enable the UE to determine the platform type supported by the cell is NTN, e.g., IE si-SchedulingInfo might include sibType(s) associated with NTN.

Since the UE may have to decode and process additional system information to determine the type of NTN cell, a spare bit in the MIB may be added to indicate NTN platform (flag not present assumes TN platform). This may be utilized in combination with some other control information (e.g., implicit indication in the MIB) or indication in SIB1 to determine the NTN scenario in case the indicator in MIB indicates NTN platform type. If the UE does not support NTN, the UE may ignore the cells with the additional indication for NTN type/scenario (e.g., LEO, MEO, GEO, etc.). If the UE does not support TN, the UE may ignore any TN cells that are broadcasting without the NTN indicator.

Observation 8: The existence of NTN-specific SI can be inferred by the UE to determine not only that the platform of the cell is NTN type, but also that the platform is of a specific NTN scenario. Conversely, exclusion of NTN-specific SI can determine TN. This implicit indication could be combined with some MIB specific encoding/indication as well.

Proposal 2: RAN2 agrees to leverage NTN-specific System Information to implicitly determine the platform type (NTN vs. TN) and NTN scenario (LEO/MEO (earth-moving cells), GEO, or HAPS). Specific SI for platform indication is FFS.

# 3  Conclusion

To summarize, the following observations and proposals were examined:

Observation 1: Consistent with the endorsed running CR for 38.300, the network type (i.e., TN or NTN) shall be known to UE.

Observation 2: The NTN scenario (e.g., LEO, MEO, earth moving/fixed) shall be known to UE.

Observation 3: Explicit platform type indicator/indication may not be necessary as other already available information may be signalled from the gNB to the UE.

Observation 4: Implicit platform type indicator/indication does not require any additional overhead nor require additional definition of a separate indicator.

Observation 5: A majority of RAN2 companies do not support an explicit indication of network type (e.g. NTN vs TN) or network scenario.

Observation 6: Association of a frequency band with a particular platform (TN/NTN, or NTN type) may help identify a network type. However, TNs and NTNs may be deployed in the same bands, which likely requires some indication.

Observation 7: The existence of a timing relationship offset can be inferred by the UE to determine not only that the platform type of the cell is NTN, but also that the platform is of a specific NTN scenario.

Observation 8: The existence of NTN-specific SI can be inferred by the UE to determine not only that the platform type of the cell is NTN, but also that the platform is of a specific NTN scenario. Conversely, exclusion of NTN-specific SI can determine TN. This implicit indication could be combined with some MIB specific encoding/indication as well.

Proposal 1: Based on the majority RAN2 view, RAN2 agrees to leverage an implicit platform indicator/indication to the UE. The details of an implicit network type and scenario indication should be concluded in an email discussion at RAN2#115e.

Proposal 2: RAN2 agrees to leverage NTN-specific System Information to implicitly determine the platform type (NTN vs. TN) and NTN scenario (LEO/MEO (earth-moving cells), GEO, or HAPS). Specific SI for platform indication should be further discussed by RAN2.

# 4  References

[1] 3GPP RP-211557, "Revised WID: Solutions for NR to support non-terrestrial networks (NTN)," June 2021.

[2] 3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), (Release 16).

[3] R2-2106539 NTN Stage2 running CR 38.300, THALES

[4] R2-214085 Report from [Post113bis-e] [101] [NTN] cell reselection (ZTE)

[5] 3GPP MCC, “Draft Report of 3GPP TSG RAN WG1 #103-e v0.2.0”