3GPP TSG-RAN WG2 #115-e	Tdoc R2-2108421

Online, August 16th – 27th 2021

Agenda Item:	8.4.2

Source:	Ericsson

Title:	On Topology-wide Fairness, Multi-hop Latency, and Congestion in IAB Network

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#113e, the following topics were endorsed for further discussion and possible enhancements to topology-wide fairness, multi-hop latency, and congestion in IAB network.

ISSUES: eIAB work on topology-wide fairness will focus on the following issues - IF-1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality (such as e.g. using downstream link quality measurements to adjust the scheduling weights so as to achieve proportional fairness for different bearers/RLC channels across multiple child-IAB nodes) - IF-2: Congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested, thereby leading to some users experiencing longer latency and violating fairness requirement. - IF-4: IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers and/or carry bearers with higher load per bearer (i.e. IAB node cannot give more resource to those BH RLC CHs with higher aggregate load) ISSUES: In the first instance, eIAB work on multi-hop latency will focus on the following issues: - IL-1: IAB node cannot help ensure that overall or remaining PDB is met for a packet (e.g. by prioritizing bearers with higher number of hops), as it does not have a latency reference for the packets being scheduled, resulting in packets with the same QoS requirement ending up with different latency - IL-2: IAB node may need to report joint buffer status for LCHs which have rather differing QoS requirements, due to the current (Rel-16) limit on the number of LCGs - IL-3: Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors as it is left to implementation in Rel-16 - IL-5: The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free IL-6: The CU is unable to configure routing based on actual (real-time) latency per BH RLC channel - IC-1: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets - IC-7: CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion. Both IC-1 and CI-7 are related to RAN3. RAN3 seems to also work on this, so to what extent R2 shall work on this is currently not clear.

Furthermore, during RAN2#113bis-e, several proposals related to the above topics were discussed but due to lack of consensus among companies, only the following one was agreed:

LCG range to be extended for IAB-MT. Size of LCG and enhancements to BSR are FFS

This paper analyses the above issues and presents our point of views as well as possible solutions.

# 2	Discussion

This section provides our perspective on each topic agreed for further discussion in RAN2, which are documented in [1].

## 2.1	Topics Related to Topology-wide fairness

- IF-1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality (such as e.g. using downstream link quality measurements to adjust the scheduling weights so as to achieve proportional fairness for different bearers/RLC channels across multiple child-IAB nodes).

For this topic, few companies argued for defining new HbH assistance signaling that includes radio link conditions from descendent nodes. In our view, this does not only requires considerable specification work but also will inflict significant signalling overhead during network operation. Besides, it is not clear how this HbH assistance signaling will improve topology-wide fairness, and most importantly, why the scheduler of an IAB node should take into account the quality of other links. The schedulers of the various IAB nodes should rather act as independent entities, each of them trying to exploit at the best the radio link(s) they are serving, otherwise, this may result in underutilization or unbalanced utilization of the radio link capacity. Additionally, by the time this radio link quality indication is received by an IAB node of a previous hop, such radio link quality may have already changed which will make the usage of such information even more troublesome.

Considering all these points, in our view, RAN2 should deprioritize this topic (IF-1) for further discussion and rather focus on other important topics related to topology-wide-fairness.

It is not clear what benefits the sharing of the link qualities over multiple hops brings, especially considering the significant specification work, and the large overhead.

The schedulers of the various IAB nodes should act as independent entities, each of them trying to exploit at the best the radio link(s) they are serving, otherwise, this may result in an unbalanced utilization of the radio link capacity.

RAN2 to downprioritize any proposed solution related to issue IF-1.

- IF-2: Congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested, thereby leading to some users experiencing longer latency and violating fairness requirement.

- IF-4: IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers and/or carry bearers with higher load per bearer (i.e. IAB node cannot give more resource to those BH RLC CHs with higher aggregate load)

For these topics, some companies proposed to extend BAP-header by including new fields for UE-bearer-ID during RAN2#113bis-e. However, because of significant overhead and no real benefit, these proposals were not agreed upon. The issue of unbalanced load on BH RLC channels carrying UE bearers with the same or similar QoS requirements does not need a specification solution as it can be addressed via proper implementation, for instance, by letting the CU to properly balance the traffic across the available/configured BH RLC channels. For instance, a simple implementation solution for this issue would be to assign a uniform number of DRBs per BH RLC channel and later reconfigures (via F1 signaling) the BH RLC channels, whenever there is a significant imbalance in the traffic load (i.e., number of UE DRBs) carried by such BH RLC channels. For example, RAN3 is working on defining congestion indications to allow the IAB node to report the congestion status per BH RLC channel. This indication can be obviously used by the CU to re-arrange the BH RLC channel configuration.

The issue of unbalanced load on BH RLC channels carrying UE bearers with the same or similar QoS requirements does not need a specification solution as it can be addressed via proper implementation, and IAB node indication on BH RLC channel congestion status (currently studied by RAN3).

Furthermore, related to IF-4, we also note that the F1 configuration allows the CU to indicate the QoS level parameters per BH RLC channel (see TS 38.473). For example, the CU can indicate for each BH RLC channel the GBR uplink/downlink bit rate that may reflect the amount of DRBs conveyed in such channel and their traffic characteristics. In our view, this is the information that an IAB node scheduler should take into account, together with the ingress data rate and obviously other QoS properties. 
To fix this issue, it was proposed in different contributions to indicate in the F1 configuration “the number of DRBs” conveyed in a certain BH RLC channel. In our view, it is not clear why “the number of DRBs” should be taken into account by the scheduler to prioritize certain BH RLC channels against others, because an higher “number of DRBs” does not necessarily imply higher bit rate of the individual DRBs or higher priority. For example, if there is a BH RLC channel with GBR requirement but with very few DRBs mapped, that should not be certainly treated with lower priority than another BH RLC channel with no GBR requirement and a lot of DRBs mapped.

A scheduler of an IAB node should take into account the already existing QoS properties of the BH RLC channels, rather than “the number of DRBs” which may not correctly reflect the characteristics of the traffic conveyed in the BH RLC channels.

Still related to IF-4, we note that the traffic patterns of the UEs/DRBs mapped to a certain BH RLC channel might be very much different from each other, depending not only on the QoS of the traffic but also on the traffic model generated by the end-user, the radio conditions, the congestion situations at the various intermediate IAB nodes, the number of hops apart, etc. Thus, indicating in the BH RLC channel configuration the “number of DRBs” would be like assuming that the BH RLC channel is always at “full load”, i.e. that all packets of this BH RLC channel always convey data for all the UEs/DRBs associated with this BH RLC channel. This is not a correct assumption in a real-world scenario.

Assuming that a BH RLC channel is always at “full load”, i.e. the BH RLC channel continuously conveys all the traffics for all DRBs/UEs, is not realistic. The traffic conveyed by one BH RLC channel for the various UEs/DRBs may vary over time depending several factors.

Hence, in our view, topology-wide fairness issues can be addressed via implementation, e.g., by simply letting the CU to properly balance the traffic across the available BH RLC channels possibly based on IAB node feedbacks on channel congestion status. If more granularity is required, RAN2 can assume that it can be addressed by implementation using the eLCID field space (i.e., 16 bits) for IAB networks. For example, establishing several BH RLC channels rather than one BH RLC channel for carrying all UE bearers traffic served by an IAB node. In addition, the IAB-donor CU can map the same number of UE bearers to each BH RLC channel and can then maintain/update this mapping whenever the network status changed due to either new UEs connecting to or some existing UEs depart from the network.

RAN2 to assume that IF-4 and IF-2 can be addressed by network implementation.

In any case, if RAN2 eventually decides to pursue some enhancements to the current framework, such enhancements should be adopted in the BAP header rather than F1 signalling, because F1 signaling might be inefficient for frequently updating the mapping of UE bearers to BH RLC channels, and would not give significant gains compared with existing techniques, i.e., the CU reconfigures the BH RLC channels when needed. Including this information in the BAP header would also give the advantage to the IAB node scheduler to always know the exact number of UEs/DRBs contained in a BAP packet, which may vary over time depending on a number of factors.

If RAN2 decides to design solutions for IF-2/IF-4, RAN2 should prioritize solutions in which the number of UEs/DRBs is conveyed in a BAP header, i.e. solution F2-3 in [1].

## 2.2	Topics Related to Multi-hop latency

- IL-1: IAB node cannot help ensure that overall or remaining PDB is met for a packet (e.g. by prioritizing bearers with higher number of hops), as it does not have a latency reference for the packets being scheduled, resulting in packets with the same QoS requirement ending up with different latency

For this topic, some companies argued to include timing information (e.g., timestamp or remaining PDB, etc.) to the BAP header so that the IAB node scheduler can prioritize packets based on latency and discard packets whose timer have expired. However, it is not obvious how this mechanism will work as highlighted by several companies during RAN2#113bis-e. We consider such a complex mechanism is not needed as the IAB-donor-CU gives a configuration (to IAB nodes) to meet the required QoS on a bearer basis, and Rel-16 IAB-donor CU knows the network topology under its domain. In other words, each backhaul channel and each route that is set up may have different levels of QoS, hence can lead to different latency. When routing packets, the network may choose an appropriate route to meet the QoS requirements including latency requirements. In addition, the pre-BSR might be able to assist the network to minimize UL latency.

Proper configuration of the BH bearers and routes are tools to deliver the committed QoS parameters rather than specifying new complex mechanisms such as including timing information in the BAP header.

Rel-16 IAB mechanism, such as pre-emptive BSR can assist the network to minimize UL latency.

Additionally, for ensuring PDB management, according to Rel-16 specifications, it is already possible for the CU to configure via F1 the IAB node with a PDB requirement per BH RLC channel. Since the Rel-16 IAB-donor CU knows the network topology under its domain, it can configure the PDB HbH at each node along the path towards an access IAB node. Based on this information, a network node can discard a packet if its scheduler cannot satisfy the PDB for the associated BH RLC channel, without the need to specify any specific discarding rule.

In Rel-16, the CU can configure an intermediate IAB node with a per-hop PDB per BH-RLC channel. The IAB node can apply packet discarding policies on the basis of this information and scheduling decisions.

If RAN2 foresee enhancement for improving HbH latency, then one such enhancement would be to extend the granularity of PDB configuration, making it possible for the IAB-donor CU to indicate to the IAB node the PDB per BAP destination for traffic mapped to a backhaul RLC channel, rather than just per backhaul RLC channel. However, on the issue of whether information related to the remaining PDB for a given packet should be transferred from one node to another, in our view, RAN2 should aim at minimum specification changes, and avoid specifying unnecessary procedures that can be left to the IAB node implementation.

For example, in some papers it was argued that a timestamp should be introduced. Besides the overhead to carry such timestamp in each packet, it not clear how an IAB node should use this information. In fact, the IAB node does not know neither the number of hops away to the destination, nor the latency requirements (PDB) for this traffic at the next hops. Moreover, considering that the PDB is already available at the IAB node, the network implementation can determine when/how to discard a certain packet, as it already happens in non-IAB networks. The only enhancement that can be envisaged is an indication in the BAP header on the PDB that was left at previous hop from the configured PDB. For example, if a parent IAB node was configured with a PDB of 30ms for a given BH RLC channel, and it committed the transmission in 20ms, then the remaining PDB is 10ms. This information can be used by the child node to adjust its configured PDB and schedule the packet accordingly.

If RAN2 foresee the need for solutions to solve IL-1 then RAN2 should focus on the following solutions:

  - Introducing a PDB per BH RLC channel per destination, i.e. solution L1-4 in [1].

  - Introducing an indication in each packet on the remaining PDB left at previous hop from the configured PDB of the BH RLC channel, i.e. solution L2-2 in [1].

RAN2 to avoid specifying latency-related solutions that can be handled by the IAB node implementation, e.g., packet discarding.

- IL-2: IAB node may need to report joint buffer status for LCHs which have rather differing QoS requirements, due to the current (Rel-16) limit on the number of LCGs

For this topic, RAN2 has agreed in RAN2#113-bis to extend the number of LCG for IAB-MT, while the size of LCG and enhancements to BSR are FFS. On the subject of the value for the LCG range, in our view, this value depends on the maximum number of configurable BH RLC channels (i.e., 65536) and how coarse information is provided to the parent IAB node DU. Considering that Rel-15 spec allows up to 64 LCIDs that can be grouped into 8 LCGs, if a similar equivalence is applied here it will result in a very large number of LCG which is maybe not desirable from overhead perspective. Hence, RAN2 should discuss a suitable value for LCG that can offer a good trade-off between overhead, flexibility, and valuable information for groups of UE-DRB pairs.

Rel-15 spec allows up to 64 LCIDs that can be grouped into 8 LCGs.

RAN2 to discuss a suitable number of LCGs that can offer a good trade-off between overhead, flexibility, and valuable information for groups of UE-DRB pairs.

- IL-3: Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors as it is left to implementation in Rel-16

This topic, i.e., “different ways to calculate buffer size for pre-emptive BSR” have been thoroughly discussed in Rel-16 and it was agreed that buffer size calculation may differ for nodes of different vendors and is left to the implementation. The fact that different vendors may use different ways to calculate the pre-emptive BSR has not to be seen as a problem. IAB nodes are network nodes, and 3GPP standardization work does not aim at aligning network implementations of different vendors. Additionally, it will be controversial and probably require a lot of standardization work to converge on one single mechanism. Given that the objective of the WID is “enhancements to improve topology-wide fairness, multi-hop latency, and congestion mitigation”, it is not clear how to prove that the selected mechanism will improve the IAB performances. We also note that introducing a standardized way to calculate the pre-emptive BSR for an IAB implementation may not just be an enhancement of an existing feature. Rather, that may require a redesign of the pre-emptive BSR mechanism which otherwise would not be compliant anymore with 3GPP specification. There seems to be no strong technical reason to require such a change to an IAB implementation.

During IAB Rel-16, it was agreed that buffer size calculation may differ for nodes of different vendors and is left to the implementation. Hence, it is not clear what benefit would bring standardizing the pre-emptive BSR calculation, especially considering the potential large standardization work.

- IL-5: The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free

- IL-6: The CU is unable to configure routing based on actual (real-time) latency per BH RLC channel

For these topics, few companies proposed that the IAB node to report RLC latency and/or load to the CU-CP. Considering that these values (i.e., latency and traffic load) may change frequently and it is unclear how these values will be measured, RAN2 should deprioritize such proposals. On the other hand, RAN3 has been discussing how to enhance both UP-based and CP-based approaches for DL congestion mitigation and baseline agreements have been made for both approaches.

The issues highlighted in IL-5 and IL-6 are more RAN3 specific and are related to E2E latency and flow control, which are already under discussion in RAN3.

RAN2 should not discuss potential solutions for issues highlighted in IL-5 and IL-6 since they are RAN3 specific.

## 2.3	Topics Related to Congestion mitigation

- IC-1: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets

- IC-7: CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion.

For these topics, in our view, RAN2 standardization work is not required, since RAN3 is already discussing ways to convey via F1 signalling more granular congestion indications (e.g. per BH RLC channels) to the CU.

It is up to RAN3 to address, if needed, the issues IC-1 and IC-7.

Lastly, some companies seek to bring back the topic of UL HbH flow control, which RAN2 already ruled out from the email discussion [1]. The UL HbH flow control would consist of flow control information sent in the DL from the parent to the child, that would supposedly use for some local decisions. First of all, we note that the UL scheduler resides in the parent, so it is not clear what the child should do upon receiving such information. That is different from the DL flow control feedback (sent from the child to the parent in the UL) because the parent does not know the DL buffer status of the child. However for the UL scheduling, the parent is in full control of the upstream traffic from child to parent. The child would obviously be affected by UL scheduling decisions at the parent, however it can simply determining by its UL buffer status whether UL congestion is occurring, i.e. the child does not need any UL HbH flow control indication from the parent to evaluate the UL congestion.

Hence it is not clear to us what is the technical motivation of the UL HbH flow control. If RAN2 intention is to enable some sort of local routing at the child, that is certainly doable, but it just enough to act on the buffer status at the child, e.g. the CU can configure rules on the UL buffer status at the child to enable local routing in the upstream.

Given the above, also considering the remaining limited number of time units for IAB Rel-17 to be completed, RAN2 should aim at addressing the issues shortlisted in RAN2#113e rather than including new topics.

It is not clear what is the benefit of the UL HbH flow control , given that the parent is in control of the UL scheduling and the child is aware of the UL buffer status.

RAN2 does not consider UL HbH flow control which was already downpriorized in RAN2#113e.

If RAN2 intention is to enable local routing in the upstream, it would be enough to introduce rules on the UL buffer status at the child, e.g. the CU can configure the UL buffer status level at the child to enable local routing. No need for the UL HbH flow control for the sake of UL local routing.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is not clear what benefits the sharing of the link qualities over multiple hops brings, especially considering the significant specification work, and the large overhead.

Observation 2	The schedulers of the various IAB nodes should act as independent entities, each of them trying to exploit at the best the radio link(s) they are serving, otherwise, this may result in an unbalanced utilization of the radio link capacity.

Observation 3	The issue of unbalanced load on BH RLC channels carrying UE bearers with the same or similar QoS requirements does not need a specification solution as it can be addressed via proper implementation, and IAB node indication on BH RLC channel congestion status (currently studied by RAN3).

Observation 4	A scheduler of an IAB node should take into account the already existing QoS properties of the BH RLC channels, rather than “the number of DRBs” which may not correctly reflect the characteristics of the traffic conveyed in the BH RLC channels.

Observation 5	Assuming that a BH RLC channel is always at “full load”, i.e. the BH RLC channel continuously conveys all the traffics for all DRBs/UEs, is not realistic. The traffic conveyed by one BH RLC channel for the various UEs/DRBs may vary over time depending several factors.

Observation 6	Proper configuration of the BH bearers and routes are tools to deliver the committed QoS parameters rather than specifying new complex mechanisms such as including timing information in the BAP header.

Observation 7	Rel-16 IAB mechanism, such as pre-emptive BSR can assist the network to minimize UL latency.

Observation 8	In Rel-16, the CU can configure an intermediate IAB node with a per-hop PDB per BH-RLC channel. The IAB node can apply packet discarding policies on the basis of this information and scheduling decisions.

Observation 9	Rel-15 spec allows up to 64 LCIDs that can be grouped into 8 LCGs.

Observation 10	During IAB Rel-16, it was agreed that buffer size calculation may differ for nodes of different vendors and is left to the implementation. Hence, it is not clear what benefit would bring standardizing the pre-emptive BSR calculation, especially considering the potential large standardization work.

Observation 11	The issues highlighted in IL-5 and IL-6 are more RAN3 specific and are related to E2E latency and flow control, which are already under discussion in RAN3.

Observation 12	It is not clear what is the benefit of the UL HbH flow control , given that the parent is in control of the UL scheduling and the child is aware of the UL buffer status.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to downprioritize any proposed solution related to issue IF-1.

Proposal 2	RAN2 to assume that IF-4 and IF-2 can be addressed by network implementation.

Proposal 3	If RAN2 decides to design solutions for IF-2/IF-4, RAN2 should prioritize solutions in which the number of UEs/DRBs is conveyed in a BAP header, i.e. solution F2-3 in [1].

Proposal 4	If RAN2 foresee the need for solutions to solve IL-1 then RAN2 should focus on the following solutions:

a.	Introducing a PDB per BH RLC channel per destination, i.e. solution L1-4 in [1].

b.	Introducing an indication in each packet on the remaining PDB left at previous hop from the configured PDB of the BH RLC channel, i.e. solution L2-2 in [1].

Proposal 5	RAN2 to avoid specifying latency-related solutions that can be handled by the IAB node implementation, e.g., packet discarding.

Proposal 6	RAN2 to discuss a suitable number of LCGs that can offer a good trade-off between overhead, flexibility, and valuable information for groups of UE-DRB pairs.

Proposal 7	RAN2 should not discuss potential solutions for issues highlighted in IL-5 and IL-6 since they are RAN3 specific.

Proposal 8	It is up to RAN3 to address, if needed, the issues IC-1 and IC-7.

Proposal 9	RAN2 does not consider UL HbH flow control which was already downpriorized in RAN2#113e.

Proposal 10	If RAN2 intention is to enable local routing in the upstream, it would be enough to introduce rules on the UL buffer status at the child, e.g. the CU can configure the UL buffer status level at the child to enable local routing. No need for the UL HbH flow control for the sake of UL local routing.

# 4	References

- R2-2101168, Report from email discussion [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung), Samsung (Rapporteur), 3GPP TSG-RAN WG2 #113-e.