3GPP TSG-RAN WG2 Meeting #115 electronic	                                         R2-2108524
Online, August 16 – August 27, 2021

Agenda Item:	8.12.2.2

Source: 	CMCC

Title:  	Discussion on identification and access restrictions

WID/SID:	         NR_redcap

Document for:	Discussion and Decision

Introduction

In the RAN2#114-e meeting, RedCap UE early identification and access restrictions were discussed, following are the agreements made in RAN#114-e meeting:

Agreements:

- SIB1 (not MIB) indicates cell barring for 1 Rx branch and 2 Rx branches separately for RedCap UEs. Further details of the solution are FFS

- The cell barring for RedCap UE is per cell (not per PLMN).

- RedCap UE supports the Intra Frequency Reselection Indicator.

- Either Msg1 and/or Msg3 early identification will be supported

- There is no need to support Rx branches specific early identification from RAN2 perceptive (final decision up to RAN1).

- Send LS to ask RAN3 to consider the coordination between gNBs on whether a neighbour/target gNB supports RedCap UEs, if needed, to avoid handover RedCap to a target cell that it can’t access. We can come back in the next meeting with discussions on other restrictions, e.g. related to number of RX.

Based on the discussion progress, we provide our view on RedCap UE cell selection/reselection and RedCap UE early identification in this contribution.

Discussion

2.1 Camping restriction

As described in the WID, RedCap UEs have reduced capabilities, for example reduced maximum UE bandwidth:

- Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

Although the maximum UE bandwidth is reduced,  it would be found that, no matter RedCap UE is in FR1 or FR2, it’s fine for them to acquire the system information.

Observation 1: RedCap UEs have the capability to acquire the same MIB as legacy UEs.

For legacy UE, the IE Cellbarred is used to indicate whether the cell is barred. Though it was agreed that SIB1 may indicate cell barring for 1RX branch and 2RX branches separately, but in our opinion, it should be under the assumption that the cell is not barred indicated by IE Cellbarred. If RedCap UEs ignore the IE CellBarred in MIB is supported, it means there’s a cell barring the legacy UEs but serving RedCap UEs, and it is a little strange.

Proposal 1：UE should not ignore CellBarred in MIB.

Similarly, the IE intraFreqReselection is used to control cell selection/reselection to intra-frequency cells when the highest ranked cell is barred, or treated as barred by the UE. From our point of view, it could be used for all types of UEs.

Proposal 2: there’s no need to introduce RedCap specific IFRI.

2.2 Cell selection/Reselection

Due to the reduced capability of RedCap UEs, like reduced number of RX branches, RedCap UEs may have different performance (i.e. Coverage, Data rate, Network capacity and spectral efficiency) compared with legacy UEs. And allowing not all the cells support RedCap UEs access is consistent with network deployment flexibility requirements. Because of the different performance between Redcap UEs and legacy UEs, taking different measures of access restrictions, cell selection/reselection parameters and reselection priorities for RedCap UEs is benifit for increasing the flexibility in network deployment and helpful to ensure network performance and user experience.

Proposal 3: Specific cell selection/reselection parameters and priorities could be introduced for RedCap UEs.

RedCap UEs also need to perform cell reselection procedures based on RRM measurements on serving cell and neighbour cells in IDLE/INACTIVE for Mobility management just like legacy UEs. If not all of the cells in the network allow access from RedCap UEs and a RedCap UE cannot know whether the target neighbour cell supports RedCap or not, it may perform some unnecessary measurements of intra/ inter-frequency cells before a suitable cell is found which fulfil the cell reselection criteria and support RedCap UEs access. This issue may extend the cell reselection procedures and increase RedCap UEs power consumption. Therefore, it’s better to indicate whether the neighbour cells could accept RedCap UEs’ access in the system information, and the solution could be further discussed.

Proposal 4: It’s better to indicate whether the neighbour cells could accept RedCap UEs’ access in the system information.

Besides, in a network where not all the cells of the same frequency support RedCap UEs access, RedCap UEs may be interfered by the neighbour cell which is the highest ranked cell according to cell reselection criteria but not support RedCap UEs access. It is necessary to discuss the solution of potential interference issues for RedCap UEs.

Proposal 5：RAN2 is kindly asked to discuss the possibility and the solution of potential interference issues for RedCap UEs.

2.3 Early identification

Early identification of RedCap UEs has been discussed for a long time since SI phase in both RAN1 and RAN2, and the benefits of different approaches are analysed in TR 38.875 [1].

In last meeting, RAN2 agreed that either Msg1 and/or Msg3 early identification will be supported, and there’s no need to support Rx branches specific early identification from RAN2 perceptive (final decision up to RAN1).

And the key issue of the discussion is whether both Msg1 and Msg 3 based solution should be supported, while Msg1 based solution is up to RAN1’s input.

In the last two RAN1 meetings，early identification were discussed，and some working assumptions and agreement were achieved as following ：

RAN1#104-e meeting Agreements: - At least using UE capability report according the existing framework to indicate (implicitly or explicitly) the number of Rx branches - FFS: whether/how to support earlier indication of Redcap UEs with # Rx branches by Msg1 and/or Msg3, and MsgA - FFS: Network configurability of early indication of the number of Rx branches via SIB1, if supported RAN1#105-e meeting Working assumption: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled - FFS How to support enable/disable the early indication - FFS details e.g.: - separate initial UL BWP - separate PRACH resource - PRACH preamble partitioning - FFS the possibility of supporting Msg3 for the early indication Agreement: (if the above working assumption is confirmed) - Early indication of RedCap UEs in Msg1 can be enabled/disabled via SIB

From the agreements and working assumption above, it could be found that RAN1’s discussion is mainly fucus on Msg 1 based solution. To avoid repetitive discussion, it’s better for RAN2 to send LS to inform RAN our progress, especially that RAN2 does not support separate Rx branches specific early identification.

Proposal 6：RAN2 is kindly asked to send LS to RAN1 to inform RAN2’s progress.

Besides, it should be noticed that, it was agreed in RAN1 that early identification of RedCap UEs in Msg 1 could be enabled/disabled via SIB. From our perspective, if only Msg 1 based solution is agreed as early identification approach, the RedCap specific configuration could be treated as the identification of enabling/disabling in SIB, which means no explicit indication in SIB is needed. However, if both Msg1 based solution and Msg 3 based solution are agreed, it’s better to have explicit indication in SIB to inform whether early identification is enabled/disabled, and taking the specific configuration existing or not, UE could know which early identification solution is used.

Proposal 7: If only Msg 1 based solution is agreed as early identification approach, the RedCap specific configuration could be treated as the identification of enabling/disabling in SIB, otherwise, if both Msg1 based solution and Msg 3 based solution are agreed, it’s better to have explicit indication in SIB to inform whether early identification is enabled/disabled.

Conclusion

In this contribution, we discussed RedCap related issues, including camping indication，cell selection/reselection and early identification. Following are our observations:

Observation 1: RedCap UEs have the capability to acquire the same MIB as legacy UEs.

Based on the observations, we propose:

Proposal 1：UE should not ignore CellBarred in MIB.

Proposal 2: there’s no need to introduce RedCap specific IFRI.

Proposal 3: Specific cell selection/reselection parameters and priorities could be introduced for RedCap UEs.

Proposal 4: It’s better to indicate whether the neighbour cells could accept RedCap UEs’ access in the system information.

Proposal 5：RAN2 is kindly asked to discuss the possibility and the solution of potential interference issues for RedCap UEs.

Proposal 6：RAN2 is kindly asked to send LS to RAN1 to inform RAN2’s progress.

Proposal 7: If only Msg 1 based solution is agreed as early identification approach, the RedCap specific configuration could be treated as the identification of enabling/disabling in SIB, otherwise, if both Msg1 based solution and Msg 3 based solution are agreed, it’s better to have explicit indication in SIB to inform whether early identification is enabled/disabled.

Reference

[1] RP-210918 Revised WID on support of reduced capability NR devices, Nokia, Ericsson