3GPP TSG-RAN WG2 Meeting #115-e	R2-2108613

Online, 9-27 August, 2021

Source: 	Huawei, HiSilicon

Title: 	UE onboarding and remote provisioning for SNPN

Agenda Item:	8.16.3

Document for:	Discussion and decision

Introduction

At RAN2#113-e, RAN2#113b-e and RAN2#114-e, the RAN impact for supporting UE onboarding was discussed and the following agreements were achieved [1][2][3]:

Broadcast a 1-bit indication for onboarding per O-SNPN.

R2 assumes that the 1-bit indication for onboarding is in SIB1.

The UE sends an indication for onboarding to the gNB at RRC Connection Establishment (intention to support AMF selection).

Focus on the O-SNPN scenario. Wait for SA2 further conclusion on how a PLMN can be used as onboarding network.

Will continue offline on the LS questions.

UE AS forwards the onboarding indication (and Group IDs if Proposal#1 is agreed) per SNPN to UE NAS for onboarding network selection.

No UE impact on connected mode mobility for onboarding.

A new onboarding indication is included in RRCSetupComplete message.

R2 assumes that no enhancement is needed to support onboarding for provisioning the PNI-NPN credentials to UE.

There is no need to introduce an onboarding request indication in RRC messages for UEs in RRC_INACTIVE.

Group IDs per SNPN for onboarding purpose is broadcast in the SIB. FFS whether the Group IDs for onboarding purpose and for credential by separate entity are different.

R2 assumes that onboarding will not impact cell reselection.

[029] No additional information except for the already agreed broadcast parameters is needed, unless requested by other WG.

[029] There is no need to introduce the 1-bit onboarding indication in SIB1 and optional GINs for PLMNs acting as onboarding networks.

[029] Toggling the 1-bit onboarding indication in SIB1 allows to control congestion due to onboarding request.

[029] RAN2 confirms that onboarding does not impact the cell reselection procedure.

[029] For AMF routing, no extra information is needed in addition to the already agreed onboarding request indication in RRCSetupComplete, unless explicitly requested by other WGs.

[029] Any limitation to a selected set of UEs using uSIM tags is out of RAN2 scope.

In this paper, we further analyse the possible RAN2 impacts for supporting UE onboarding and remote provisioning.

Discussion

- 2.1 Cell suitability for cell selection

As the support of onboarding may be non-uniform within the O-SNPN, different onboarding information may be broadcast by cells of the O-SNPN. When selecting a cell to carry out onboarding, the UE shall consider the onboarding indication and the GINs broadcast by cells. Therefore, for a UE indicated for Onboarding via SNPN, an additional condition for cell suitability check may be introduced to take into account the UE onboarding related information (e.g. whether the SNPN cell supports onboarding, the selected/available GINs, etc.). In addition, as onboarding is a one-shot procedure, it does not impact the cell reselection. Therefore, the update of the cell suitability definition applies only to cell selection and does not apply to cell reselection.

It is also worth pointing out that, this additional condition related to UE onboarding should have no impact on the UEs that do not support onboarding via SNPN.

For a UE indicated to perform Onboarding via SNPN, an additional condition for cell suitability check may be introduced to consider the UE Onboarding related information (e.g. "onboarding support" indication, selected GIN(s), etc.). This additional condition, if agreed, applies only to cell selection procedure for the UE performing onboarding via SNPN.

Conclusion

In this paper, we discussed the impact for supporting UE onboarding and remote provisioning for SNPN and proposed the following:

- For a UE indicated to perform Onboarding via SNPN, an additional condition for cell suitability check may be introduced to consider the UE Onboarding related information (e.g. "onboarding support" indication, selected GIN(s), etc.). This additional condition, if agreed, applies only to cell selection procedure for the UE performing onboarding via SNPN.

References

- R2-2102601, RAN2#113-e Meeting Report

- R2-2106641, RAN2#113bis-e Meeting Report

- RAN2-114-e ChairmanNotes EOM Rev2 2021-06-15.docx