3GPP TSG-RAN2 Meeting #115 electronic	R2-2108615

# Online, 9-27 August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2783 | rev |  | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on encoding format for HRNN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN-Core |  |  |  |  |  | Date: |  |  | 2021-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current specification, it is not specified which encoding format (e.g., ASCII, Unicode, UTF-8) is used for the HRNNs in SIB10. As a result, the UE does not know how to decode the HRNNs. Therefore, a specific encoding format for HRNNs should be specified to align the encoding/decoding format between network and UE. As UTF-8 is a variable-length encoding format and applies to a wide range of languages, and the name of the home eNB in LTE also is coded in UTF-8, the HRNNs in SIB10 can be coded in UTF-8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that the HRNNs in SIB10 is coded in UTF-8. Impact analysis Impacted 5G architecture options: NR SA, NR DC Impacted functionality: Decoding of the HRNNs Inter-operability: If the UE is implemented according to this CR while the network is not, or vice versa, UE may not correctly decode the HRNNs, which is the same behaviour as the current specification and will not make things worse. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot determine the encoding format of the HRNNs used by network, which may cause that UE cannot correctly decode the HRNNs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.3.1	System information blocks

#### –	SIB10

SIB10 contains the HRNNs of the NPNs listed in SIB1.

SIB10 information element

-- ASN1START

-- TAG-SIB10-START

SIB10-r16 ::=               SEQUENCE {

hrnn-List-r16               HRNN-List-r16                                   OPTIONAL,   -- Need R

lateNonCriticalExtension    OCTET STRING                                    OPTIONAL,

...

}

HRNN-List-r16 ::=           SEQUENCE (SIZE (1..maxNPN-r16)) OF HRNN-r16

HRNN-r16 ::=                SEQUENCE {

hrnn-r16                    OCTET STRING (SIZE(1.. maxHRNN-Len-r16))        OPTIONAL   -- Need R

}

-- TAG-SIB10-STOP

-- ASN1STOP

SIB10 field descriptions

HRNN-List The same amount of HRNN elements as the number of NPNs in SIB 1 are included. The n-th entry of HRNN-List contains the human readable network name of the n-th NPN of SIB1, coded in UTF-8 with variable number of bytes per character. The hrnn in the corresponding entry in HRNN-List is absent if there is no HRNN associated with the given NPN.

End OF CHANGES