3GPP TSG-RAN WG2 Meeting #115-e											  	     R2-2108653

# Electronic Meeting, August 09 – 27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0218 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ASF CAG Priority |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_enh-Core |  |  |  |  |  | Date: |  |  | 2021-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Per the current spec, it was mentioned that the UE when uses the ASF to detect CAG cells, UE has to follow the cell reselection criteria based on dedicated frequency priorities. It’s not clear, whether the UE should also follow the frequency priorities broadcasted in the SIBs or not. To clarify the UE expected behavior, UE should be allowed to use all the information provided to it, i.e. through dedicated RRC messages and broadcasted SI messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying that the UE should be allowed to use all available information about frequency priorities provided to the UE by the network (i.e. dedicated and broadcated). Impact Analysis: Impacted 5G architecture options: NR-SA, NR-DC, NE-DC, (NG)EN-DC Impacted functionality: CAG Interoperability issue: - if the network is implemented according to the CR and the UE is not, no interoperability issue is expected as UE will still be able to use information (frequency priorities) provided the RRC dedicated message to find a CAG cell. - if the UE is implemented according to the CR and the network is not, no interoperability issue, as UE eventually will be able to find a CAG cell, by using the information (frequency priorities) provided by the dedicated message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The confusion of whether the UE should only be allowed to use the dedicated frequency priorities for the reselection criteria for a CAG cell, or it should also consider the broadcasted frequency priorities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Start

#### 5.2.4.10	Cell reselection with CAG cells

In addition to normal cell reselection, a UE may optionally use an autonomous search function to detect CAG cells on serving and non-serving frequencies. However UE shall follow the cell reselection criteria based on network provided frequency priorities and only follow the autonomous cell search result if the result fulfils also the existing cell reselection criteria based on network provided frequency priorities.

Change End