3GPP TSG-RAN WG2 Meeting #115-e		R2-2108671

E-Meeting, 16th – 27th Aug, 2021

Source:	vivo

Title:	R17 TX switching enhancements

Agenda Item:	8.22

Document for:	Discussion and Decision

- Introduction

According to RAN#92-e meeting, the following scenarios have been extended to the uplink Tx switching feature in the Rel-17 FR1 RF requirements enhancement WI [1]:

- Scenario1: 2Tx-2Tx switching between two uplink carriers for SUL and UL CA

- Scenario2: 1Tx-2Tx and 2Tx-2Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B for SUL and UL CA

At the last RAN2#114-e meeting, there was some discussions on this topic and agreements reached as below.

For Rel-17 UL Tx switching enhancements, RAN2 to use the UE capability reporting signalling framework of R16 1Tx-2Tx UL Tx switching as baseline and assume the R17 UE capability should be reported in the UL Tx switching specific BC list introduced in R16 (i.e. BandCombinationList-UplinkTxSwitch) unless issue is found later.

Postpone to next meeting (expect to make better progress next meeting based on Further R1 R4 progress).

In this contribution we will further discuss the R17 TX switching enhancements based on RAN1 progress made at RAN1#105-e meeting [2].

- Discussion

  - Background

During RAN1#105-e[2], there were new agreements on R17 TX switching which are listed as below.

Agreement:

- For a UE configured with higher layer parameter supplementaryUplink and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused.

Agreement:

- For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused with the following add-on.

  - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

Agreement:

- For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured:

  - For option 2 of mapping between UL transmission ports and Tx chain

    - The switching period is only applicable in the following cases:

      - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2.

      - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1.

      - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2.

    - For other cases, the state of Tx chains of last UL transmission is assumed.

  - Note: For SUL, UL CA option 1 and UL CA option 2, in RAN1 understanding, no spec change to power configuration and power control.

Agreement:

- For a UE configured with 2Tx-2Tx UL Tx switching between two uplink carriers and configured with UL CA Option 2, if the state of Tx chains after UL Tx switching is not unique, a rule to determine the state of Tx chains after Tx switching is to be specified.

FFS: The state of Tx chains with the most of Tx chains on the most important uplink carrier is assumed, e.g. the carrier with uplinkTxSwitchingPeriodLocation configured as false.

In R16 TX Swithcing, the following options and cases are supported:

For inter-band UL CA, if UE reports via capability signaling to support uplink Tx switching, UE further reports via capability signaling which option (between Option 1 and Option 2) is supported.

- Option 1: If uplink Tx switching is configured, UE is not expected to be scheduled or configured with UL transmission on carrier 2 for case 1.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |

- Option 2: If uplink Tx switching is configured, UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 for case 1.

  - UE can be scheduled or configured with UL transmission on either carrier 1 or carrier 2.

  - UE can be scheduled or configured with UL transmission on both carrier 1 and carrier 2 simultaneously.

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |

Based on Relase 16 works and current Release 17 progress on TX switching, it is noticeable that both switched UL mode(Option 1) and dual UL mode(option 2) are supported.

- Compared to R16, R17 TX switching also supports both Switched UL mode(Option 1) and dual UL mode(option 2) as shown in Table 1.

Table 1: R16 and R17 TX switching support of both option 1 and option 2

| TX switching | Option1 (switched UL mode) | Option2(dual UL mode) |
| --- | --- | --- |
| R16 1TX-2TX switching | Only two UL carriers | Only two UL carriers |
| R17 1TX-2TX switching | Three UL carriers | Three UL carriers |
| R17 2TX-2TX switching | Both two UL carriers and three UL carriers | Both two UL carriers and three UL carriers |

  - Capability report for R17 Tx switching

RAN1 sent a LS to ask RAN4 whether the switching time reported by a UE for 2Tx-2Tx switching can be different from that reported by the UE for 1Tx-2Tx switching in three carriers case. RAN4 replied in R4-2107847[3] as below:

- For UL Tx switching in a band pair of a band combination, the set of candidate switching time for 2Tx-2Tx switching is the same as that for 1Tx-2Tx switching, i.e., the same set of {35us, 140us, 210us}.

- The exact reported value of switching time for a band pair of a band combination can be different for 2Tx-2Tx switching and 1Tx-2Tx switching.

- Meanwhile, for UE supporting 2Tx-2Tx switching, it means that the UE supports 1Tx-2Tx as well. In the case that UE only reports the capability for 2Tx-2Tx switching, the same switching time can also be applied to 1Tx-2Tx switching.

- UE capability of R17 Tx switching can be reported independently from that of R16 Tx switching.

From the UE implementation point of view, at least granularity of UE capability for TX switching should be differentinated for different cases. Based on summary from above, we can categorize the TX switching enhancement into the below sub capabilities.

- R16 1TX-2TX switching (Two UL carriers)  existing

- R17 1TX-2TX switching (Three UL carriers)

- R17 2TX-2TX switching (Two UL carriers)

- R17 2TX-2TX switching (Three UL carriers)

- Introduce at least three new R17 UE capabilities for Tx switching based on supported Tx switching type and carrier number as below:

- R17 1TX-2TX switching (Three UL carriers)

- R17 2TX-2TX switching (Two UL carriers)

- R17 2TX-2TX switching (Three UL carriers)

In R16 1TX-2TX switching (Two UL carriers), UL Tx Switching Band Pair and uplink Tx Switching-Option are reported for R16 TX swiching per band combination. R17 TX switching also has these two options and two bands for TX Switching. UL Tx Switching Band Pair and uplink Tx Switching-Option can be reported per three new R17 TX switching sub capabilities.

- UL Tx Switching Band Pair and uplink Tx Switching-Option can be reported as per three new R17 capabilities for R17 TX switching per band combination.

In R1-2106150[4], conclusion regarding bullet 2.6-Tx switching between case 1 and case 3 for UL CA option 1 and SUL is stated as below:

- For uplink Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, whether Tx switching between 2Tx on Band A and 1Tx on Band A+1Tx on Band B for UL CA option 1 and SUL is included in WID could be clarified by RAN plenary or RAN4.

For R16 1TX-2TX switching (Two UL carriers) UE capability reporting, serving cell indicates whether its carrier is carrier 1 or carrier 2 for UL Tx switching in RRC IE ServingCellConfig, which is enough with only 1 carrier per band. However, since R17 1TX-2TX switching (1Tx on Band A and 2Tx on Band B) is defined and 2TX-1TX switching (2Tx on Band A and 1Tx on Band B) might be defined, it is not suitable to extend  the value of uplinkTxSwitchingCarrier-r16 in RRC IE ServingCellConfig to {carrier1, carrier2, carrier3} only, which might lead to UE confusion of interpreting whether carrier2 belongs to Band A or Band B. Therefore, a straight forward solution is that UE reports the bandIndexUL according to Band A or Band B, and the serving cell indicates the band type of its carrier, i.e. Band A or Band B.

- For R17 UL Tx switching, the UE reports the bandIndexUL according to the band type, and the serving cell indicates the band type of its carrier.

  - RRC configuration for R17 switching

In R16 uplink Tx Switching Option, uplink Tx Switching Period Location and uplink Tx Switching Carrier types are configured to enable the R16 TX switching.

uplinkTxSwitchingOption Indicates which option is configured for dynamic UL Tx switching for inter-band UL CA or EN-DC. The field is set to switchedUL if network configures option 1 as specified in TS 38.214 [19], or dualUL if network configures option 2 as specified in TS 38.214 [19]. Network always configures UE with a value for this field in inter-band UL CA case and EN-DC case where UE supports dynamic UL Tx switching. uplinkTxSwitchingPeriodLocation Indicates whether the location of UL Tx switching period is configured in this uplink carrier in case of inter-band UL CA, SUL, or EN-DC, as specified in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of inter-band UL CA or SUL, network configures this field to TRUE for one of the uplink carriers involved in dynamic UL TX switching and configures this field in the other carrier to FALSE. In case of EN-DC, network always configures this field to TRUE for NR carrier (i.e. with EN-DC, the UL switching period always occurs on the NR carrier). uplinkTxSwitchingCarrier Indicates that the configured carrier is carrier1 or carrier2 for dynamic uplink Tx switching, as defined in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of inter-band UL CA or SUL, network configures one of the two uplink carriers involved in dynamic UL TX switching as carrier1 and the other as carrier2. In case of EN-DC, network always configures the NR carrier as carrier 2.

In R17 uplinkTxSwitching Option, uplinkTxSwitchingPeriod Location and uplinkTxSwitchingCarrier types are also needed. The uplink Tx Switching Carrier types can be extended to three carriers.

- In R17 uplinkTxSwitching Option, uplinkTxSwitchingPeriod Location and uplinkTxSwitchingCarrier types are also needed. The uplink Tx Switching Carrier types can be extended to three carriers.

For one band combination, the network can explicity configure the UE with one TX switching capability, there maybe some fallback relations between R16 and R17 TX switching based on the UE MIMO capability reporting puls R16 and R17 TX switching capability report. However there will be no ambiguty for the UE to switch the TX if the network can  configure the UE with TX operation type explicitly. One of Three configuration types can be added per CC with uplinkTxSwitchingPeriod Location and uplinkTxSwitchingCarrier types .

- Type1: R17 1TX-2TX switching (Three UL carriers) configuration

- Type2: R17 2TX-2TX switching (Three UL carriers) configuration

- Type3: R17 2TX-2TX switching (Three UL carriers) configuration

- One of the three configuration types can be added per CC with uplinkTxSwitchingPeriod Location and uplinkTxSwitchingCarrier types as below:

- Type1: R17 1TX-2TX switching (Three UL carriers) configuration

- Type2: R17 2TX-2TX switching (Two UL carriers) configuration

- Type3: R17 2TX-2TX switching (Three UL carriers) configuration

- Conclusion

Based on the discussion, we have the following observation and proposals:

- Comparing with R16, R17 TX switching also supports both Swiched UL mode (Option 1) and dual UL modes (option 2).

- UE capability of R17 Tx switching can be reported independently from that of R16 Tx switching.

- Introduce at least three new R17 UE capabilities for Tx switching based on supported Tx switching type and carrier number as below:

- R17 1TX-2TX switching (Three UL carriers)

- R17 2TX-2TX switching (Two UL carriers)

- R17 2TX-2TX switching (Three UL carriers)

- UL Tx Switching Band Pair and uplink Tx Switching-Option can be reported as per three new R17 capabilities for R17 TX switching per band combination.

- For R17 UL Tx switching, the UE reports the bandIndexUL according to the band type, and the serving cell indicates the band type of its carrier.

- In R17 uplinkTxSwitching Option, uplinkTxSwitchingPeriod Location and uplinkTxSwitchingCarrier types are also needed. The uplink Tx Switching Carrier types can be extended to three carriers.

- One of the three configuration types can be added per CC with uplinkTxSwitchingPeriod Location and uplinkTxSwitchingCarrier types as below:

- Type1: R17 1TX-2TX switching (Three UL carriers) configuration

- Type2: R17 2TX-2TX switching (Two UL carriers) configuration

- Type3: R17 2TX-2TX switching (Three UL carriers) configuration

- References

- RP-211368, revised WID: RF requirements enhancement for NR frequency range 1 (FR1)

- RAN1 Chair’s Notes 105

- R4-2107847, Reply LS on Rel-17 uplink Tx switching

- R1-2106150, Summary of email discussion on Rel-17 uplink Tx switching