3GPP TSG-RAN WG2 Meeting #115 electronic   						    	R2-2108800

E-Meeting, 9th Aug – 27th Aug, 2021

Source: 	Xiaomi Communications

Title:  	PRACH congestion due to multicast paging

Agenda Item:	8.1.3.2

Document for:	Discussion and Decision

# Introduction

In the RAN2#114e meeting [1], RAN2 made the following agreements for the MBS paging for delivery mode 1:

- Use PCCH for Multicast activation notification (also for MBS supporting nodes). - Confirm that we convey the MBS session ID in the notification. - Use of paging in all (legacy) PO with PRNTI is the baseline assumption (can still discuss other variants).

In this contribution, we discuss the PRACH collision issue caused by the multicast paging.

# Discussion

## PRACH congestion

When a UE is interested to receive a MBS service, the UE would monitor the multicast paging, so as to receive the activation indication (i.e. the MBS session ID) from the PCCH. When the UE receives the activation indication of its interested MBS session, the UE will start to setup the RRC connection by using the RACH procedure. When the number of UEs interested in the MBS service are large, the PRACH resource could be congested. Then the PRACH collision issue will happen.

Here, we think that RAN2 should find a solution to alleviate the PRACH congestion issue caused by the multicast paging. One simple solution would be to re-use the UAC framework by providing the access parameters (as provided above) for the multicast paging. Then the RRC connection requests (e.g. RRC connection establishment or RRC connection resumption) from the UEs interested in the multicast service are randomized. The detailed configurations of the access parameters for the MBS paging can be left to the stage-3 discussion.

Observation: The access parameters for UAC can randomize the access requests from multiple UEs.

Proposal 1: After the reception of the multicast session activation in paging, the RRC connection request from UEs are randomized.

Proposal 2: The access parameters as configured for the UAC can be configured for the access request due to the multicast paging.

# Conclusions

According to the analysis given above, we have the following observation and proposals:

Observation: The access parameters for UAC can randomize the access requests from multiple UEs.

Proposal 1: After the reception of the multicast session activation in paging, the RRC connection request from UEs are randomized.

Proposal 2: The access parameters as configured for the UAC can be configured for the access request due to the multicast paging.

# Reference

[1] RAN2#113-e meeting minutes.