3GPP TSG RAN WG2#116-e	R2-2109346

e-Meeting, 1st - 12th November, 2021

3GPP TSG-RAN WG3 #113-e	R3-214464

16-26 Aug 2021

Title:	LS on UP security policy updated by intra-cell handover

Response to:

Release:	Release 16

Work Item:	TEI16

Source:	RAN3

To:	RAN2

Cc:	SA3

Contact Person:

#### Name:	Sen Xu

E-mail Address:	xusen@chinatelecom.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

Per TS33.501 clauses 6.1.1, at Xn handover SMF shall send its locally stored UE's UP security policy of the corresponding PDU sessions to the target gNB via Path Switch Request Acknowledge message. If this UE's UP security policy does not match the one ongoing in the target ng-eNB/gNB the target gNB shall initiate intra-cell handover procedure. This procedure includes RRC Connection Reconfiguration procedure to reconfigure the DRBs to activate or de-activate the UP integrity/confidentiality as per the received policy from SMF.

However, according to TS38.331, ciphering and integrity protection can be changed only by releasing and adding the DRB. RAN3 would like to know if the UP security policy of an ongoing DRB can be updated by releasing and adding the same DRB ID within the same RRC Reconfiguration message through intra-cell handover procedure.

RAN3 has discussed whether UP security policy can be updated via E1 message UE Context Modification. Before making any such decision, RAN3 would like to confirm with RAN2 if the enabling/disabling of ciphering or integrity protection of one or multiple DRBs can be changed by intra-cell handover.

2. Actions:

To RAN1 : 	RAN3 kindly requests RAN2 to provide feedback whether the enabling/disabling of ciphering or integrity protection of one or multiple DRBs can be achieved by intra-cell handover within one RRC reconfiguration message.

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP TSG RAN WG3#114-e	1 Nov - 11 Nov, 2021			Online

3GPP TSG RAN WG3#115	-e	21 - 25 Feb, 2022			Athens, GR