3GPP TSG RAN WG2#116-e	R2-2109353

e-Meeting, 1st - 12th November, 2021

3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2114751

# Electronic Meeting, August 16-27, 2021

Title:	LS on specification impact for methods on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths

Response to:

Release:	Rel-17

Study Item:	FS_NR_eff_BW_util

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

Name: 	Iwajlo Angelow

E-mail Address: 	iwajlo.angelow@nokia.com

Attachments:	N/A

1. Overall Description:

RAN4 is currently studying four methods [1] on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths. One of the Study Item objectives is to evaluate and minimize the impact to RAN1 and RAN2.

RAN4 respectfully requests to provide information if each studied method is compatible with RAN1/RAN2 specifications and in particular to clarify/confirm the following aspects:

- For the wider CBW:

  - clarify if there is any limitation for the UL carrier positions (not just BWP positions) legacy UEs support for uplinkChannelBW-PerSCS-List and scs-SpecificCarrierList in symmetric operating bands with a fixed duplex distance and asymmetric UL/DL channel bandwidth.

  - confirm UE behaviour if it is possible to configure a carrier that is not fully contained in the NR band, i.e. the carrier can extend beyond the low edge of the band and/or the high edge of the band?

- For the overlapping CBWs from network perspective (one cell approach):

  - clarify whether a single SSB and CORESET (e.g. for cases where irregular BWs >10 MHz where a 4.28 MHz wide initial BWP can be in the common frequency range), can be used to configure UEs with different channel BWs on different parts of the BS channel.

  - clarify whether two time staggered SSBs and CORESET#0 on the same frequency (when the frequency separation is not enough to send them simultaneously at the same time and thus time staggering is needed) are supported in RAN1/2 specifications so that UEs configured with left and right channels of the next smaller regular size can track their own time staggered SSB and CORESET#0.

- For the overlapping CBWs from UE perspective (two cell approach / CA approach):

  - if two different Bandwidth Parts for the UE are overlapping, and both contain a subset of CSI-RS resources that are mapped to the same subset of overlapping RBs for the same UE, please clarify how does UE report CSI for the overlapped part, e.g. does UE report CSI for each cell separately, or just once for the overlapping part, or something else?

  - clarify how PDCCH reception in overlapped CA when PCell and SCell PDCCH resources partially overlap and whether there are any impacts to cross-carrier scheduling

- For the overlapping CBWs from UE perspective (one cell approach):

  - Is it possible to configure the UE with a dedicated carrierBandwidth in the ServingCellConfig that is wider than/partially outside the carrierBandwidth configured in SIB1?

  - Clarify for equalization purposes in the DL, does the BS need to know the split between the subset of PRBs from a main RF carrier versus PRBs from an additional RF carrier are received on different channel/antenna before combining. If pre-coding assumes all PRBs experience the same channel/antenna, is signalling required so that BS pre-coding can account for the path differences of main carrier PRBs and additional carrier PRBs.

2. Actions:

To RAN1/RAN2:

ACTION: RAN4 respectfully asks RAN1 and RAN2 to answer questions above.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#101-e	Nov 2021	E-meeting

4. References:

[1] 3GPP TR 38.844, v0.0.4