3GPP TSG-RAN WG2 Meeting #116-e                                                                 R2-2109424

Online, Nov 01 – Nov 12, 2021

Source:	CATT, CBN

Title:	Open Issues on MBS interest indication

Agenda Item:	8.1.3.1

Document for:	Discussion and Decision

Introduction

For MBS interest indication, it has been discussed in past meetings and some agreements have been reached in last RAN2 meeting, according to [1],

- For CONNECTED: The UE reports the following MBS interest information (as LTE SC-PTM): - MBS frequency list - priority between the reception of all listed MBMS frequencies and the reception of any unicast bearer - TMGI list If MBS frequencies are allowed to be reported, the MBS frequencies reported by the UE is sorted by decreasing order of interest, as LTE SC-PTM. Send an LS to SA3 to check whether the MBS interest information can be reported by the UE before security activation. FFS whether the MII is reported via UEAssistanceInformation or a new RRC message.

Furthermore, the remaining issues on MBS interest indication also have been discussed in email 091 [2].

In this contribution, we aim at address the remaining issues which are still controversial. More specifically, the following aspects are covered,

- Message for MBS interest indication

- Service interruption during RRC connection establishment

- Frequencies in MBS interest indication

Discussion

Message for MBS interest indication

Regarding which message (i.e. UAI message or new RRC message) is used to carry MBS interest information, it has been discussed in previous meeting and no conclusion yet. Obviously it is flexible to use a new message from triggering and content perspective, and it should not be a big effort to define a new message.

Proposal 1: Define a new RRC message for MBS interest indication.

Service interruption during RRC connection establishment

Upon entering RRC connected mode, gNB may configure an active BWP to UE in MSG4, which is before service activation.

If gNB is not aware of that UE is receiving broadcast session, a UE receiving broadcast may be configured to operate on an active BWP other than CFR (located with initial BWP according to RAN1 agreement) in MSG4. To continue broadcast session in this case, UE is required to monitor both the CFR for broadcast services and active BWP simultaneously. However, it may beyond UE’s capability as UE is not required to operate on more than one BWP simultaneously in legacy NR.

Observation 1: Upon RRC connection establishment, the broadcast reception may be interrupted if gNB configures an active BWP which does not cover the CFR for MBS.

To avoid this, one straight way is gNB always configures an active BWP overlapped with the CFR carrying broadcast session. If the CFR for different broadcast session does not overlap, gNB needs to know which broadcast session UE is receiving before gNB configures the active BWP to UE. It may require UE to send the session id (i.e. TMGI) of its interest service to gNB before security activation during RRC connection establishment.

However, SA3 has clarified that reporting TMGI list should be avoided by the UE before security activation, according to the reply LS from SA3 [3].Hence it is not feasible for UE to send the session id (i.e. TMGI) to gNB before security activation.

There may be other alternative solutions to address this issue, but firstly RAN2 should discuss whether such service interruption issue is worth to address.

Proposal 2: Discuss whether to avoid service interruption caused by BWP configuration during RRC connection establishment.

Frequencies in MBS interest indication

In LTE SC-PTM, UE capability on carrier aggregation is considered when determining whether a frequency is included in MBMS interest indication, according to TS 36.331 [4],

5.8.5.3 Determine MBMS frequencies of interest The UE shall: 1> consider a frequency to be part of the MBMS frequencies of interest if the following conditions are met: …… 2> the UE is capable of simultaneously receiving MRBs and/or is capable of simultaneously receiving SC-MRBs on the set of MBMS frequencies of interest, regardless of whether a serving cell is configured on each of these frequencies or not; and 2> the supportedBandCombination the UE included in UE-EUTRA-Capability contains at least one band combination including the set of MBMS frequencies of interest;

Observation 2: In LTE SC-PTM, frequencies reported by UE depend on the UE capability on carrier aggregation.

Regarding the MSB frequencies reported in MBS interest indication, companies’ understanding is divergent, according to the discussion in [2]. Some companies think it is simultaneous MBS reception capability via multiple frequencies.

In LTE SC-PTM, the MBS frequencies reported in MBS interest indication are used by eNB to ensure simultaneously unicast and MBMS reception. So there is a restriction that the set of MBS frequencies of interest reported by UE should be part of a band combination of the UE, according to TS 36.300 [5].

Except for NB-IoT UEs, in RRC_CONNECTED, the UE that is receiving or interested to receive MBMS via MBSFN or SC-PTM informs the network about its MBMS interest via a RRC message and the network does its best to ensure that the UE is able to receive MBMS and unicast services subject to the UE's capabilities: …. - the E-UTRAN reuses the SupportedBandCombination IE to derive the UEs MBMS related reception capabilities, i.e. the E-UTRAN tries to ensure that the UE is able to receive MBMS and unicast bearers by providing them on the frequencies indicated in SupportedBandCombination IE signalled by the UE. The UE supporting MBMS reception via MBSFN or SC-PTM shall support MBMS reception via MBSFN or SC-PTM respectively, on any serving cell and on any cell that may be additionally configured as serving cell according to the UE capabilities.

When it comes to NR MBS, the LTE SC-PTM mechanism should the baseline.

Proposal 3: MBS frequencies reported in MBS interest indication is used for ensuring unicast reception and broadcast reception in UE simultaneously.

According to the email discussion [2], some companies think that the reported frequencies are also used for handover decision. It seems unnecessary. As TMGI is also included in MBS interest indication, serving gNB can make HO decision (i.e.to determine the target cell) based on TMGI. The assumption is that gNB is aware of which neighbor cell providing what broadcast session identified by TMGI. However, it should be confirmed with RAN3 on this point.

Observation 3: Whether MBS frequencies in MBS interest indication are used for handover decision depends on whether TMGI is sufficient for handover decision, to be confirmed by RAN3.

Therefore, we need to confirm with RAN3 on whether gNB is aware of which neighbor cell/frequencies providing what broadcast session identified by TMGI.

Proposal 4: For MBS frequencies reported in MBS interest indication, discuss whether it is used for handover decision.

Conclusion

In this contribution, we discuss remaining aspects on service continuity of Delivery Mode 2, based on which the observations and proposals are summarized as the following.

Message for MBS interest indication

Proposal 1: Define a new RRC message for MBS interest indication.

Service interruption during RRC connection establishment

Observation 1: Upon RRC connection establishment, the broadcast reception may be interrupted if gNB configures an active BWP which does not cover the CFR for MBS.

Proposal 2: Discuss whether to avoid service interruption caused by BWP configuration during RRC connection establishment.

Frequencies in MBS interest indication

Observation 2: In LTE SC-PTM, frequencies reported by UE depend on the UE capability on carrier aggregation.

Proposal 3: MBS frequencies reported in MBS interest indication is used for ensuring unicast reception and broadcast reception in UE simultaneously.

Observation 3: Whether MBS frequencies in MBS interest indication are used for handover decision depends on whether TMGI is sufficient for handover decision, to be confirmed by RAN3.

Proposal 4: For MBS frequencies reported in MBS interest indication, discuss whether it is used for handover decision.

Reference

- RAN2#115e chairman notes

- [Post115-e][091][MBS] Remaining control plane issues (Huawei)

- R2-2109381, Reply LS for the security issue of MBS interest indication

- 36.331, Radio Resource Control (RRC) Protocol specification, v16.6.0

- 36.300, (E-UTRAN) Overall description, v16.6.0