3GPP TSG-RAN WG2 Meeting #116-e	R2-2109450

Online, 1 – 12 November 2021

Agenda item:	8.12.3.2

Source:	Qualcomm Incorporated

Title:	Remaining issues on RRM relaxations in RRC Connected

WID/SID:	NR_redcap-Core

Document for:	Discussion and Decision

# Introduction

This paper discusses a few remaining issues on RRM relaxations.

# Discussion

In RAN2#114-e, it was agreed that RRM relaxations in RRC Connected should be under full control of network. For example, after UE evaluates RRM relaxation criteria configured by network and determines that it meets the criteria, it still needs to report the evaluation result back to network. It cannot relax its RRM measurements until it is enabled by network.

Since this principle is important to network, we think the same should be applied when UE exists the relaxation, i.e. when UE determines that it no longer meets the relaxation criteria, it should inform network before changing its measurement behaviors. This is especially important in the case where network reconfigures UE’s measurement configuration when enabling the relaxation.

Observation 1. If we follow the principle that relaxations should be under the full control by network, UE should also inform network when it exits relaxations.

To signal this request (to exist relaxation), we think UE Assistance Information (UAI) is a better fit than measurement report. For example, in the case of low mobility criterion is used to evaluation UE’s stationarity, a measurement report of UE’s current RSRP does not fully reflect change in UE’s mobility status. On the other hand, UAI is a more direct and explicit way to inform network of the change. In addition, it is an established RAN2 protocol that if UAI is used to indicate a UE state, it is sent when UE first enters the state, and it should be sent again when UE exists state too (e.g. UAI for overheating). Hence it is natural to use UAI for UE to inform network when it prefers to enter and exist RRM relaxation.

Observation 2. UE Assistance Information is a more appropriate signaling method for UE to inform network its preference to enter or exist a UE state.

Proposal 1.  	UE sends UE Assistance Information to inform network when it meets or no longer meets RRM relaxation criteria.

It is possible that network has not configured RRM relaxation criteria but UE detects that it is stationary by other means (e.g. positioning sensor, etc). In that case, UE may want to relax its RRM measurements to save power. Currently, there is no signaling available for UE to send such a request. We think UE Assistance Information can be leveraged for the purpose. For example, after network receives UE’s request to enable RRM relaxation, network configures relaxation criteria for the UE. UE then evaluates if its mobility status meets network’s criteria and reports back to network.

Proposal 2.  	In RRC Connected, UE can use UE Assistance Information to request configuration of RRM relaxation criteria.

RRM relaxation enhancements studied in R17 focus on stationary UEs. Relaxation criteria such as stationarity and not-at-cell-edge actually are common between both Redcap and non-Redcap UEs and do not depend on reduced capabilities at all. Therefore, it is desirable if all the RRM relaxation enhancements studied in R17 are applicable to Redcap and non-Redcap UEs.

Proposal 3.   	R17 RRM relaxations are applicable to both RedCap and non-RedCap UEs.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1. If we follow the principle that relaxations should be under the full control by network, UE should also inform network when it exits relaxations.

Observation 2. UE Assistance Information is a more appropriate signaling method for UE to inform network its preference to enter or exist a UE state.

Proposal 1.  	UE sends UE Assistance Information to inform network when it meets or no longer meets RRM relaxation criteria.

Proposal 2.  	In RRC Connected, UE can use UE Assistance Information to request configuration of RRM relaxation criteria.

Proposal 3.   	R17 RRM relaxations are applicable to both RedCap and non-RedCap UEs.

# References