3GPP TSG-RAN WG2 Meeting #116-e                                                    R2-2109477

Electronic, 1st - 12th Nov, 2021

Source:              CATT

Title:	             Leftover Issues for Sidelink Unicast DRX

Agenda Item:	8.15.2

Document for:	Discussion and Decision

Introduction

In this contribution, we focus on the key leftover issues without consistent understanding for sidelink unicast DRX. The related issues are listed below:

Regarding to the sidelink DRX configuration:

- Issue 1: When Tx-UE is OOC, whether Tx-UE decides the DRX configuration in signalling-2 (Tx->Rx) with or without relying on pre-configuration?

Regarding to the sidelink DRX timer maintenance:

- Issue 2: Whether the value of SL HARQ RTT timer is required to be configured explicitly?

- Issue 3: How to start the retransmission timer?

Alignment between Uu/SL DRX:

- Issus 4: For Tx UE in RRC CONNECTED, how to align the Uu DRX and SL DRX?

- Issue 5: In case of Mode 1 sheduling, how to align the Uu DRX of Tx UE and SL DRX of Rx UE?

Others:

- Issue 6: How to enhance SL LCP and resource (re)selection procedure to support SL DRX?

- Issue 7: Whether the sensing procedure will be impact by SL DRX?

In this contribution, the above issues will be analyzed one by one, and our preference will be given based on the analysis.

Discussion

Leftover issues for sidelink DRX configuration

Pre-configuation for determing the Tx-centric DRX configuration for OOC UE

According to [1], based on the majority view, it is proposed:

Proposal 4 [11/21] In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, when Tx-UE is OOC, RAN2 discuss whether Tx-UE decides the DRX configuration in signalling-2 (Tx->Rx) with or without relying on pre-configuration. Proposal 6 [16/21] In SL unicast, for DRX configuration of the direction where one UE as Tx-UE and the other as Rx-UE, when Tx-UE is in-coverage and in RRC_IDLE/RRC_INACTIVE state, Tx-UE obtain DRX configuration from SIB to generate signalling-2 (Tx->Rx).

Based on the above proposal 4, it is still unclear whether the pre-configuration is needed for Tx UE in OOC to determine the Tx-centric DRX configuration. There are mainly three options according to the comments in email discussion:

- Option 1: Pre-configuration is not considered;

- Option 2: Pre-configuration needs to be considered, and the pre-configuration refers to the resource pool configuration only;

- Option 3: Pre-configuration similar as sidelink groupcast/broadcast can be considered, e.g., a group of DRX configurations can be pre-configured, each one is corresponding to one PQI/PQI list.

For Option 2, we agree that the pre-configured Tx resource pool should be considered when determining the sidelink DRX configuration. But Tx resource pool configuration is already existed in the legacy pre-configuration. We think what we should discuss is whether new pre-configuration information specific for SL DRX configuration determination is needed or not.

Option 3 gives the new SL DRX specific pre-configuration, which is similar as the pre-configuration as sidelink groupcast/broadcast. For sidelink groupcast/broadcast, the main motivation of introducing the pre-configured DRX configuration is to save the power consumption of Rx UE because the Rx UE can only monitor the sidelink in sidelink active time and the sidelink active time can be determined based on its interested broadcast/groupcast service. But for sidelink unicast, since there is interaction between Tx and Rx UE, Rx UE can perform the sidelink monitoring based on the DRX configuration received from Tx UE. Hence for sidelink unicast, there is no strong motivation to introduce new pre-configuration for SL DRX determination. But according to the Proposal 6 of [1], 16 of 24 companies already agreed to consider the SIB configuration for IDLE/INACTIVE Tx UE to determine the Tx-centric DRX configuration. Hence, in order to make similar solutions for IDLE/INACTIVE UE and OOC UE, a compromise can be made, that is to introduce the pre-configuration for OOC UE.

Proposal 1: If Tx UE is OOC, pre-configuration information which contains the suggested DRX configuration for each PQI/PQI list can be taken into account.

Sidelink DRX timer maintainence

Regarding to the sidelink DRX timer maintenance, many agreements were reached in RAN2#115-e, but there are still some open issues, which are listed below:

- Issue 1: Whether the value of SL HARQ RTT timer is required to be configured explicitly?

- Issue 2: How to start the retransmission timer?

In the following, the above issues will be further discussed one by one.

Implicit or explicit  SL HARQ RTT timer length?

In the RAN2#113bis-e meeting, it was agreed that:

19:	Working assumption: SL HARQ RTT timer can be derived from the retransmission resource timing when the SCI indicates a retransmission resource. FFS whether explicitly configured SL HARQ RTT timer may be still required. If big problem is identified next meeting, we can revisit it.

20:	The value(s) of the SL HARQ RTT Timer, when explicitly configured and not determined via SCI (if agreed to do so), is determined by UE or NW implementation.

In [4], some exceptional cases have been identified by RAN1:

Q1): For R17 SL DRX design, from RAN1 perspective, whether it is feasible for the Rx-UE to determine the time location of the next retransmission resource(s) of the TX UE (assuming that resource is not reselected by the TX UE) based on the “Time resource assignment” field in SCI? Answer: Regarding RAN2’s question, in RAN1’s opinion it is feasible, other than in the following exceptional cases: - SL transmission dropping due to prioritization or congestion control - Due to re-evaluation, a re-selected resource is earlier than a reserved resource by UE implementation in Mode 2 - If (pre-)configured with many-to-one mapping between Tx and Rx resource pools in some cases (e.g., when PSFCH is not configured)

According to the reply LS from RAN1, if sidelink RTT timer based on SCI, Rx UE may miss retransmission packets in above cases RAN1 mentioned, it is not feasible for the Rx UE to determine sidelink RTT timer based on SCI.

Proposal 2: For sidelink unicast, an explicit sidelink HARQ RTT timer length should be configured to Rx UE, not determined based on the SCI.

Start of the Retransmission timer

In NR Uu, retransmission timer is started when HARQ RTT timer expired, RAN2 was agreed that HARQ RTT is supported for both HARQ enabled and HARQ disabled cases by allowing HARQ RTT timer to be set to different values. The Uu scheme for start retransmission timer can be reuse for SL retransmission timer.

9:	HARQ RTT is supported for both HARQ enabled and HARQ disabled cases by allowing HARQ RTT timer to be set to different values.  FFS on the specific values that can be used for HARQ disabled case.

Proposal 3: For sidelink unicast, retransmission timer can be started after the HARQ RTT timer expiry.

Uu/SL DRX alignment

In RAN2#114-e meeting, the following agreements were made:

Agreements on alignment between Uu DRX and SL DRX 1: Alignment of Uu DRX and SL DRX for UE may comprise the full overlapping between Uu DRX and SL DRX in time. 2: Alignment of Uu DRX and SL DRX for UE may comprise the partial overlapping between Uu DRX and SL DRX in time. 3: For at least SL RX-UEs in RRC CONNECTED, the alignment of Uu DRX and SL DRX is up to gNB. FFS for SL TX-UE. 4: RAN2 to down-scope alignment of Uu DRX and SL DRX for UEs in RRC IDLE and RRC INACTIVE from Rel-17. 5: In case of Mode 1 scheduling, the alignment of Uu DRX of Tx UE and SL DRX of Rx UE shall be considered. FFS on how alignment is achieved.

Two issues should be further discussed:

- Issus1: For Tx UE in RRC CONNECTED, how to align the Uu DRX and SL DRX?

- Issue2: In case of Mode 1 sheduling, how to align the Uu DRX of Tx UE and SL DRX of Rx UE?

In the following, the above issues will be further discussed one by one.

Alignment of Uu DRX and SL DRX for Tx UE in RRC CONNECTED

In RAN2#114-e meeting, according to the DRX configuration, following agreements were made:

Agreements on TX-UE centric or RX-UE centric DRX configuration determination

1: 	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, support signalling exchange including both 1) Signaling-1: signalling from RX-UE to TX-UE, and 2) Signaling-2: signalling from TX-UE to RX-UE.

2:	For SL unicast, TX-UE centric DRX configuration based on the assistance information from RX-UE is agreed as baseline.

2a: 	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, signaling-1 (Rx->Tx) is carried via a new PC5-RRC message, from Rx-UE to Tx-UE.

2b:	In SL unicast, for DRX configuration of the direction where one UE as Tx-UE and the other as Rx-UE, signaling-2 (Tx->Rx) is carried via RRCReconfigurationSidelink, to deliver DRX configuration from Tx-UE to Rx-UE.

3:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may report the information received in signaling-1 (Rx->Tx) to the serving network.

4:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may obtain DRX configuration from dedicated RRC to generate signalling-2 (Tx->Rx).

5:	In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Rx-UE is in-coverage and in RRC_CONNECTED state, Rx-UE report the DRX configuration received in signalling-2 (Tx->Rx) to the serving network.

2 signals are defined, which are:

Signal-1(Rx->Tx), which is a new PC5-RRC message and,

Signal-2(Tx-> Rx) carried via RRC ReconfigurationSidelink.

And Tx UE centric DRX configuration was agreed as baseline. It is natually that Tx UE’s serving gNB’s is responsible for allignment of Uu DRX and SL DRX for Tx UE. The reference procedure may be as follows:

Step1: Rx UE may send assistance information to Tx UE via signaling-1;

Step2: Tx UE sends the assistance information to its serving gNB;

Step3: Tx UE’s serving gNB makes the allignment of Uu DRX and SL DRX and sends SL DRX configuration to Tx UE;

Step4: Tx UE sends the SL DRX configuration to Rx UE via signaling-2.

Proposal 4: For Tx UE in RRC CONNECTED, how to align the Uu and SL DRX depends on its serving gNB’s implementation.

SL LCP enhancement for SL DRX

In Rel-16, Tx UE selects the destination associated to the MAC CE and the logical channel with the highest priority.

For unicast, the DRX is configured for each pair of src/dest L2 ID. For groupcast, the DRX is suggested to be configured for each PQI/PQI list. Hence, Tx UE may select a destination which is not in active time if reusing Rel-16 LCP procedure. To avoid SL transmission failure due to Rx UE is not in active time, LCP enhancements for destination selection was agreed:

Proposal 33 – [19/21] RAN2 assumes LCP enhancements for ensuring a TX UE transmits data in the active time of an RX UE are needed. FFS on the resource (re)selection enhancements (e.g. limiting the resources to the active time for peer UE).

When performing destination selection during LCP procedure, the Tx UE shall only select the LCH and/or MAC CE which corresponding src/des L2 ID (for sidelink unicast) or corresponding PQI(for sidelink broadcast/groupcast) is in SL DRX active time.

Proposal 5: When performing destination selection, UE should only select the logical channel with highest priority and/or the MAC CE corresponding destination/PQI which is in active time.

In addition, for the case that a Tx UE is in DRX active time of a destination (for sidelink unicast) /PQI (for sidelink broadcast/groupcast), and there is SL data to the destination/PQI available for transmission, but there isn’t SL grant can be used within DRX active time for the destination/PQI, resource (re)selection (mode 2) or SR/BSR (mode 1) needs to be triggered. And the selected grant shall match the DRX configuration of the destination/PQI.

Proposal 6:For mode 2 Tx UE, resource (re)selection needs to be triggered when there isn’t SL grant can be used in DRX active time for the destination (for sidelink unicast)/PQI(for sidelink broadcast/groupcast) which has SL data available for transmission. And the selected grant shall match the DRX configuration of the destination/PQI.

Proposal 7: For mode 1 Tx UE, SL SR/BSR needs to be triggered when there isn’t SL grant can be used in DRX active time for the destination/PQI which has SL data available for transmission.

Whether the sensing procedure is impacted by SL DRX?

In [3], RAN1 informed that UE can perform sensing in SL DRX inactive time.

Agreement A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS: When such reception and measurement is performed, whether it is subject to specification, or is up to UE implementation - FFS: Other details

From RAN1’s agreement, UE can perform sensing whenever SL DRX is in active time or inactive time.  This means that RAN2 keeps working on completing sidelink DRX design without considering sensing. Therefore, sensing procedure is independent of SL DRX.

Proposal 8: Sensing procedure is independent of SL DRX.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: If Tx UE is OOC, pre-configuration information which contains the suggested DRX configuration for each PQI/PQI list can be taken into account.

Proposal 2: For sidelink unicast, an explicit sidelink HARQ RTT timer length should be configured to Rx UE, not determined based on the SCI.

Proposal 3: For sidelink unicast, retransmission timer can be started after the HARQ RTT timer expiry.

Proposal 4: For Tx UE in RRC CONNECTED, how to align the Uu and SL DRX depends on its serving gNB’s implementation.

Proposal 5: When performing destination selection, UE should only select the logical channel with highest priority and/or the MAC CE corresponding destination/PQI which is in active time.

Proposal 6:For mode 2 Tx UE, resource (re)selection needs to be triggered when there isn’t SL grant can be used in DRX active time for the destination (for sidelink unicast)/PQI(for sidelink broadcast/groupcast) which has SL data available for transmission. And the selected grant shall match the DRX configuration of the destination/PQI.

Proposal 7: For mode 1 Tx UE, SL SR/BSR needs to be triggered when there isn’t SL grant can be used in DRX active time for the destination/PQI which has SL data available for transmission.

Proposal 8: Sensing procedure is independent of SL DRX.

Reference

- R2-2104841 Summary of [POST113-e][704][V2X/SL] TX UE centric or RX UE centric DRX configuration determination  OPPO  discussion	Rel-17	NR_SL_enh-Core

- Summary of [POST114-e][706][V2X/SL] Discussion on remaining FFSs/open issues in SL DRX timer maintenance  InterDigital

- R1-2108580 Reply LS on SL DRX design

- R1-2108622  Reply LS on time gap information in SCI