3GPP TSG-RAN WG2 Meeting #116-e                             	R2-2109517

Online, Nov 01 – Nov 12 2021

Agenda item:	8.1.2.2

Source:	Samsung

Title:	Power Saving and Scheduling Aspects for MBS

Document for:	Discussion & Decision

# Introduction

In RAN2#115-e meeting [1], following agreements were made related to DRX operation for MBS services

- For multicast PTM transmission, Multicast DRX pattern is configured on a per G-RNTI basis (i.e. independent of legacy UE-specific DRX for unicast transmission). - Legacy UE-specific DRX pattern for unicast is reused for PTP transmission of NR MBS, which means the UE specific DRX pattern are for both unicast services and the MBS PTP bearer of UE - Multicast long DRX support is baseline for PTM. FFS whether to support optional short DRX or not. - The Multicast Long DRX operation has to support the following parameters which are similar to the UE-specific DRX for unicast, where the last two parameters are needed if the HARQ- feedback is enabled: - - drx-onDurationTimerPTM - - drx-InactivityTimerPTM - - drx-LongCycleStartOffsetPTM - - drx-SlotOffsetPTM - - drx-HARQ-RTT-TimerDLPTM - - drx-RetransmissionTimerDLPTM - For NR Broadcast, the DRX pattern is configured per G-RNTI. - For NR Broadcast, DRX configuration includes: drx-onDurationTimerPTM, drx-SlotOffsetPTM, drx-InactivityTimerPTM, drx-CycleStartOffsetPTM.

Paper discusses further, the issues related to power saving and scheduling aspects for MBS.

# Discussion

## DRX for PTM (Delivery Mode 2)

As agreed in RAN2, for NR broadcast (DM2), each MBS session can have a service specific DRX configuration for reception with PTM mode and is associated with a specific G-RNTI. However, there are some important aspects that need to be further considered.

There may be a large number of broadcast services (e.g. LTE SC-PTM has max 1024 services). It is reasonable to consider that NR MBS should possibly have same number of broadcast services. Notably, SCPTMConfiguration message over SC-MCCH in LTE SC-PTM is not bounded in size (size could be roughly about 13K bytes with 1024 services). However, there are “new constraints” for RRC message over MCCH (e.g. MBSBroadcastConfiguration) in NR MBS, captured as below observations:

Observation 1: PDCP layer is applicable for NR MBS stack in DM2, and message over MCCH is now subject to maximum PDCP SDU size limitation (9K bytes). This puts a bound on message size over MCCH for NR MBS. That is, RRC message over MCCH having maximum MBS services with all configuration parameters and DRX scheduling timers and configurations must abide by the PDCP bound for NR MBS.

Observation 2: It is quite likely that multiple MBS services may have same DRX configurations given the similar latency requirements for multiple broadcast services.

Observation 3: MCCH transmission time is constrained from beam sweeping in time domain and MCCH is restricted to initial MBS BWP or lower MBS BWP/CFR as configured. This leads to extension in MCCH transmission time (M beams x N MCCH segments) and resultant power consumption for UE.

It would be efficient from power saving as well as signaling perspective to group multiple services which share same DRX configuration (unlike LTE SC-PTM which carries separate and independent signaling for DRX configuration for each service leading to a large RRC message over SC-MCCH). Effectively, multiple MBS PTM services can share a common DRX configuration for DM2 and it can be flexibly configured by the network. In practice, MBSBroadcastConfiguration over MCCH can comprise of a list of DRX configurations, and a list of MBS services configurations each of which is mapped to the list of DRX configurations with a DRX configuration index. Our companion contribution [2] provides message structure for DRX configuration.

Proposal 1: For delivery mode 2, multiple MBS PTM services can share a common DRX configuration.

Proposal 2: For delivery mode 2, multiple MBS PTM services that share a common DRX configuration, are grouped and signalled together in MCCH for DRX configuration (e.g. with a common DRX configuration index).

## DRX for PTM (Delivery Mode 1)

### Retransmission path for PTM initial transmission

Further, RAN2 should discuss and conclude as to how PTP or PTM based retransmission for the PTM initial transmission is informed to or interpreted by the UE. Options can include:

- Option 1: Network explicitly configures the UE for the PTP or PTM based retransmission for PTM initial transmission (e.g. HARQ-ReTxPath field in the MBS bearer configuration is set to either PTP or PTM)

- Option 2: UE expects to receive either the PTP or PTM based retransmission for PTM initial transmission. That is, there is no explicit configuration given to UE.

In general, option 1 seems to be more efficient from power consumption and processing perspective as compared to option 2.

Proposal 3: RAN2 to discuss and select one among the following options for determining retransmission path for PTM initial transmission:

- Option 1: Network explicitly configures the UE for the PTP or PTM based retransmission for PTM initial transmission (e.g. HARQ-ReTxPath field in the MBS bearer configuration is set to either PTP or PTM).

- Option 2: UE expects to receive either the PTP or PTM based retransmission for PTM initial transmission. That is, there is no explicit configuration given to UE.

### PTP retransmission for PTM initial transmission

Though PTM and PTP have different DRX configurations i.e. service specific and UE specific DRX respectively, they may be linked in some cases for HARQ retransmissions. That is, PTM provides initial transmission and PTP carries HARQ retransmissions addressed by C-RNTI. This implies that PTP should be in active time to receive HARQ retransmissions.

Observation 4: When PTM HARQ retransmissions are to be received on PTP path, PTP DRX should be in active time to receive HARQ retransmissions.

Three options were discussed in email discussion [2].

- Option 1: the UE monitors UE specific PDCCH/C-RNTI when either drx-onDurationTimerPTM or drx-InactivityTimerPTM or drx-RetransmissionTimerDLPTM are running.

- Option 2: the UE monitors UE specific PDCCH/C-RNTI only when drx-RetransmissionTimerDLPTM is running. For example, when drx-onDurationTimerPTM and drx-InactivityTimerPTM are running but drx-RetransmissionTimerDLPTM is not running, the UE does not monitor UE specific PDCCH/C-RNTI.

- Option 3: the UE monitors UE specific PDCCH/C-RNTI only during unicast DRX’s active time. Unicast DRX’s RTT timer can be started when PTP retransmission is to be received.

Figure 1 and 2 depict respectively option 2 and option 3.

Figure 1: Illustration for Option 2

Figure 2: Illustration for Option 3

Table 1: Comparison of DRX solutions for PTP ReTx for PTM initial Tx

| Solution → Factors ↓ | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- |
| Power consumption | High | Less | Less |
| Specification effort | High | High | Low (follows legacy unicast DRX largely) |
| Effectiveness | Lacks for multiple PTP retransmission case | Lacks for multiple PTP retransmission case | Addresses multiple PTP retransmissions (as in legacy unicast DRX protocol) |
| Protocol impact & complexity | High (new behavior) | High (new behavior) | Low (adhere to legacy unicast DRX) |

As highlighted, option 1 and option 2 may have issue when multiple PTP retransmissions (i.e. 2nd retransmission or further) are to be received. When first PTP retransmission is received addressed by C-RNTI, PTM timers may not be operated and it is unclear how further PTP retransmissions can be received. In case PTM timers are operated upon PTP retransmission reception, it is expected to cause a new and complex specification impact, which is undesired.

On contrary, as in option 3, when Unicast DRX’s RTT timer is started when PTP retransmission is to be received, the legacy unicast DRX protocol takes care for further PTP retransmissions reception.

Based on the analysis, it seems rational to maintain the separate DRX configurations for PTM and PTP, and only bring PTP DRX to active time whenever it is required to receive HARQ retransmissions. More specifically, drx-HARQ-RTT-TimerDL timer is started for PTP when MBS initial transmission is received for PTM and HARQ retransmission is to be received through PTP.

Proposal 4: UE monitors UE specific PDCCH/C-RNTI only during unicast DRX’s active time. drx-HARQ-RTT-TimerDL timer is started for PTP when MBS initial transmission is received for PTM and HARQ retransmission is to be received through PTP.

### PTM retransmission for PTM initial transmission

Broadly following HARQ feedback scenarios are possible for MBS reception

- ACK/NACK HARQ feedback

- NACK only HARQ feedback

- HARQ feedback is not configured

- HARQ feedback is disabled

It is to be noted that in all cases, i.e. including cases c) and d), UE can receive HARQ retransmission irrespective of HARQ feedback or no HARQ feedback. RAN1#106bis agreements below [4] indicate that HARQ feedback is enabled/disabled per G-RNTI by UE specific configuration and therefore, UE not providing feedback can still avail HARQ retransmissions meant for other UEs (e.g. this is quite useful for an outlier UE or a cell-edge UE to enhance reliability of MBS reception).

- The group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

- If the group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback is not configured, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured per G-RNTI by UE RRC signalling.

With regard to above understanding, in order to track the retransmissions timing, UE needs to operate drx-HARQ-RTT-TimerDLPTM and drx-RetransmissionTimerDLPTM irrespective of HARQ feedback or no HARQ feedback. Note that drx-HARQ-RTT-TimerDLPTM is started in accordance with the timing where HARQ feedback would have been transmitted, even in feedback disabled case. On expiry of drx-HARQ-RTT-TimerDLPTM and unsuccessful data decoding, drx-RetransmissionTimerDLPTM is started. Therefore, RAN2 should confirm this over previous DRX agreements.

Proposal 5: RAN2 to confirm that drx-HARQ-RTT-TimerDLPTM and drx-RetransmissionTimerDLPTM are always configured for UE irrespective of whether HARQ feedback is enabled or disabled.

It is to be noted that ACK-NACK HARQ feedback may be configured with UE specific PUCCH resources (different timings for different UEs) while NACK-only HARQ feedback may use common PUCCH resources (same timing for different UEs). Whereas, when HARQ feedback is not configured or HARQ feedback is disabled, there is no actual feedback transmission event to base the timing for the drx-HARQ-RTT-TimerDLPTM operation.

However, it seems feasible and reasonable to have a common and uniform DRX mechanism for the UE while catering to different configurations and feedback modes as specified and this can be based on a common drx-HARQ-RTT-TimerDLPTM timer configured to the UE as

- UE starts drx-HARQ-RTT-TimerDLPTM timer at the end of GC-PDCCH reception

Proposal 6: UE starts drx-HARQ-RTT-TimerDLPTM timer at the end of GC-PDCCH reception for all HARQ feedback scenarios as

- ACK/NACK HARQ feedback

- NACK only HARQ feedback

- HARQ feedback is not configured

- HARQ feedback is disabled

## WUS for MBS Multicast

WUS (Wake-Up Signal) is introduced in Release 16 for Connected mode UE power saving. It is worthwhile to consider whether WUS approach which has been defined for unicast is also applicable to MBS multicast, particularly to PTP reception path which resembles unicast, and also follow unicast DRX configuration. In either of the two cases, i.e. WUS is applied to PTP reception or not, there is a need for clarity for UE behavior in the specification.

- Option 1: WUS is not applied to PTP - UE needs to be prepared to receive on PTP reception path, regardless of WUS indication for the next DRX cycle.

- Option 2: WUS is applied to PTP - UE should skip receiving MBS on PTP reception path, when WUS indicates no allocation for the next DRX cycle. However, MBS specific measurements and/or CSI reporting may be operated as per DRX-On-duration timer durations.

In general, approach (a) appears to be complicated with maintaining different handling for PTP reception and unicast reception. Approach (b) is aligned with unicast and may need to additionally specify MBS specific measurements and/or CSI reporting operations during drx-on-DurationTimer duration. Notably, these aspects need to be addressed by RAN1. From RAN2 perspective, approach (b) seems reasonable and further details should be worked out by RAN1, if any.

Proposal 7: RAN2 to confirm that WUS is applicable for MBS multicast for PTP reception path. RAN2 should request RAN1 to consider working out further details needed, if any.

With regard to WUS for PTM, it is expected that it will need more specification effort and may not be suitable in the Rel-17 WI scope. RAN2 should confirm not to address this in present release.

Proposal 8: RAN2 to confirm that WUS is not applicable for MBS multicast for PTM reception path in Rel-17.

## BWP Scheduling Aspect in Connected Mode

UE’s interested MBS service may be ongoing on non-active BWP.

- Option 1: Active BWP shall be switched for MBS service

- Option 2: UE is provided MBS service in PTP mode on active BWP

- Option 3: MBS-active BWP and Unicast-active BWP can be different or not receivable together. A mechanism to resolve this issue is necessary, e.g., reporting this mismatch, autonomous BWP switch for MBS, TDM-like switch, etc.

- Option 4: MBS capable UE has two RF chains per cell. UE can receive both MBS BWP and active unicast BWP.

Considering the complexity from RAN2 perspective, option 2 looks feasible. Option 1 may work if there is no active unicast service on active BWP; otherwise active BWP need to be switched together for unicast and multicast by network.

Proposal 9: RAN2 to further discuss the alternatives to receive UE’s interested MBS service ongoing on non-active BWP.

Besides, in order to support multicast PTM reception for low capability UEs i.e. UEs not having spare receiver, RAN2 should also address service distribution aspects with respect to BWP as follows:

- Configure all these UEs with same BWP (concentration)

- Support distribution of these UEs across BWPs implying MBS services are duplicated on different BWPs (distribution)

- TDM approach i.e. during slot, involved UEs can be distributed across different BWPs (switching)

Proposal 10: RAN2 to further discuss the different MBS service distribution aspects with regard to BWP in order to support low capability UEs receiving MBS services.

## BWP Switching

Consider that MBS service is being received on MBS BWP or frequency resource associated with unicast BWP. UE currently receiving its interested MBS services should not switch to other unicast BWP by e.g. Network’s BWP switching indication or bwp-InactivityTimer expiry, in order to support MBS service reception.

- Network’s BWP switching: Network shall not indicate BWP switching to UE receiving MBS service. For multicast MBS service received by UE in RRC_CONNECTED state, gNB is aware of it. Notably, for broadcast services in RRC_IDLE/RRC_INACTIVE or RRC_CONNECTED state, MBS service would be provided in Initial BWP and moreover, it is provided in best effort manner and therefore, any guaranteed mechanism is not needed.

- bwp-InactivityTimer expiry: UE is not needed to switch to default BWP or Initial BWP when UE is receiving it’s interested MBS service.

As it is purely a MAC issue and RAN2 should address this in a manner which has minimum spec impact. It is, therefore, proposed that:

Proposal 11: UE is not allowed to switch BWP at bwp-InactivityTimer expiry, if it is receiving interested MBS service.

# Conclusion

We request RAN2 to discuss and possibly agree to the observations and proposals made as:

Power Saving Aspects:

Proposal 1: For delivery mode 2, multiple MBS PTM services can share a common DRX configuration.

Proposal 2: For delivery mode 2, multiple MBS PTM services that share a common DRX configuration, are grouped and signalled together in MCCH for DRX configuration (e.g. with a common DRX configuration index).

Proposal 3: RAN2 to discuss and select one among the following options for determining retransmission path for PTM initial transmission:

- Option 1: Network explicitly configures the UE for the PTP or PTM based retransmission for PTM initial transmission (e.g. HARQ-ReTxPath field in the MBS bearer configuration is set to either PTP or PTM).

- Option 2: UE expects to receive either the PTP or PTM based retransmission for PTM initial transmission. That is, there is no explicit configuration given to UE.

Proposal 4: UE monitors UE specific PDCCH/C-RNTI only during unicast DRX’s active time. drx-HARQ-RTT-TimerDL timer is started for PTP when MBS initial transmission is received for PTM and HARQ retransmission is to be received through PTP.

Proposal 5: RAN2 to confirm that drx-HARQ-RTT-TimerDLPTM and drx-RetransmissionTimerDLPTM are always configured for UE irrespective of whether HARQ feedback is enabled or disabled.

Proposal 6: UE starts drx-HARQ-RTT-TimerDLPTM timer at the end of GC-PDCCH reception for all HARQ feedback scenarios as

- ACK/NACK HARQ feedback

- NACK only HARQ feedback

- HARQ feedback is not configured

- HARQ feedback is disabled

Proposal 7: RAN2 to confirm that WUS is applicable for MBS multicast for PTP reception path. RAN2 should request RAN1 to consider working out further details needed, if any.

Proposal 8: RAN2 to confirm that WUS is not applicable for MBS multicast for PTM reception path in Rel-17.

BWP Scheduling Aspects:

Proposal 9: RAN2 to further discuss the alternatives to receive UE’s interested MBS service ongoing on non-active BWP.

Proposal 10: RAN2 to further discuss the different MBS service distribution aspects with regard to BWP in order to support low capability UEs receiving MBS services.

Proposal 11: UE is not allowed to switch BWP at bwp-InactivityTimer expiry, if it is receiving interested MBS service.

# References

[1] Draft_R2-115e_Meeting_Report_v1

[2] R2-2109538 MCCH Contents and RRC Aspects (Samsung)

[3] [Post115-e][092][MBS] Remaining User plane issues (Lenovo)

[4] Chair notes RAN1#106bis-e meeting