3GPP TSG-RAN WG2 Meeting #116-e                                                                 R2-2109560

Online, Nov 01 – Nov 12, 2021

Source:	CATT

Title:	Open Issues on UE Onboarding and Provisioning for NPN

Agenda Item:	8.16.3

Document for:	Discussion and Decision

Introduction

At RAN2#115-e meeting, the objective “Support UE onboarding and provisioning for NPN” was discussed and some agreement has been reached [1] as the following:

RAN2 didn’t identify a need for modification to access control for SNPN access for onboarding (could be discussed in other groups)

In this contribution, we discuss remaining issues on this objective.More specifically,the following aspects are covered,

- Impact to suitable cell criteria

- UE AS reads SIB containing GIN when requested by NAS

Discussion

Impact to suitable cell criteria

In reply to RAN2 LS according to [1], SA confirmed that onboarding can be enabled in only part of the SNPN network.

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?) [SA2 answer] The ”onboardingEnabled” bit can be set/enabled per cell e.g. when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs. The parameter is used to assist the UE in network selection.

According to above SA2 answer, a specific cell of a onboarding SNPN may set the ”onboardingEnabled” bit to TRUE or FALSE. Hence, even a SNPN supporting onboarding has been selected by UE, UE performing onboarding should again check whether the candidate cell supports onboarding or not before choosing it as the target cell during cell selection/reselection.

Therefore the legacy SNPN suitable cell criteria need to be modified to reflect this new requirement.

Proposal 1: Suitable cell criteria are modified to support SNPN onboarding.

UE AS read SIB containing GIN when requested by NAS

For the purpose of onboarding and separate credential, it is agreed that GIN is broadcasted in a new SIB and the GIN will be acquired by UE AS and send to UE NAS during SNPN selection.

GIN is broadcasted by new SIB

However, GIN is only need by NAS for some cases, i.e. GIN is not necessary for all the SNPN selection cases.

GIN is needed only when the pre-configured ON-SNPN selection information GINs is available according to TS 23.501 [3].

For automatic or manual selection, the UE may select and attempt to register to an ON-SNPN which broadcast the Onboarding enabled indication described in clause 5.30.2.10.2.3 and matches the pre-configured ON-SNPN selection information such as SNPN network identifier and/or GIN(s) (if available) described in clause 5.30.2.10.2.4 according to the UE implementation-specific logic.

Since GIN is broadcasted in a new SIB, UE need more time to perform the available SNPN search as normally UE only need to read the MIB and SIB1 for the found cell. Obviously it will increase the latency of the SNPN selection procedure and UE power consumption is necessarily increased. Hence it is reasonable for UE AS to acquire the new SIB containing GIN only when it is required from NAS perspective.

Proposal 2: For onboarding SNPN selection, UE AS should only read the new SIB containing GINs when it is requested by NAS.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: Suitable cell criteria are modified to support SNPN onboarding.

Proposal 2: For onboarding SNPN selection, UE AS should only read the new SIB containing GINs when it is requested by NAS.

Reference

[1] R2-115-e Chairman notes

[2] TR 23.700-07, Study on enhanced support of non-public networks,v17.0.0

[3]TS 23.501, System architecture for the 5G System (5GS),v17.2.0