3GPP TSG RAN WG2#116	R2-2109569

e-Meeting, Nov., 2021

Title:	(Reply) LS on CSI report of PUCCH SCell

Response to:	R4-2115339

Release:	Release 17

Work Item:	NR_RRM_enh2-Core

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact Person:

Name:	Zhongda Du

E-mail Address:	duzhongda@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

On CSI Report of PUCCH SCell when it is being activated, here are the answers to RAN4’s questions:

Q1: Whether UE can report CSI (e.g. L1-RSRP) of the target being-activated PUCCH SCell belonging to secondary PUCCH group by configuring CSI report setting (e.g. CSI-ReportConfig) on any active serving cells belonging to primary PUCCH group

Answer1: No.

Q2: Whether the above observation is correct, i.e. the identified four cases are not supported by the current RAN1 and RAN2 specification

Answer2: Current RAN2 spec doesn’t support.

Q3: Whether the above identified cases can be supported by RAN1 and RAN2 spec updates within Rel-17 timeframe

Answer3: Yes. RAN2 agreed that UE can trigger to report BFR MAC CE in primary PUCCH group to indicate the right beam which can be finished in Rel17 timeframe.

2. Actions:

To RAN4:

RAN2 respectfully asks RAN4 to take RAN2’s response into account.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#117	21 Feb.2022~3 March 2022	Electronic Meeting