3GPP TSG-RAN WG2 Meeting #116-e 	R2-2109577

Electronic, 1st - 12th Nov, 2021

Agenda item:	8.12.2.2

Source:	Huawei, HiSilicon

Title:	Identification and access restriction of RedCap UE

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #115-e, there are some discussions about Redcap UE identification and access restriction, and some agreements are given as follows [1]:

- Msg1 identification which can be configured to be enabled/disabled can be specified from RAN2 point of view. - Solution for early identification for 2-step RACH will be specified. - Specify separate indications in SIB1 for barring RedCap UEs with 1 Rx chain and 2 Rx chains. - Specify a RedCap specific IFRI in SIB1. - IFRI for RedCap UEs in SIB1 is common for UEs with 1 Rx or 2 Rx branches. - If RedCap-specific IFRI is absent from broadcast SI, the UE considers the cell does not support RedCap. - A Msg3 early identification based on dedicated LCID is supported (if SA3 confirms there is no problem) - RedCap UE applies the existing cellBarred field in MIB

In this paper, we will further discuss the issues on identification and access restriction for RedCap UEs.

- Discussion

  - Msg1 early identification

RAN1 meeting #106-e： Agreement Confirm the following working assumption with the modifications in red: - For 4-step RACH, support the early indication of RedCap UEs at least in Msg1. - The early indication in Msg1 can be configured to be enabled/disabled via SIB - From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported) - separate PRACH resource - PRACH preamble partitioning Whether/how to support early indication of RedCap UEs in Msg3 in Rel-17 is up to RAN2.

In RAN1 meeting #106-e, the agreement about early identification for RedCap UEs are achieved as shown in the above table. As pointed in the agreements, early indication in Msg1 for CBRA is supported and can be enabled/disabled via SIB. Thus, from the RAN2 side, we can confirm that RedCap UE use Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured by NW. This is because in any CBRA case, the Msg2 scheduling requires the awareness of RedCap UE by NW in Msg1,

Proposal 1: In MAC perspective, RedCap UE use Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured by NW.

RAN1 also agreed how to implement early indication both for shared initial UL BWP and separate initial UL BWP (if supported), i.e. separate PRACH resource and PRACH preamble partitioning. Therefore, RAN2 confirm that both dedicated ROs and dedicated PRACH preamble should be supported for Msg1 early identification.

Proposal 2: For Msg1 early identification, RAN2 confirm both dedicated ROs and dedicated PRACH preamble should be supported.

As stated in the aforementioned RAN1 agreements, the early indication in Msg1 can be configured to be enabled/disabled via SIB. A general method is to use the dedicate RACH configuration for Msg1 early identification to implicitly indicate whether the early indication in Msg1 is configured. When the dedicate RACH configuration for Msg1 early identification is present, it means the Msg1 early identification is enabled; otherwise, it means the Msg1 early identification is disabled.

Proposal 3: Msg1 early identification is enabled/disabled implicitly by the presence of dedicate RACH configuration for Msg1 early identification.

  - Msg3 early identification

In last meeting, RAN2 has agreed that Msg3 early identification based on dedicated LCID is supported. However, two logical channels are used to carry Msg3, i.e. CCCH and CCCH1. As for CCCH can be used for transferring RRCSetupRequest, it is apparently that a new LCID should be introduced for Msg3 early identification, since the Network has no stored information for the UE. While for CCCH1, it is only used for RRCResumeRequest1, normally, when the gNB recieves the RRCResumeRequest1, it can fetch the UE capability from the stored UE context and recognize the UE is RedCap UE. But in some cases, the RRC resume procedure may fail and fallback to RRC connection establishment procedure,  the target gNB is not able to know/fetch the UE context by its Msg3, and can not identify it is RedCap UE by its I-RNTI. Therefore, it is also helpfu to suppor thte Msg3 early identficiation for the RRC resume procedure with CCCH1.

Proposal 4: Two reserved LCIDs are used for CCCH and CCCH1 cases respectively for Msg3 early identification.

When RedCap UE access to Network, it should always use the dedicated LCID for Msg3 early identification. Such a method can make UE have easy implementation regardless of whether Msg1 early identification is configured, and it does not introduce extra signaling overhead or any complexity.

Proposal 5: In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, whenever the Msg3 includes the CCCH data.

  - MsgA early identification

From RAN 2 side, the Msg 3 early identification method can be reused by MsgA early identification. Compared to the solution for Msg1 early identification, it does not depend on further dedicated PRACH resource for MsgA early identification, which saves the resource of Network. As for Msg1 based early identification method whether can be configured for MsgA early identification is up to RAN1.

Proposal 6: At least the dedicated LCID (i.e. the Msg3 early identification solution) can be supported for MsgA early identification. It is up to RAN1 on the need of dedicated preamble and/or dedicated PUSCH resource configuration.

  - Cell barring and RedCap supporting

It is agreed that SIB1 indicates cell barring for 1 Rx branch and 2 Rx branches separately for RedCap UEs. For the ASN.1 format, we propose to use two mandatory IEs in SIB1 with {barred, notBarred} values for 1RX and 2RX respectively. It not only can be used to indicate whether gNB supports RedCap UE, but also save signaling overhead compared to each IE configured as optional fields separately.

Proposal 7: Introduce two mandatory IEs in SIB1 with {barred, notBarred} values for 1RX and 2RX RedCap UE respectively.

In last meeting, RAN2 agreed the presence/absence of RedCap-specific IFRI UE can be used to indicate the cell does support RedCap or not. Besides, the RedCap specific cellBarred for 1RX and 2RX can be also used by UE to consider the support of RedCap UE in the Network. RAN2 to discuss whether RedCap UE uses the absence of RedCap specific cellBarred or RedCap specific IFRI, to consider the gNB as not supporting RedCap.

Proposal 8: RAN2 to discuss whether RedCap UE uses the absence of RedCap specific cellBarred or RedCap specific IFRI, to consider the gNB as not supporting RedCap.

In case that the RedCap specific IFRI/cellBarred is absent, the UE considers that the cell does not support RedCap and cannot camp on this cell. It is, however, not clear how the UE performs intra-frequency reselection in this case since there is no RedCap specific IFRI to follow.

One possible way is to let the UE apply the IFRI in MIB which is mandatory present. However, considering that the cell is incapable of RedCap function, we think a more reasonable solution is to directly allow the UE to consider other cells on the same frequency instead of applying the legacy IFRI which is not intended for RedCap. By allowing the intra-frequency reselection, RedCap UEs can have more opportunities to find a suitable cell for camping, since one cell not supporting RedCap does not have to mean other intra-frequency cell has the same situation.

Proposal 9: Intra-frequency cell reselection is considered as “allowed” by RedCap UEs, in case the cell is barred due to not supporting RedCap.

  - RAN3 LS reply

In the LS, RAN3 has some questions about RedCap UE’ behaviour and how access control work in the legacy cells. From RAN 2 point of view, we can give a confirmation to RAN3 that RedCap UEs should not attempt to camp/access in legacy cells or be handed over to such cells. When RedCap UEs select a legacy cell and receive the SIB1, if the RedCap specific cellBarred or IFRI is not present, RedCap UEs consider the cell is barred.

Proposal 10a: RAN2 reply RAN3 that RedCap UEs should not attempt to camp/access in legacy cells or be handed over to such cells, and RedCap UEs consider the legacy cell by the absence of RedCap specific cellBarred or IFRI.

When a legacy gNB receive the (RedCap) UE Radio Capabilities (e.g. at Handover preparation), the gNB cannot recognize the new fields and detect the UE is a RedCap UE. The legacy gNB may also accept the Handover request, and send the fullconfig to the UE. Then the RedCap UE may failed to access the legacy gNB, result to handover failure.

Proposal 10b: RAN2 reply RAN3 that legacy gNB can NOT detect via the (RedCap) UE Radio Capabilities (e.g. at Handover preparation) that it cannot configure or serve the RedCap UE.

  - Cell selection

As discussed in SI [2], for FR1, under the consideration of potential reduced antenna efficiency due to device size limitations, the MIL(s) of PUSCH and/or Msg3 are worse than that of the bottleneck channel for the reference NR UE. Thus, considering the bad coverage of UL, it is better to introduce the specific cell selection parameters for RedCap UEs to allow them to select a suitable cell and work normally.

Proposal 11: Support the RedCap specific cell selection parameters.

  - UAC

SA1 [3] has concluded that no new UAC Access Category or Access Identity is required, since current UAC can apply also to RedCap UEs, and UAC Access Categories and Access Identities are not intended to differentiate UEs based on device radio capabilities.

In order to provide flexible and separate control for RedCap UEs, as the network may want to control RedCap UEs separately from non-RedCap UEs, it is better to broadcast a specific set of UAC parameters for RedCap UEs, which is similar to NB-IoT and eMTC. However, it will increase signalling overheads if a complete set of UAC parameters (i.e. UAC-BarringInfoSet which contains uac-BarringFactor, uac-BarringTime and uac-BarringForAccessIdentity) specific to Redcap UEs is broadcast. Therefore, how to design the RedCap specific UAC parameters needs to be further considered in order to reduce the signalling overhead and enable a flexible configuration. For example, based on the CT1 LS [4] discussed in RAN2#115-e meeting, there is a solution provided by CT1 that an offset value can be introduced to the unified access control barring information. By this way, a specific UAC parameters can be provided for RedCap UEs with only limited signalling overhead.

Proposal 12: Support the RedCap specific UAC parameters, with consideration of signalling overhead reduction.

- Conclusion

The contribution focuses on UE identification and access restriction for RedCap UEs. Corresponding observations and proposals are listed as below:

Proposal 1: In MAC perspective, RedCap UE use Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured by NW.

Proposal 2: For Msg1 early identification, RAN2 confirm both dedicated ROs and dedicated PRACH preamble should be supported.

Proposal 3: Msg1 early identification is enabled/disabled implicitly by the presence of dedicate RACH configuration for Msg1 early identification.

Proposal 4: Two reserved LCIDs are used for CCCH and CCCH1 cases respectively for Msg3 early identification.

Proposal 5: In MAC perspective, RedCap UE uses the dedicated LCID for Msg3 early identification, whenever the Msg3 includes the CCCH data.

Proposal 6: At least the dedicated LCID (i.e. the Msg3 early identification solution) can be supported for MsgA early identification. It is up to RAN1 on the need of dedicated preamble and/or dedicated PUSCH resource configuration.

Proposal 7: Introduce two mandatory IEs in SIB1 with {barred, notBarred} values for 1RX and 2RX RedCap UE respectively.

Proposal 8: RAN2 to discuss whether RedCap UE uses the absence of RedCap specific cellBarred or RedCap specific IFRI, to consider the gNB as not supporting RedCap.

Proposal 9: Intra-frequency cell reselection is considered as “allowed” by RedCap UEs, in case the cell is barred due to not supporting RedCap.

Proposal 10a: RAN2 reply RAN3 that RedCap UEs should not attempt to camp/access in legacy cells or be handed over to such cells, and RedCap UEs consider the legacy cell by the absence of RedCap specific cellBarred or IFRI.

Proposal 10b: RAN2 reply RAN3 that legacy gNB can NOT detect via the (RedCap) UE Radio Capabilities (e.g. at Handover preparation) that it cannot configure or serve the RedCap UE.

Proposal 11: Support the RedCap specific cell selection parameters.

Proposal 12: Support the RedCap specific UAC parameters, with consideration of signalling overhead reduction.

- Reference

- Report of 3GPP TSG RAN WG2 meeting #115-e, Online

- TR 38.875 V2.0.0: Study on support of reduced capability NR devices.

- S1-211363, Reply LS on Unified Access Control (UAC) for RedCap, SA1.

- C1-213527 LS on UAC enhancements for minimization of service interruption when disaster condition applies, CT1.