3GPP TSG RAN WG2 Meeting #116-e           		              		R2-2109612

Electronic meeting, 1st – 12th Nov 2021

Agenda Item:	8.4.3

Source:	Intel Corporation

Title:	IAB-node dual connection, RLF and local rerouting

Document for:	Discussion and Decision

# Introduction

In last RAN2 #114e meeting, type-2 RLF indication is agreed to be triggered at RLF detection, FFS whether for both single and dual parent IAB-node. Also, RAN2 further captured following agreement for local rerouting triggered by dual-connected IAB-node:

- If an IAB node with dual parents (via DC) receives type-2 BH RLF indication from one parent, IAB-node may trigger a local re-routing to the other parent. The detail of local re-routing and whether/how the action on type-2 indication is configurable is FFS.

In Rel-16, the NR DC framework (e.g. MCG/SCG-related procedures) is used to configure the dual radio links with the parent nodes. This principle remains the same in Rel-17. In NR-DC framework, constraints are considered during RLF at one of the links. A dual-connected UE may take different actions based on its configuration and which link is failed (e.g. MCG or SCG). For a dual-parent IAB-node, the trigger condition and behavior upon receiving RLF indication (type-2/3/4) highly depends on dual-parent IAB-node’s RLF and recovery procedure. For example, another path may not be available (e.g. released/changed) during RLF recovery, which leads to no option for local rerouting at dual-parent IAB-node.

In this contribution, we first analyze the availability of paths of a dual-parent IAB-node based on different configurations and RLF scenario. Based on that, we further provide a solution to support IAB-node local-rerouting without considering RLF recovery constraints in NR-DC framework.

Based on the proposed solution, we also discuss the trigger condition and behavior upon receiving type-2/3 RLF indication. In the end, local rerouting trigger condition are discussed.

# Discussion

## RLF recovery of dual-parent IAB-node

### MCG/SCG failure handling and local rerouting

In Rel-16, if one IAB-node is connected to two parent nodes, NR-DC framework is used to configure dual radio links as BH links, which is specified in TS38.300 section 4.7.4.3 [1]:

The parent nodes' gNB-DU functionality together with the IAB-donor-CU obtains the role of the IAB-MT's master node or secondary node. The NR DC framework (e.g. MCG/SCG-related procedures) is used to configure the dual radio links with the parent nodes (TS 37.340 [21]).

Besides, if RLF occurs at the IAB-node, for an IAB-node (including both single-parent and dual-parent IAB-node), following principle should be following as specified in TS38.300 section 9.2.7 [1]:

When RLF occurs at the IAB BH link, the same mechanisms and procedures are applied as for the access link. This includes BH RLF detection and RLF recovery.

“The same mechanisms and procedures” refers to UE’s RLF declaration and RLF recovery mechanisms and procedures. To be more specific, in RAN2 #107bis meeting, RAN2 also agreed when NR-DC is configured for the IAB-node, BH RLF recovery for DC case reuses UE’s MCG and SCG failure recovery procedures specified in Rel-16.

- When NR DC is configured for the IAB-node, 2.1 RLF is detected separately for the MCG-link and for the SCG-link, and 2.2 existing UE procedures are used for MCG-link and SCG-link failure handling. - The following is agreed as working assumption: BH RLF recovery for DC case reuses UE’s MCG and SCG failure recovery procedures specified in Rel-16. - For DC case, the IAB-node considers the radio link is failed and uses RRC existing or Rel-16 Mechanism (e.g. MCG or SCG failure report, RRC reestablishment) if “Recovery Failure” notification is received from parent nodes on MCG-link or/and SCG-link.

As specified in [1], one IAB-node can perform local rerouting when its BH link has RLF.

The IAB-node can receive multiple routing configurations with the same destination BAP address but different BAP path IDs. These routing configurations may resolve to the same or different egress BH links. In case the BH link has RLF, the IAB-node may select another BH link based on routing entries with the same destination BAP address, i.e., by disregarding the BAP path ID. In this manner, a packet can be delivered via an alternative path in case the indicated path is not available.

Recalling the UE procedure of MCG and SCG failure recovery, the following three scenarios of DC-configured IAB-node RLF are considered:

1) MCG-link RLF, fast MCG link recovery is not configured

2) MCG-link RLF, fast MCG link recovery is configured

3) SCG-link RLF

Figure 1. Intra-donor dual-parent IAB-node

Figure 1 shows a topology of a dual-parent IAB-node which is connected to two parent nodes within the same IAB-donor CU. Assuming BH RLC CH C between IAB-node 1 and IAB-node 3 is the MCG-link, while BH RLC CH D between IAB-node 2 and IAB-node 3 is the SCG-link. We will discuss the above three scenarios respectively. In the first two scenarios (i.e. MCG-link RLF), the original link for traffic transmission is MCG-link. In the third scenario (SCG-link RLF), we assume SCG-link is the original link.

MCG-link RLF, fast MCG link recovery is not configured

MCG-link RLF of IAB-node 3 can be triggered by RLF of the BH link itself or upon receiving type-4 RLF indication from IAB-node 1 (due to RLF recovery failure of BH RLC CH A). Assuming fast MCG link recovery is not configured at IAB-node 3, IAB-node 3 declares MCG-link RLF, release the connection to IAB-node 1, and initiate RRC reestablishment. According to TS37.340 and TS38.331, MR-DC release is performed during RRC reestablishment, including spCellConfig release (if configured) [2][3]. In this case, the SCG-link in Figure 1 (i.e. BH RLC CH D between IAB-node 2 and IAB-node 3) is not available anymore, as SN is released during MN RRC reestablishment. Hence, local rerouting is not possible when MCG-link is RLF and fast MCG link recovery is not configured.

- When MCG-link is in RLF, SN is released and SCG-link is not available. Local rerouting is not available if fast MCG link recovery is not configured.

MCG-link RLF, fast MCG link recovery is configured

If fast MCG link recovery is configured at IAB-node 3, following SCG/MCG failure handling procedure specified in [2], IAB-node 3 shall suspend MCG transmission for all radio bearers and reports the failure with MCG Failure Information message to the MN via the SCG. Upon reception of the MCG Failure Indication, the MN can send RRC reconfiguration with reconfigurationWithSync message to IAB-node 3 and IAB-node 3 will resume MCG transmissions for all radio bearers after MCG handover. Together with this RRC reconfiguration message, SN can be changed/released or remain the same.

Before receiving RRC reconfiguration message from MN, there’s a short period that SCG-link is still available, which allows IAB-node 3 to reroute the traffic to IAB-node 2 via SCG-link (i.e. BH RLC CH D). However, once IAB-node 3 receives RRC reconfiguration message from MN, whether local rerouting is still available depends on the SN status.

- If fast MCG link recovery is configured and before RRC reconfiguration with reconfigurationWithSync message is received from MN, local rerouting is temporarily available.

It was discussed in RAN2 #114-e meeting whether UE can continue the transmission on SCG during handover. Following agreement is captured:

- [005] Clarify that UE may stop the SCG transmission/reception during the HO without SCG reconfigurationWithSync configuration (no TS impact)

It indicates that SN may not be changed in the RRC Reconfiguration message during MCG HO, local rerouting via SCG-link may be still available. However, if

- If fast MCG link recovery is configured, local rerouting via SCG-link remains available during MCG handover, as IAB-node may continue traffic in SCG-link if there is no change in SCG.

However, after receiving reconfigurationWithSync, if SN is changed or released, IAB-node 3 will receive a new SCG configuration (including new BAP configuration of SCG parent as well) or release SCG. The SCG which is used for holding local rerouting traffic will not be available after this point. IAB-node 3 shall restart the traffic originally transmitted via MCG-link to the new MN after successful RACH procedure.

- If fast MCG link recovery is configured and SN is changed or released, SCG-link is not available anymore for local rerouting upon reception of reconfigurationWithSync message from MN.

In some rare cases, the MN can also send a RRC release message to IAB-node 3, upon which IAB-node 3 need to release all the radio bearers and configurations. SCG is also released together with MCG release, which is the same as the case when fast MCG link recovery is not configured. In this case, IAB-node 3 is not integrated to any parent node.

- If fast MCG link recovery is configured and IAB-MT receives a RRC release from MN, the IAB-node is not integrated to any parent node.

SCG-link RLF

For traffic originally routing via SCG-link, when SCG-link is RLF, if MCG is not suspended, the UE will report the SCG Failure Information to the MN, instead of triggering re-establishment. In this case, MCG-link is still available for local rerouting, IAB-node 3 can reroute traffic via SCG-link to MCG-link if the same destination address is shared between two paths.

- When SCG-link is RLF, traffic originally routed via SCG-link can be rerouted locally to MCG-link.

In summary, local rerouting at a dual-parent IAB-node may not work well in some failure scenarios.

### MCG/SCG failure handling enhancement for IAB

The main reason that leads to unsuccessful local rerouting during RLF is that MCG-link and SCG-link is not treated equally. In NR-DC framework, SCG may be impacted when MCG is released/reconfigured/re-established. This makes the SCG-link of one IAB-node may not be always available when MCG-link is RLF. It should be noted that NR-DC is originally proposed to improve system throughput, where one UE can connect to two gNBs simultaneously.

For an IAB network, when an IAB-node has two parent nodes, treating two links with two parent nodes identically can help to improve system flexibility, as well as improve system robustness via topology redundancy.

In this case, to remain the availability of SCG-link in the first two scenarios (MCG-link RLF) in section 2.1.1, SCG should not be changed or released when IAB-MT’s MCG-link is RLF. To minimize the specification impact, a simple way is to avoid SCG change or release when MCG is reconfigured and configure MCG fast recovery at IAB-MT is necessary. A text proposal to TS37.340 and TS38.300 can be found in Annex.

- Capture that “Fast recovery should be configured for IAB-MT with NR-DC.  SCG should not be changed or released when IAB-MT’s MCG-link is RLF” in TS37.340 and TS38.300.

## Type-2/3 RLF indication

Assuming proposal 1 in Section 2.1 will be adopted, in this section, we mainly focus on the trigger condition of type-2/3 RLF indication and behaviour after receiving one of the indications.

### Trigger condition

Single-connected IAB-node Trigger Condition

We first consider when a single-connected IAB-node should (e.g. IAB-node 1 or IAB-node 2 in Figure 1) send trigger condition to its child node. It was agreed in RAN2 #114e meeting:

- The trigger to generate a type 2 RLF indication is at RLF detection. FFS whether for both: single and dual connection cases.

Hence, after an IAB-MT declares RLF (i.e. when a BH RLF is detected at IAB-MT), for each egress link associated with the IAB-DU, the transmitting part of the collocated BAP entity at the IAB-DU should construct a BAP Control PDU for type-2 RLF indication and transmit this constructed PDU through configured egress BH RLC channel to its child IAB-node.

- For a single-connected IAB-node, IAB-DU should construct a BAP Control PDU for type-2 RLF indication and transmit to its child IAB-node through configured egress BH RLC channel after a BH RLF is detected at the associated IAB-MT.

For type-3 RLF indication, after an IAB-MT recovers from RLF (i.e. RRC re-establishment successful), for each egress link associated with the IAB-DU, the transmitting part of the collocated BAP entity at the IAB-DU may construct a BAP Control PDU for type-3 RLF indication and transmit this constructed PDU to its child IAB-node.

- For a single-connected IAB-node, IAB-DU may construct a BAP Control PDU for type-3 RLF indication and transmit to its child IAB-node through configured egress BH RLC channel after successful RRC re-establishment (i.e. sending RRCReestablishmentComplete to its parent node).

Dual-connected IAB-node Trigger Condition

For a dual-connected IAB-node (e.g. IAB-node 3 in Figure 1), when to send the type-2 RLF indication may be different. As defined in TS38.300, in case the BH has RLF, the IAB-node may select another BH link based on routing entries with the same destination BAP address, i.e., by disregarding the BAP path ID. In this manner, a packet can be delivered via an alternative path in case the indicated path is not available.

As analyzed in Section 2.1, it is proposed to configure fast MCG recovery and not change/release SCG during MN RLF recovery. If proposal 1 is agreed, the availability of the back-up path is independent from RLF at the source path. The dual-parent IAB-node can first do local rerouting and select the pack-up path without impacting its child node. The local rerouting performed at this dual-parent IAB-node is transparent to its child node, that is, there’s no need to send type-2 RLF indication to its child node.

However, if the back-up path is also experiencing RLF, or this dual-parent IAB-node also receives a type-2 or type-4 RLF indication from the back-up path, this scenario is regarded as no available path for local re-routing, as the link between its parent IAB-node and grandparent IAB-node is RLF or RLF recovery failed. Both links of this dual-parent IAB-node declare RLF or both its grandparent BH RLC link declares RLF. Type-2 RLF indication should follow the same principle for type-4 RLF indication, that is, the dual-parent IAB-node sends type-2 RLF indication to its child node when it receives type-2 RLF indication from both BH links, which can be used to indicate this dual-parent IAB-node is temporarily experiencing RLF.

On the other hand, if proposal 1 is not agreed, the availability of back-up path for local rerouting is still constrained by RLF at MCG/SCG link. Type-2 RLF indication should only be sent to its child node when both BH RLC links are unavailable.

- If local rerouting is supported at the dual-parent IAB-node, type-2 RLF indication should be provided only when both BH links are temporarily unavailable.

The traffic transmission is suspended when type-2 RLF indication is triggered at a dual-parent IAB-node. If one of the BH RLC links is available (i.e. recovered from scenarios listed in proposal 4), this dual-connected IAB-node can resume/reroute traffic transmission to the available path which is first recovered.

- A dual-parent IAB-node sends a type-3 RLF indication to its child IAB-nodes when one of the BH RLC links is available and resumes traffic transmission through the available BH RLC channel. A dual-parent IAB-node triggers type-3 RLF indication after following scenario:

- Sending RRCReestablishmentComplete

- Successful RACH procedure to MCG after receiving reconfigurationWithSync

- Successful RACH procedure to SCG after receiving RRCReconfiguration

Each scenario of type-3 RLF indication reflects the successful recovery of at least one BH links after type-2 RLF indication. Hence, type-3 RLF indication can only be triggered if type-2 RLF indication is triggered beforehand.

- IAB-node should not trigger type-3 RLF indication if type-2 RLF indication is not triggered beforehand.

### Behavior upon receiving of a type-2/3 RLF indication

In email discussion [075][eIAB] open issues on re-routing, whether to specify the behaviour upon receiving type-2/3 RLF indication was discussed. In this section, we analysed the possible behaviours of an IAB-node after it receives type-2/3 RLF indication.

It was defined in Rel-16 that an IAB-node will declare RLF upon receiving type-4 RLF indication. Different from type-4 RLF indication, type-2 RLF indication will be triggered at RLF detection. IAB-node may still transmit packets to its parent nodes, as the BH RLC link of its parents may be recovered soon. Considering that, an IAB-node will not declare RLF when receiving a type-2 RLF indication from its parent node. Furthermore, as type-2 RLF indication will not trigger RLF declaration, the IAB-node which receives the type-2 RLF indication will not generate a new type-2 RLF indication to its child IAB-node. That is, type-2 RLF indication will not propagate to child IAB-node.

- Upon receiving type-2 RLF indication, IAB-node will not declare RLF. IAB-node will not propagate type-2 RLF indication to its child IAB-node.

De-activation of iab-support in SIB upon receiving type-2 RLF indication

As discussed above, considering IAB nodes are fixed, the RLF may be mainly caused by temporary blockage, which can be recovered soon. The intention of deactivation of IAB-supported in SIB is to avoid IAB nodes attempting to get access to a failed IAB node which may not be recovered soon. IAB node who sends type-2 RLF indication should not be banned.

Moreover, triggering the deactivation of IAB-support by type-2 RLF indication also needs to modify system information twice. The first modification is to mute “IAB support” in SIB1 in order to bar the access to new IAB nodes when sending type-2 RLF indication. The second modification is to modify the system information back to “IAB-support” once RLF is recovered (when sending type-3 RLF indication). However, SIB modification can be very expensive, which makes deactivation of IAB-support by type-2 RLF indication inefficient.

- IAB-node should not deactivate iab-support in SIB upon receiving a type-2 RLF indication.

Trigger local rerouting

For a single-parent IAB-node, there’s no back-up path for local re-routing. It is straightforward that a single-parent IAB-node will not trigger local rerouting upon receiving type-2 RLF indication.

For a dual-parent IAB-node, if there’s an available link, local re-routing should be supported in case any BH RLC link is failed. It is also the same for type-3 RLF indication, once there’s a BH RLC link recovered from RLF (i.e. become available), IAB-node may resume the traffic through the available BH link.

To be more specific, following cases should be considered as the available BH link. An IAB-node can switch traffic to the available link if one of the BH links is not available.

1) no RLF detection

2) no RLF indication received from parent IAB-node (including type-2 and type-4 RLF indication)

3) fast MCG RLF recovery is configured, and SCG-link is not changed or released during MN RRC reconfiguration

It was also discussed in [6] whether additional information should be included in the control PDU. Similar as type-4 RLF indication, type-2 RLF indication is also sent per BH link. It is possible that the source path will be recovered within a short time. To avoid overloaded the available path, local rerouting upon receiving a type-2 RLF indication can be supported per BH link or per routing ID. It can be left to implementation which one to be selected based on the channel and congestion status of the available BH link.

- Per BH link or per routing ID local rerouting is supported upon receiving a type-2 RLF indication. The available BH link of the IAB-node is selected as the target rerouting path.

Trigger CHO upon receiving type-2 RLF indicaiton

From our understanding, if a child IAB-node (i.e. IAB-node 2) is dual-connected to two IAB parent nodes (i.e. IAB-node 1 and IAB-node 3), if the link between IAB-node 2 and IAB-node 3 is still available, it is possible that IAB-node 2 can reroute upstream packets locally to anther link (i.e. to IAB-node 3). Compared with CHO, local rerouting will not experience service interruption, as RRC reconfiguration is not needed for IAB-node 2 (i.e. child IAB-node which receives type-2 RLF indication) and its descendant IAB-nodes, which can also save a lot of signaling overhead. For this circumstance, CHO is not necessary for a dual-connected IAB-node which receives type-2 RLF indication when local rerouting is possible.

- Upon receiving type-2 RLF indication from its parent node, a dual-connected IAB-node can trigger local rerouting if the other link is still available, CHO is not necessary.

If IAB-node is single-connected to one parent IAB-node, one may claim that CHO at the child node can help to reduce service interruption, especially during the time when its parent node is trying to recover from RLF. However, we think the benefit is quite limited for following two reasons:

1) Service interruption

As discussed in [5], RLF is rare event. Besides, due to IAB-nodes are fixed nodes, for most cases, RLF will be recovered soon. If IAB-node 1 (i.e. parent node) is configured with CHO, the service interruption can be further reduced, as the parent node can adopt the conditional RRC reconfiguration directly if the suitable cell is one of the CHO candidate cell.

- The parent IAB-node can be recovered from RLF soon. The delay can further be reduced if the parent IAB-node is configured with CHO.

However, if type-2 RLF indication triggers CHO at the child IAB-node (IAB-node 2), for this child IAB-node itself, it surely can reduce service interruption by not waiting for RRC reestablishment at its parent node. However, if this child (IAB-node 2) node also has descendant IAB-nodes, more delay will be introduced due to RRC reconfiguration for those descendant IAB-nodes. Besides, BAP routing configuration may also need to be updated as IAB-node 2 has migrated to a new parent IAB-node. Extra work and delay will be introduced for scheduling update and BAP header remapping. With more layers of descendant IAB-node, the delay will be increased. If IAB-node 2 selects a new parent node which is under a different IAB-donor CU, it is also necessary for all descendant IAB-node to perform RACH. This will introduce more service interruption delay. Considering above listed delay, the service interruption time of performing CHO under type-2 RLF indication would be longer than waiting for parent node’s recover from RLF.

- If CHO is executed when receiving type-2 RLF indication, the delay of service interruption includes RRC reconfiguration of CHO-performing IAB-node and its descendant IAB-nodes, BAP routing configuration update, scheduling information update, and RACH procedure delay (if new parent is under a different IAB-donor CU), which is longer than RLF recovery.

- Type-2 RLF indication as CHO execution condition is not beneficial to reduce service interruption of IAB network.

2) Reduce signaling overhead

As analyzed by above context, it is also obvious to see that if CHO is triggered by type-2 RLF indication, more signaling is required to be exchanged within the IAB network, e.g. RRC reconfiguration for descendant IAB nodes after CHO, BAP mapping information update via F1-C messages. However, those messages can be avoided if IAB-node waiting for its parent node RLF recovery. This can also help to maintain the stability of the IAB network topology.

- Execute CHO upon receiving type-2 RLF indication will introduce more signaling overhead compared with waiting for parent IAB-node RLF recovery.

- Type-2 RLF indication is not needed as CHO execution condition.

## local rerouting

Local IAB nodes can make faster decision of routing path changing comparing to centralized rerouting which require measurement report from local nodes and send BH mapping information update from IAB-donor-CU accordingly.

### Trigger Condition

In previous RAN2 meetings, following agreements are captured for local rerouting:

- Local rerouting can be triggered by indication of hop-by-hop flow control. Further details, e.g., on trigger information, trigger conditions, role of CU configuration, are FFS. - Local re-routing based on flow control feedback is allowed based on certain value of available buffer size. FFS further details. (Current hbh fc is for DL traffic. - If an IAB node with dual parents (via DC) receives type-2 BH RLF indication from one parent, IAB-node may trigger a local re-routing to the other parent. The detail of local re-routing and whether/how the action on type-2 indication is configurable is FFS. - A configured threshold of available buffer size based on flow control feedback is used to determine the congestion, for the purpose of local re-routing.

Type-2 RLF indication and type-3 RLF indication

As discussed in section 2.2.2, local rerouting can be triggered upon receiving type-2 and type-3 RLF indication.

Upstream HbH flow control

As proposed in [4], upstream hop-by-hop flow control should be introduced avoid upstream traffic congestion.

As agreed in RAN2 #115e meeting:

- A configured threshold of available buffer size based on flow control feedback is used to determine the congestion, for the purpose of local re-routing.

Similar as downstream HbH flow control, local rerouting for upstream traffic can also be triggered after receiving upstream HbH flow control from the parent IAB-nodes.

Upstream HbH flow control triggered local rerouting is different from local rerouting which is triggered by RLF indication. When an IAB-node receives a RLF indication, the BH link is treated as not available. In this case, none of the packets should continue its transmission towards the original parent IAB-node. However, the BH link is still available (i.e. no RLF detected, or no RLF indication) when receiving an upstream HbH flow control feedback. In this scenario, the dual-parent IAB-node can reroute partial traffic to the back-up path. This approach can not only reduce the burden to the congested parent IAB-node, but also adaptively response to HbH flow control feedback with local rerouting in a smaller granularity (e.g. per routing ID, per UE bearer).

- Local re-routing based on uplink hop-by-hop flow control feedback is supported based on a threshold value of available buffer size.

# Conclusion

In this contribution, we first analyze the availability of the MCG/SCG link of one dual-connected IAB-node during RLF recovery. Based on the availability of MCG and SCG link of one dual-connected IAB-node, we further discussed the trigger condition of type-2/3 RLF indication, as well as the behavior upon receiving one, e.g. iab-support should not be disabled in SIB after receiving type-2 RLF indication, type-2 RLF indication is not used to trigger CHO, etc. In the end, we discussed other trigger conditions for local rerouting at one IAB-node.

We propose following observations and proposals:

Observation 1:When MCG-link is in RLF, SN is released and SCG-link is not available. Local rerouting is not available if fast MCG link recovery is not configured.

Observation 2:If fast MCG link recovery is configured and before RRC reconfiguration with reconfigurationWithSync message is received from MN, local rerouting is temporarily available.

Observation 3:If fast MCG link recovery is configured, local rerouting via SCG-link remains available during MCG handover, as IAB-node may continue traffic in SCG-link if there is no change in SCG.

Observation 4:If fast MCG link recovery is configured and SN is changed or released, SCG-link is not available anymore for local rerouting upon reception of reconfigurationWithSync message from MN.

Observation 5:If fast MCG link recovery is configured and IAB-MT receives a RRC release from MN, the IAB-node is not integrated to any parent node.

Observation 6:When SCG-link is RLF, traffic originally routed via SCG-link can be rerouted locally to MCG-link.

Proposal 1:Capture that “Fast recovery should be configured for IAB-MT with NR-DC.  SCG should not be changed or released when IAB-MT’s MCG-link is RLF” in TS37.340 and TS38.300.

Proposal 2:For a single-connected IAB-node, IAB-DU should construct a BAP Control PDU for type-2 RLF indication and transmit to its child IAB-node through configured egress BH RLC channel after a BH RLF is detected at the associated IAB-MT.

Proposal 3:For a single-connected IAB-node, IAB-DU may construct a BAP Control PDU for type-3 RLF indication and transmit to its child IAB-node through configured egress BH RLC channel after successful RRC re-establishment (i.e. sending RRCReestablishmentComplete to its parent node).

Proposal 4:If local rerouting is supported at the dual-parent IAB-node, type-2 RLF indication should be provided only when both BH links are temporarily unavailable.

Proposal 5:A dual-parent IAB-node sends a type-3 RLF indication to its child IAB-nodes when one of the BH RLC links is available and resumes traffic transmission through the available BH RLC channel. A dual-parent IAB-node triggers type-3 RLF indication after following scenario:

- Sending RRCReestablishmentComplete

- Successful RACH procedure to MCG after receiving reconfigurationWithSync

- Successful RACH procedure to SCG after receiving RRCReconfiguration

Proposal 6:IAB-node should not trigger type-3 RLF indication if type-2 RLF indication is not triggered beforehand.

Proposal 7:Upon receiving type-2 RLF indication, IAB-node will not declare RLF. IAB-node will not propagate type-2 RLF indication to its child IAB-node.

Proposal 8:IAB-node should not deactivate iab-support in SIB upon receiving a type-2 RLF indication.

Proposal 9:Per BH link or per routing ID local rerouting is supported upon receiving a type-2 RLF indication. The available BH link of the IAB-node is selected as the target rerouting path.

Observation 7:Upon receiving type-2 RLF indication from its parent node, a dual-connected IAB-node can trigger local rerouting if the other link is still available, CHO is not necessary.

Observation 8:The parent IAB-node can be recovered from RLF soon. The delay can further be reduced if the parent IAB-node is configured with CHO.

Observation 9:If CHO is executed when receiving type-2 RLF indication, the delay of service interruption includes RRC reconfiguration of CHO-performing IAB-node and its descendant IAB-nodes, BAP routing configuration update, scheduling information update, and RACH procedure delay (if new parent is under a different IAB-donor CU), which is longer than RLF recovery.

Observation 10:Type-2 RLF indication as CHO execution condition is not beneficial to reduce service interruption of IAB network.

Observation 11:Execute CHO upon receiving type-2 RLF indication will introduce more signaling overhead compared with waiting for parent IAB-node RLF recovery.

Proposal 10:Type-2 RLF indication is not needed as CHO execution condition.

Proposal 11:Local re-routing based on uplink hop-by-hop flow control feedback is supported based on a threshold value of available buffer size.

# References

- TS38.300, NR and NG-RAN overall description, Stage 2 (Release 16)

- TS37.340, Multi-connectivity, Stage 2 (Release 16)

- TS38.331, Radio Resource Control (RRC) protocol specification (Release 16)

- R2-2109611, Discussion on remaining open issues of LCG range extension and congestion, Intel

- R2-2102238, Report from [Post112-e][066][eIAB] Topology Adaptation (QC)

- R2-2107251, Summary of [Post114-e][075][eIAB] Open Issues on Re-routing, Huawei

# Annex A – TP to TS38.300

////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

### 6.11.3	Routing and BH-RLC-channel mapping on BAP sublayer

Figure 6.11.3-1: Routing and BH RLC channel selection on BAP sublayer

Routing on BAP sublayer uses the BAP routing ID, which is configured by the IAB-donor-CU. The BAP routing ID consists of BAP address and BAP path ID. The BAP address is used for the following purposes:

1.	Determination if a packet has reached the destination node, i.e. IAB-node or IAB-donor-DU, on BAP sublayer. This is the case if the BAP address in the packet's BAP header matches the BAP address configured via RRC on the IAB-node, or via F1AP on the IAB-donor-DU.

2.	Determination of the next-hop node for packets that have not reached their destination. This applies to packets arriving from a prior hop on BAP sublayer or that have been received from IP layer.

For packets arriving from a prior hop or from upper layers, the determination of the next-hop node is based on a routing configuration provided by the IAB-donor-CU via F1AP signalling or a default configuration provided by the IAB-donor-CU via RRC signalling. This F1AP configuration contains the mapping between the BAP routing ID carried in the packet's BAP header and the next-hop node's BAP address.

Table 6.11.3-1: Routing configuration

| BAP routing ID | Next-hop BAP address |
| --- | --- |
| Derived from BAP packet's BAP header | Egress link to forward packet |

The IAB-node resolves the next-hop BAP address to a physical backhaul link. For this purpose, the IAB-donor-CU provides the IAB-node/IAB-donor-DU with its child-node's BAP address via F1AP, and it provides the IAB-node with its parent-node's BAP address via RRC.

The IAB-node can receive multiple routing configurations with the same destination BAP address but different BAP path IDs. These routing configurations may resolve to the same or different egress BH links. In case the BH link has RLF, the IAB-node may select another BH link based on routing entries with the same destination BAP address, i.e., by disregarding the BAP path ID. In this manner, a packet can be delivered via an alternative path in case the indicated path is not available.

NOTE: To support rerouting to another egress BH link with the same destination BAP address, fast recovery should be configured for IAB-MT with NR-DC.  SCG should not be changed or released when IAB-MT’s MCG-link is RLF.

# Annex B – TP to TS37.340

////////////////////////////////////////////////////////////irrelevant operations skipped//////////////////////////////////////////////////////////////////

## 7.7	SCG/MCG failure handling

RLF is declared separately for the MCG and for the SCG.

If radio link failure is detected for MCG, and fast MCG link recovery is configured, the UE triggers fast MCG link recovery. Otherwise, the UE initiates the RRC connection re-establishment procedure. During the execution of CPC, if radio link failure is detected for MCG, the UE initiates the RRC connection re-establishment procedure.

During fast MCG link recovery, the UE suspends MCG transmissions for all radio bearers and reports the failure with MCG Failure Information message to the MN via the SCG, using the SCG leg of split SRB1 or SRB3.

The UE includes in the MCG Failure Information message the measurement results available according to current measurement configuration of both the MN and the SN. Once the fast MCG link recovery is triggered, the UE maintains the current measurement configurations from both the MN and the SN, and continues measurements based on configuration from the MN and the SN, if possible. The UE initiates the RRC connection re-establishment procedure if it does not receive an RRC reconfiguration message, MobilityFromNRCommand message, MobilityFromEUTRACommand message or RRC release message within a certain time after fast MCG link recovery was initiated.

Upon reception of the MCG Failure Indication, the MN can send RRC reconfiguration message, MobilityFromNRCommand message, MobilityFromEUTRACommand message or RRC release message to the UE, using the SCG leg of split SRB1 or SRB3. Upon receiving an RRC reconfiguration message, MobilityFromNRCommand message or MobilityFromEUTRACommand message, the UE resumes MCG transmissions for all radio bearers. Upon receiving an RRC release message, the UE releases all the radio bearers and configurations.

NOTE 1:	It is up to network implementation to guarantee that the RRC-related messages are delivered to the UE by the SN before the release of its control plane resources.

NOTE 2: For an IAB-MT, fast recovery should be configured for IAB-MT with NR-DC. SCG should not be changed or released when IAB-MT’s MCG-link is RLF.