3GPP TSG RAN WG2 Meeting #116-e                                                        	   R2-2109615

E-Meeting, 1st – 12th November, 2021

Agenda Item:	8.16.3

Source: 	Intel Corporation

Title:	GINs for network selections

Document for: 	Discussion/Decision

# Introduction

SA2 has replied to RAN2 questions on whether the GINs are common and whether same or different values can be used for onboarding and for credential holders in S2-2106708.

In this contribution, this issue is being discussed.

# Discussion

Based on the SA2 TR conclusion, the group IDs need to be also considered for onboarding network selection:

-	The NG-RAN of the Onboarding network includes an indication for Onboarding enabled in the SIB (per O-SNPN, considering that the NG-RAN can be shared) so that the UE can discover and select an appropriate O-SNPN. The UE may or may not be pre-configured with O-SNPN network selection information (e.g. O-SNPN network identifiers or Group ID(s)). The O-SNPN network selection information can assist the UE such that the UE either preferably or exclusively select an O-SNPN corresponding to the O-SNPN network identifiers or Group ID(s).

NOTE 2:	The format of the pre-configured information assisting the UE for O-SNPN selection is not specified.

NOTE 3:	The Group ID(s) in the SIB that UE can use for selecting an O-SNPN are the same as the Group ID(s) in the SIB that the UE uses for SNPN selection as part of KI#1.

NOTE 4:	Whether the indication for Onboarding is sufficient or more SIB information is needed can be further discussed in the normative phase.

From NOTE 3, SA2 already clarifies that the Group ID(s) in the SIB that UE can use for selecting an O-SNPN are the same as the Group ID(s) in the SIB that the UE uses for SNPN selection as part of KI#1 (Support of credentials of third party entities in SNPN).  This note was added based on a discussion in SA2 [4] where it was discussed whether the Group ID needs to also contain an onboarding indicator. Extracted from [4] “Under the assumption that the list of Group IDs in case of network sharing is broadcasted per SNPN-ID, there is no need for separate onboarding indicator per Group ID since if there is a desire an (O/S)-SNPN to have separate interconnection agreements for onboarding and access to Separate Entity (SE) with 3rd party credentials it can do so by using different Group IDs. This option does not require any extra standardization.”  See the diagram extracted from [4]:

Figure 1: NG-RAN broadcasting different Group IDs for Onboarding and access to SE with 3rd party credentials (from [4])

A LS [3] has also been sent to SA2 to check with SA2 on this with the following questions:

- Question 1a: Can a common list of GINs be used for both onboarding and SNPN access using external credentials?

- Question 1b: Can RAN2 assume that higher layers may use different GIN values for each feature?

And SA2 have replied ‘Yes’ to the Question 1a and for question 1b, it stated that SA2 does not make any assumption i.e. either same or different GIN value for each feature is possible.

From this, it means that there is no need for separate Group IDs per SNPN for onboarding purpose and for support of credentials of third party entities in SNPN; which GID is used for onboarding or CH can be done transparent to the AS.

Observation#1: Which GID for onboarding or for CH or for both are configured over the NAS and hence there is no need for separate GID signalling or explicit indication for each GID in the SIB broadcast whether they are for onboarding or CH or for both,

Proposal#1: No separate Group IDs per SNPN signalling or indication per GID is needed for differentiating onboarding purpose and for support of credentials of third-party entities in SNPN; which GID is used for onboarding or CH can be done transparent to the AS.

Also in the last meeting, the following is agreed as part of the support of separate entities in SNPN:

- GIN for access using CH is broadcast only if Indication of accessing using CH is broadcast.

Since it is assumed that the same GIN list can be used for both onboarding and separate entities support, the broadcast of the GIN list should not be made dependent to the indication of accessing using CH as network may support onboarding but not separate entities support for a SNPN. GIN list for a SNPN is optionally present either if the onboarding indication is broadcast for the SNPN or if indication of accessing using CH is broadcast or both.

Observation#2: The GIN list should not be made dependent of the Indication of accessing using CH of an SNPN as the SNPN may use GIN list for just onboarding.

Proposal#2: GIN list for a SNPN is optionally present either if the onboarding indication is broadcast for the SNPN or if indication of accessing using CH is broadcast or both.

# Conclusion

It is requested that RAN2 discussed the following observations and proposals:

Observation#1: Which GID for onboarding or for CH or for both are configured over the NAS and hence there is no need for separate GID signalling or explicit indication for each GID in the SIB broadcast whether they are for onboarding or CH or for both,

Proposal#1: No separate Group IDs per SNPN signalling or indication per GID is needed for differentiating onboarding purpose and for support of credentials of third-party entities in SNPN; which GID is used for onboarding or CH can be done transparent to the AS.

Observation#2: The GIN list should not be made dependent of the Indication of accessing using CH of an SNPN as the SNPN may use GIN list for just onboarding.

Proposal#2: GIN list for a SNPN is optionally present either if the onboarding indication is broadcast for the SNPN or if indication of accessing using CH is broadcast or both.

# References

[1] TR23.700-07 v17.0.0 Study on enhanced support of non-public networks

[2] S2-2101076 Reply LS on clarification request for eNPN features

[3] R2-2106545 LS on Group IDs for Network selection (GINs)

[4] S2-2100807, O-SNPN selection, Qualcomm Inc