3GPP TSG RAN WG2 Meeting #116-e                                                       R2-2109619

# Electronic meeting, 1st-12th November 2021

Agenda item:		8.6.3

Source:			Intel Corporation, ZTE corporation, Sanechips, Samsung,

Qualcomm, OPPO, Sharp, Xiaomi, Sony, CATT, Apple

Title:		DCCH vs CCCH based approach for indication of non-SDT data arrival

Document for:	 	Discussion and decision

# Introduction

This contribution discussed the differences and the open topics to enable the switch from SDT to RRC_CONNECTED upon detecting non-SDT traffic in UE via DCCH-based approach vs via CCCH-based approach. It takes as baseline the discussion and proposals captured in previous email discussion [Post114-e][507] [1] where the following summary points were captured for each solution.

Summary of CCCH-based approach:

- If anchor relocation is required, UE is pushed back to INACTIVE (Proposal 1); if a new solution is to be developed instead, liaise with SA3 on issue of key update during node change (Proposal 2)

- PDCP entities of non-SDT RBs are re-established using existing mechanisms (Proposal 3)

- PDCP is suspended and PDUs flushed. FFS whether PDCP operation is same or not as legacy one (depdendent to other details to be agreed on CCCH-based approach) (Proposal 4)

- UE performs UAC and initiates RACH. (Observation 4)

- Same I-RNTI is used in the 2nd RRCResumeRequest and network needs to differentiate that 2nd RRCResumeRequest for that UE (e.g. implicit detection via I-RNTI). (Proposal 5)

- Discussion needed on whether PDCP count is reset or not.  And if reset, discussion about a new security mechanism with SA3 needed to prevent the re-use of the same security key for data after PDCP COUNT is reset. (Proposal 6 and Proposal 8)

- Liaise with SA3 on whether there is an issue with the re-use of ResumeMac-I in the 2nd Resume request; if considered an security issue, discussion of a new security mechanism with SA3 needed. (Proposal 7, Proposal 8)

- Liaise RAN3 to enable a mechanism to identify the UE context in the new gNB if the UE context was relocated, as I-RNTI in second ResumeRequest points to the context in the old gNB. (Proposal 9).

- Liaise with SA3 on whether a new KgNB is needed in serving gNB (different from anchor gNB) for the 2nd gNB after 2nd ResumeRequest if context was previously anchored. (Proposal 10)

- Liaise with CT1 to complete the discussion on whether NAS will trigger a new ResumeRequest for non-SDT data. (Proposal 11)

Summary of DCCH-based approach:

- If anchor relocation is required, UE is pushed back to INACTIVE (Proposal 1); if a new solution is to be developed instead, liaise with SA3 on issue of key update during node change (Proposal 2)

- PDCP entities of non-SDT RBs are re-established using existing mechanisms (Proposal 3)

- AS to initiate the DCCH message. (Proposal 11)

- Discuss whether NAS will trigger the DCCH message message and if so, liaise with CT1 to complete the discussion on whether NAS will trigger it. (Proposal 11)

- Discuss which DCCH message to use (new or UE assistance information). (Proposal 12)

- New Resume cause to be provided in DCCH message. (Proposal 13)

- Discuss what additional information is needed in the DCCH message (list of RB IDs/data volume per RB). (Proposal 13)

- UE waits for network response after sending DCCH message; discuss whether a confirmation of reception of DCCH message is needed (Proposal 14)

- UE initiates RACH procedure if there is no UL grant to send DCCH message (Proposal 15)

# Discussion

Figure 1 shows the flow diagram on how DCCH-based approach works in high level to switch from SDT to RRC_CONNECTED when non-SDT data is available.

Figure 1: DCCH-based approach to switch from SDT to RRC_CONNECTED when non-SDT data is available

Figure 2 shows the flow diagram on how CCCH-based approach works in high level to switch from SDT to RRC_CONNECTED when non-SDT data is available. Two options are explained considering the inputs provided in email discussion [Post114-e][507] [1]: option A) where the 2nd CCCH msg is decoded by the old anchor gNB, and option B) where the 2nd CCCH msg is decoded by the new anchor gNB (which requires a new RRCResumeRequest message, as well as, a change of the resume MAC-I calculation). In Figure 2, the key points specific to CCCH-based approach are marked in red (for common behaviours between both option A) and B)) and blue (for different ones).

Figure 2: CCCH-based approach to switch from SDT to RRC_CONNECTED when non-SDT data is available

The focus is to compare the technical details on how each approach works taking into consideration companies views provided on email discussion [Post114-e][507] [1]. It is also included when any of the point is categorized here as a positive aspect , a negative one  or neutral impact .

| Characteristics to enable the switch from SDT to RRC_CONNECTED | CCCH-based | DCCH-based |
| --- | --- | --- |
| Node that controls the switch to CONNECTED upon detecting non-SDT data | UE autonomously terminates the SDT session (i.e. without network control) and immediately triggers a 2nd new RRCResumeRequest msg | New anchor gNB is in control as it triggers the fallback from SDT to CONNECTED after non-SDT data is detected |
| UE updates network when UL non-SDT data is available during SDT session | 2nd RRCResumeRequest needed - For Opt.A), Legacy RRCResumeRequest - For Opt.B), New RRCResumeRequest | UE may provide the notification when non-SDT data is detected via a new RRC msg or reusing a legacy one) |
| RACH access | Always needed | Only needed when UE does not have any UL grant |
| Data loss | Preventing data loss may be a problem: - Any DL SDT data sent before the 2nd CCCH message is decoded (and before contention resolution) will be lost. - depending on how the mechanism to switch is defined (considering the handling of the PDCP suspension, PDCP COUNT, and security key) | Not a problem since the SDT bearers are not interrupted |
| Data duplication | Same as for data loss | Not a problem |
| Update required of current resume/suspend mechanism | Needed , for the handling of: - PDCP suspension - PDCP COUNT - security key/resumeMAC-I calculation | Not needed |
| New RRC message | Depends: - For Opt.A), Not needed - For Opt.B), Needed | Depends (UE may provide the notification when non-SDT data is detected via a new RRC msg or reusing a legacy one) |
| Impact in case of anchor relocation | RAN3 impacts (mainly for Opt.A)): - Old anchor node needs to retain the UE context even after anchor relocation and path switch - Old anchor gNB will perform MAC-I verification (even though the UE context has been relocated to a new anchor gNB) - Old anchor gNB needs to indicate to the new anchor gNB using new Xn message to move a UE (w/ context previously relocated) to RRC_CONNECTED state - For option B: to ensure that I-RNTI is not re-allocated to another UE by the anchor gNB RAN2 impacts: - For Opt.A), New procedure may be needed to move the UE to CONNECTED state when old anchor gNB “informs” to transition to the new anchor gNB - For Opt.B), A new CCCH msg with new MAC-I needs to be defined. (using new key of the current cell, I-RNTI of context in old gNB, new ResumeMAC-I calculation using new varResumeMAC-I (new CRNTI, source and target cell ID of new cell), not setting Count, Bearer and Direction to all binary zeros) - New anchor gNB needs to store and identify I-RNTI used that points to other old anchor gNB. General performance impact: - Data session is interrupted for both SDT and non SDT. For opt.A), there is additional Xn round trip delay incurred for the UE to transition to connected state (i.e. whilst the UE is verified and a trigger is received from the old anchor to the new anchor) | No problem as new anchor gNB already has a copy of UE AS Context, controls the switch and terminates the RRC layer. |

On summary, the following observations summarizes the potential disadvantages explained in above table:

The CCCH based approach where UE autonomously terminates the SDT session and initiates another CCCH message to the network when data on non-SDT DRB is detected has the following disadvantages:

  - Requires changes to current suspend/resume procedure to prevent data loss and detect data duplication over the air interface.

  - Requires changes to current suspend/resume procedure to prevent loss of buffered data in the network by identifying the UE context of an existing UE with ongoing SDT session in the new anchor gNB and retransmission of the buffered DL data to the UE.

  - Requires changes to Resume procedure to meet the security requirement captured in RRC spec that same COUNT and security key are not used.

  - Wrong I-RNTI in the 2nd RRCResumeRequest i.e. new anchor gNB will not be able to identify the UE context if the network had relocated the UE context to old anchor gNB during the SDT session unless a new procedure is defined/done (by the new anchor gNB), as assumed in for option B).

  - Requires work for MAC-I verification: to be done by the old anchor gNB for 2nd RRCResumeRequest (for option A)) or a new one to be done by the new anchor gNB for a new 2nd RRCResumeRequest message (for option B)).

  - Additional delay and signalling to the network due to the RACH, 2nd RRCResumeRequest and potential Xn signaling between new and old anchor gNBs.

  - Reset L2 and potentially re-tx SDT data if PDCP COUNT is reset.

  - For option A), new procedure in the new gNB may be needed to move the UE to connected when old anchor gNB informs the new anchor gNB of transitioning to RRC_CONNECTED a UE (with a context previously relocated).

  - For option B), a new CCCH msg with new ResumeMAC-I needs to be defined (using new key of the current cell, I-RNTI of context in old gNB, new varResumeMAC-I (new CRNTI, source and target cell ID of new cell), not setting Count, Bearer and Direction to all binary zeros),

  - More complex solution from implementation and specification point of view.

  - Large impact to RAN3 specification work, such as, for option A) after relocating a UE context, the old anchor needs to retain the UE context, perform MAC-I verification and indicate to the new anchor gNB using a new Xn message to move the UE (with a context previously relocated) to RRC_CONNECTED state, and for option B) coordination between old and new anchor gNB to guarantee that I-RNTI is not relocated while SDT session is ongoing.

  - Inter-dependencies with SA3 and CT1 to be addressed.

The DCCH based approach where UE notifies gNB when non-SDT DRB is detected and network triggers the fallback to RRC_CONNECTED has the following disadvantages:

  - A new UL RRC message may be defined or legacy UEAssistanceInformation msg may be reused for UE to indication when non-SDT data is available.

  - If UE does not have any UL grant, UE needs to perform RACH.

Therefore considering the disadvantages analyzed above we propose RAN2 already take a decision not to adopt the CCCH based approach irrespective of the outcome of the CT1 and SA3 discussions.

- RAN2 only enables DCCH-based approach where UE in RRC_INACTIVE with an ongoing SDT session indicates the network when data on non-SDT RB(s) is available.

# Conclusion

The observations captured are the following:

Observation 1.	The CCCH based approach where UE autonomously terminates the SDT session and initiates another CCCH message to the network when data on non-SDT DRB is detected has the following disadvantages:

(a)	Requires changes to current suspend/resume procedure to prevent data loss and detect data duplication over the air interface.

(b)	Requires changes to current suspend/resume procedure to prevent loss of buffered data in the network by identifying the UE context of an existing UE with ongoing SDT session in the new anchor gNB and retransmission of the buffered DL data to the UE.

(c)	Requires changes to Resume procedure to meet the security requirement captured in RRC spec that same COUNT and security key are not used.

(d)	Wrong I-RNTI in the 2nd RRCResumeRequest i.e. new anchor gNB will not be able to identify the UE context if the network had relocated the UE context to old anchor gNB during the SDT session unless a new procedure is defined/done (by the new anchor gNB), as assumed in for option B).

(e)	Requires work for MAC-I verification: to be done by the old anchor gNB for 2nd RRCResumeRequest (for option A)) or a new one to be done by the new anchor gNB for a new 2nd RRCResumeRequest message (for option B)).

(f)	Additional delay and signalling to the network due to the RACH, 2nd RRCResumeRequest and potential Xn signaling between new and old anchor gNBs.

(g)	Reset L2 and potentially re-tx SDT data if PDCP COUNT is reset.

(h)	For option A), new procedure in the new gNB may be needed to move the UE to connected when old anchor gNB informs the new anchor gNB of transitioning to RRC_CONNECTED a UE (with a context previously relocated).

(i)	For option B), a new CCCH msg with new ResumeMAC-I needs to be defined (using new key of the current cell, I-RNTI of context in old gNB, new varResumeMAC-I (new CRNTI, source and target cell ID of new cell), not setting Count, Bearer and Direction to all binary zeros),

(j)	More complex solution from implementation and specification point of view.

(k)	Large impact to RAN3 specification work, such as, for option A) after relocating a UE context, the old anchor needs to retain the UE context, perform MAC-I verification and indicate to the new anchor gNB using a new Xn message to move the UE (with a context previously relocated) to RRC_CONNECTED state, and for option B) coordination between old and new anchor gNB to guarantee that I-RNTI is not relocated while SDT session is ongoing.

(l)	Inter-dependencies with SA3 and CT1 to be addressed.

Observation 2.	The DCCH based approach where UE notifies gNB when non-SDT DRB is detected and network triggers the fallback to RRC_CONNECTED has the following disadvantages:

(a)	A new UL RRC message may be defined or legacy UEAssistanceInformation msg may be reused for UE to indication when non-SDT data is available.

(b)	If UE does not have any UL grant, UE needs to perform RACH.

The proposals captured are the following:

Proposal 1.	RAN2 only enables DCCH-based approach where UE in RRC_INACTIVE with an ongoing SDT session indicates the network when data on non-SDT RB(s) is available.

# Reference

- R2-2107292, Report of email discussion [Post114-e][507][SData] Non-SDT data arrival handling, Intel Corporation, August 2021.