3GPP TSG RAN WG2 Meeting #116-e                                                         R2-2109623

# Electronic meeting, November 2021                                    (Revision of R2-2107297)

Agenda item:		8.6.5

Source:			Intel Corporation

Title:		CG-SDT leftover Issues

Document for:	 	Discussion, Decision

# Introduction

In RAN2#115-e meeting, there was extended discussion on leftover aspects related to CG-SDT operation and the following agreements were made [1]:

Agreements 1. If none of the SSBs’ RSRP is above the RSRP threshold of CG-SDT criteria in the type selection phase, UE should select RA-SDT if RA-SDT criteria is met 2. MAC PDU rebuilding is not required (unless we find a case that is needed) 3. During subsequent CG transmission phase (i.e. after the UE has received response from NW) UE can initiate at least legacy RACH procedure (e.g. trigger due to no UL resources). No MAC PDU rebuilding is required. FFS if the RA-SDT RA resources can be used for subsequent data. a. At least the following conditions are agreed: (1) no qualified SSB when the evaluation is performed; (2) when TA is invalid; (3) when SR is triggered due to lack of UL resource 4. UE should release CG-SDT resource (if stored) when UE initiates RRC resume procedure from another cell which is different from the cell in which the RRCRelease is received. 5. The C-RNTI previously configured in RRC_CONNECTED state is used for UE to monitor PDCCH in CG-SDT. 6. CS-RNTI based dynamic retransmission mechanism can be reused for CG-SDT. FFS whether CS-RNTI is the same one as the one previously configured in RRC_CONNECTED or a new CS-RNTI one is provided to the UE 7. During the subsequent new CG transmission phase, for the purpose of CG resource selection, UE re-evaluates the SSB for subsequent CG transmission. FFS what happens if no SSBs are valid or if no sample is available 8. From RAN2 perspective, at least the following parameters should be included in the CG-SDT configuration. FFS whether these parameters are common for multiple CG-SDT configurations or per CG-SDT configuration. - The new TA timer in RRC_INACTIVE; - The RSRP change threshold for TA validation mechanism in SDT (details dependent on RAN1); The SSB RSRP threshold for beam selection (i.e. UE selects the beam and associated CG resource for data transmission).

This contribution further discusses unresolved aspects related to the usage of configured grant for SDT aiming to add further details or topic with the ones addressed in related email discussion [2].

# Discussion

## RSRP criterion for SSB selection

In the last RAN2 meeting, it was agreed that unlike the case of RACH, if the UE performing CG-SDT cannot find a SSB with RSRP above the configured threshold for selection of CG resources, UE should select RA-SDT if RA-SDT criteria is met. Moreover, it was also agreed that for the subsequent CG transmissions, the UE re-evaluates the SSB for subsequent CG transmissions, but it is still FFS what happens if no SSBs are considered valid or if the UE is unable to obtain the (filtered) RSRP measurement. In our view, the main reason this FFS was raised was because of concern from some companies that the UE may not be able to obtain the SSB RSRP measurement for consecutive CG transmissions. However, the same principle is applicable for non-SDT RACH procedure where the UE shall evaluate SSB RSRP for RA resource selection. Thus, given that the underlying principle for RSRP measurement for SSB to select associated resources is the same, we think the same behavior as in the case RA-SDT can be applied, i.e. UE selects any SSB if there is no qualified SSB for CG-SDT. Therefore, in this case, we may not need to define any additional behavior for the case of CG-SDT.

Proposal 1:	For the case when the UE has to re-evaluate the SSBs for subsequent CG transmissions, in case no qualified SSB can be selected for selection of associated CG resources, the UE selects any SSB (as in the case of legacy RA and RA-SDT).

## RA-SDT RA resource usage

In the post-meeting email discussion [2], it was discussed whether the UE can use the resources configured for RA-SDT in the subsequent data transmission phase when using CG-SDT. While it was agreed that the UE can initiate legacy RACH procedure during an ongoing CG-SDT procedure, the question is whether the UE can use the resources configured for RA-SDT for subsequent transmissions. From our perspective, there is a clear distinction between the RA-SDT and CG-SDT procedure, at least in terms of the SSB-to-RA/CG resource mapping for UL transmissions. Allowing the UE to switch to using RA-SDT resources for subsequent CG-SDT transmissions would require additional handling because of different resource mapping and power control as well as different fallback procedures in case SSB RSRP criteria is not met. In addition, the main use case outlined by companies during the discussion to support this behaviour was for the case of triggering SR, but it is worth noting that the NW can anyway configure legacy RACH resources for SR. Therefore, there seems little motivation to allow this inter-mixing of different resource usage with limited gain and substantial design complexity. Therefore, we proposed to not support this enhancement for either RA-SDT or CG-SDT and just removing the FFS.

Proposal 2:	UE should not be allowed to use RA resources allocated for RA-SDT during an ongoing SDT procedure, irrespective of whether the ongoing SDT procedure was initiated using RA or CG resources.

## PDCCH monitoring after CG transmission

One key issue that needs further discussion is on the window/timer which was agreed to be started by the UE after an UL CG-SDT transmission, using either CG or DG. It is an open question whether a new or existing timer is used, how it operates and the UE behavior when it expires, etc. While there was a proposal submitted as part of email discussion on CG related aspects during RAN2#115 e-meeting, there was no agreement made in this regard. In the following discussion, we seek to elaborate on some of the fundamental assumptions to be made regarding this timer:

- The UE starts/restarts the timer/window after each UL CG-SDT transmission (either CG or DG) or reception.

- The UE stops the timer when SDT session successfully terminates (e.g. reception of RRCRelease msg).

- Once the timer expires, SDT session is considered terminated, in which case UE behavior needs to be described

Before going into more details, we think it is worthwhile to highlight two distinct scenarios regarding the UE operation. Specifically, the timer/window used for PDCCH monitoring after CG/DG transmission also depends on whether the UE has received some acknowledgement from the NW for the first UL CG-SDT message or not. In our understanding, in case the UE has not yet received any acknowledgement of that first UL CG-SDT message, the CG-SDT session has not formally successfully begun since it is possible that the network has not received any UL transmission from the given UE. On the other hand, if the UE has already received some acknowledgement from the network for previously transmitted UL CG-SDT, the SDT session can be considered as ongoing and switching to RA-SDT can only happen if the UE detects CG-SDT failure, i.e. as part of a new SDT session. Therefore, it is clear that the PDCCH monitoring timer in this case behaves very similar to the already agreed T319’ timer for SDT failure detection. Specifically, the UE behavior for 3) should be similar between the RA and CG case, i.e. common SDT failure handling as we discuss in [3].

Proposal 3:	The T319’ (i.e. SDT failure detection) timer is (re)used for PDCCH monitoring after each CG-SDT transmission.

Proposal 4:	The UE (re)starts the timer after each CG/DG transmission and reception and stops it upon successful termination of the SDT session. If the timer expires, SDT failure is declared and subsequent SDT failure handling is triggered.

## TA validity mechanism

On the TA validation mechanism, RAN2 agreed so far to the following:

In RAN2#113e: - “TAT-SDT is started upon receiving the TAT-SDT configuration from gNB, i.e. RRCrelease message, and can be (re)started upon reception of TA command. - From RAN2 point of view, assume similar to PUR, that we introduce a TA validation mechanism for SDT based on RSRP change, i.e. RSRP-based threshold(s) are configured. Ask RAN1 to confirm. FFS on how to handle CG configuration when TA expires or when is invalid due to RSRP threshold. Details of the TA validation procedure can be further discussed. - UE releases CG-SDT resources when TAT expires in RRC_Inactive state” In RAN2#112e: - “A new TA timer for TA maintenance specified for configured grant based small data transfer in RRC_INACTIVE should be introduced. FFS on the procedure, the validity of TA, and how to handle expiration of TA timer. The TA timer is configured together with the CG configuration in the RRCRelease message”

Furthermore, RAN1 has sent and LS detailing their work on defining the SSB subset for RSRP based TA validation procedure. It is concluded that the SSB subset is determined at least based on a configured absolute RSRP threshold. However, there remains some FFS issues for TA validation mechanism discussed below:

- The SSB subset for RSRP based TA validation is determined at least based on a configured absolute RSRP threshold. - The SSB subset for RSRP based TA validation is determined as - Option 1: Within a set of SSBs configured per CG configuration - Option 2: Within a set of SSBs configured for all CG configurations - Option 3: Within a set of all SSBs actually transmitted as indicated in SIB1 - Option 4: Highest N SSBs of all SSBs actually transmitted as indicated in SIB1

Assuming that UE is still in the same cell where the CG-SDT configuration was provided, TA validation mechanism will require the UE to check two criteria:

- Criterion 1) whether TA timer has expired or not

- Criterion 2) whether RSRP delta meets or not the configured threshold for the delta.

Based on incoming LS from RAN1 (R1-2108649), it seems like RAN1 cannot make a consensus on which of the above options is selected and leaves it to RAN2 to decide. From our perspective, option 1 offers the most flexible approach, i.e. set of SSBs configured per CG configuration. Hence, we propose to go with Option 1 above, i.e. the SSB subset of RSRP based TA validation is determined from within a set of SSBs configured per CG configuration.

Proposal 5:	Regarding the selection of SSB for RSRP based TA validation, the SSB subset for RSRP based TA validation is configured per CG configuration (i.e. Option 1 as per RAN1 LS).

We should also discuss the UE behavior when the UE does not meet this RSRP criteria (i.e. Criterion (2)). Specifically, when TA validation criteria of the RSRP delta is not met, the handling of the CG-SDT configuration should be discussed. Two options are possible: option (1) UE autonomously releases the CG-SDT configuration and option (2) UE keeps the CG-SDT configuration but considers it as invalid.

If option (1) were allowed, UE and gNB would have a mismatch of UE’s stored configuration (i.e. UE releases CG-SDT config. vs gNB that assumes it valid while TAT is running). Depending on the TAT value, this mismatch can be substantial and so, from specification point of view, we should avoid creating this mismatch in purpose. Therefore, we suggest enabling option (2). The UE can thus keep the CG-SDT configuration stored but should not use it until it meets the delta RSRP criterion again. By considering stored CG-SDT configuration as not valid, the proposed handling to continue the initiation phase via RA-SDT would also be aligned with previous RAN2 agreement as explained above.

Proposal 6:	While the TA validation criteria of the RSRP delta threshold is not met, UE shall not use the CG resources but keeps stored the CG-SDT configuration (although is considered as not valid).

It is worth noting that in case one or both of the criteria above are met, the UE can still continue with the SDT session (rather than declaring SDT failure) by using dynamic grants for transmission scheduled by the network. In this scenario, the relevant UE behaviour regarding the keeping or release of the CG-SDT configuration can still apply as reference in proposals above. It should also be pointed out that the discussion here pertains to the CG-SDT specific TA timer. In our companion contribution [4], we discuss the details of normal/legacy TAT timer and how the UE handling differs with the CG-SDT specific TA timer. On summary, a CG-SDT procedure would require both normal/legacy TAT timer and CG-SDT specific TA timer.

Proposal 7:	During an ongoing SDT session, if CG-specific TAT expires (or is not running) or TA criterion for RSRP-delta threshold is not met, the SDT session can still continue using DG (understanding that CG-SDT configuration is released upon CG-specific TAT expiry and is considered invalid when TA criterion for RSRP-delta is not met).

## Assistance information

The UE can provide some assistance information to the network before moving to RRC_INACTIVE in case it is interested in performing SDT and request CG configuration by the network. It seems that while this aspect was originally proposed to be included as part of the email discussion [2] scope, it was removed since some companies think it should be discussed in RAN3 first since the gNB already has the “TSC Assistance Information” which can be reused for the CG SDT procedure. However, from our perspective, this entire topic falls in the purview of RAN2 and RAN2 is the WG aware of the information helpful for the gNB to configure the proper configuration associated with the CG-SDT procedure.  In addition, if RAN2 preference is to reuse TSCAI related information, we assume that SA2/RAN3 confirmation would be indeed required on its applicability for eMBB (understanding that this container was defined in relation to Rel-17 IIOT and Rel-17 5G-AIS). Summarizing, if companies think TSCAI related information is sufficient, then we should get confirmation from SA2 first before agreeing to use it.

In any case, the provision of this assistance information in LTE can be accomplished by PURConfigurationRequest message, which includes information like the traffic periodicity and offset, number of PUR occasions, requested TBS, etc. Since the gNB might not be aware of the UE’s expected traffic pattern during RRC_INACTIVE, we think it is useful to be able to indicate this information to the gNB, to indicate that the UE is interested to be configured with CG-SDT and provide SDT related information to the gNB, which can then be taken into consideration when providing a CG configuration to the UE for SDT. It can also indicate if the UE is no longer interested in CG-SDT (i.e. via a ReleaseRequest). Therefore, we propose to consider some assistance information by the UE to the gNB for requesting CG configuration for SDT. The assistance information can be part of the legacy UE assistance information framework and the detailed contents of this information can follow PUR related information as a baseline.

| Parameter | Usage |
| --- | --- |
| requestedNumOccasions | The number of requested CG-SDT occasions (can be one or infinite) |
| requestedPeriodicityAndOffset | Indicates the requested periodicity for the CG-SDT occasions and time offset until the first CG-SDT occasion |
| requestedTBS | Indicates the requested TBS for the CG-SDT. The maximum requested TBS is limited to the UL TBS size supported by the UE. |
| rrc-ACK | Indicates RRC response message is preferred by the UE for acknowledging the reception of a transmission using CG-SDT. |

Proposal 8:	RAN2 is proposed to agree that assistance information for configuration of CG-SDT resource can be provided by the UE to the gNB in RRC_CONNECTED. At least at least requestedNumOccasions and requestedPeriodicityAndOffset can be reused from LTE as baseline.

# Conclusion

This contribution discusses the open aspects related to usage of configured grant for SDT operation and makes the following proposals:

Proposal 1:	For the case when the UE has to re-evaluate the SSBs for subsequent CG transmissions, in case no qualified SSB can be selected for selection of associated CG resources, the UE selects any SSB (as in the case of legacy RA and RA-SDT).

Proposal 2:	UE should not be allowed to use RA resources allocated for RA-SDT during an ongoing SDT procedure, irrespective of whether the ongoing SDT procedure was initiated using RA or CG resources.

Proposal 3:	The T319’ (i.e. SDT failure detection) timer is (re)used for PDCCH monitoring after each CG-SDT transmission.

Proposal 4:	The UE (re)starts the timer after each CG/DG transmission and reception and stops it upon successful termination of the SDT session. If the timer expires, SDT failure is declared and subsequent SDT failure handling is triggered.

Proposal 5:	Regarding the selection of SSB for RSRP based TA validation, the SSB subset for RSRP based TA validation is configured per CG configuration (i.e. Option 1 as per RAN1 LS).

Proposal 6:	While the TA validation criteria of the RSRP delta threshold is not met, UE shall not use the CG resources but keeps stored the CG-SDT configuration (although is considered as not valid).

Proposal 7:	During an ongoing SDT session, if CG-specific TAT expires (or is not running) or TA criterion for RSRP-delta threshold is not met, the SDT session can still continue using DG (understanding that CG-SDT configuration is released upon CG-specific TAT expiry and is considered invalid when TA criterion for RSRP-delta is not met).

Proposal 8:	RAN2 is proposed to agree that assistance information for configuration of CG-SDT resource can be provided by the UE to the gNB in RRC_CONNECTED. At least at least requestedNumOccasions and requestedPeriodicityAndOffset can be reused from LTE as baseline.

# References

| [1] | “RAN2#115 e-Meeting, Chairman Notes”. |
| --- | --- |
| [2] | “R2-210xxxx_Summary_of_[Post115-e][509][SDT]_CG_open_issues_(Xiaomi)”. |
| [3] | “R2-2109620, Control plane leftover issues for SDT procedure (Intel)”. |
| [4] | “R2-2109621, User plane leftover issues for SDT procedure (Intel)”. |