3GPP TSG-RAN WG2 Meeting #116-e	R2-2109642

E-Meeting, Nov 1st – Nov 12th, 2021

Agenda item: 	8.17.3

Source:		Intel Corporation

Title:	Remaining issues on mTRP BFR

Document for:	Discussion and Decision

# Introduction

In this document, we discuss two remaining issues on mTRP BFR based on RAN1 and RAN2 agreements made so far.

# PUCCH-SR

In RAN2 #115e meeting, it was agreed that if beam failure is detected on both TRPs of SpCell, random access procedure is initiated on SpCell. If beam failure is detected on either TRP, BFR is triggered for the BFD-RS set which means BFR MAC CE is transmitted. When BFR MAC CE is generated, if there is no new UL grant, the UE should trigger SR. If there is a valid SR, the UE could send SR on PUCCH-SR.

Observation 1: PUCCH-SR for BFR should be used for SpCell BFR for the case where beam failure recovery is triggered on one of TPRs in SpCell.

It is our understanding that RAN1 is under discussion on various PUCCH-SR cases for mTRP BFR as follows.

- Two PUCCH-SRs with association between BFD-RS set and PUCCH-SR

- Two PUCCH-SRs without association between BFD-RS set and PUCCH-SR

- One PUCCH-SR without repetition

- One PUCCH-SR with repetition

Except case 1, there is no clear agreement yet in RAN1. RAN1 would agree at least one of case 3 and case 4 because RAN1 agreed to have one PUCCH-SR for mTRP BFR. Since RAN1 agreed that the support of association between BFD-RS set and PUCCH-SR is UE capability, case 2 might be a possible case but there is no point to allow this case. In case 4, repetition is to support multiple TRPs (spatial relationship) but the explicit association to a certain TRP is not known to the UE. That is why repetition is supported.

Based on above RAN1 status, RAN2 also need to consider different PUCCH-SR operation for mTRP BFR with respect to the possible cases.

- Case 1: Two PUCCH-SRs with association between BFD-RS set and PUCCH-SR

  - The UE can determine which PUCCH-SR is invalid (i.e. beam failure is triggered for the BFR-RS) via the explicit association. When beam failure is detected on one of TRPs, SR should be transmitted on the valid PUCCH-SR. That is, PUCCH-SR associated to BFD-RS set which beam failure is not detected is considered as valid, while PUCCH-SR associated to the beam failed TRP should be considered as invalid.

Proposal 1: If two PUCCH-SRs are configured with explicit association with BFD-RS set, when beam failure is detected in one of TRPs on SpCell, PUCCH-SR associated to the beam failed TRP should be considered as invalid.

- Case 2: Two PUCCH-SRs without association between BFD-RS set and PUCCH-SR

  - The UE cannot determine which PUCCH-SR is invalid. Therefore, even if two PUCCH-SRs are configured, the UE should assume that both PUCCH-SRs are invalid. Random access should be triggered to send SR. On the other hand, in this case, it is questionable why gNB would configure two PUCCH SRs.

Proposal 2: RAN2 asks RAN1 if two PUCCH-SRs without association should be supported. If supported,  when beam failure is detected in one of TRPs on SpCell, both PUCCH-SR should be considered as invalid.

- Case 3: One PUCCH-SR without repetition

  - In this case, when beam failure is detected in one of TRP on SpCell, it is hard for the UE to know whether PUCCH-SR is still valid because it may be possible that the PUCCH-SR is associated with the TRP which beam failure is detected.

  - In order to successfully transmit SR, the UE should trigger random access if PUCCH-SR associated to the BFD-RS set is invalid (i.e. beam failure is triggered for the BFR-RS) or if the UE doesn’t know the association.

  - Currently, it is our understanding that RAN1 has not discussed whether association between BFD-RS set and PUCCH-SR is supported for one PUCCH-SR.

Proposal 3: If one PUCCH-SR is configured without repetition, when beam failure is detected in one of TRPs on SpCell, the PUCCH-SR should be considered as invalid.

- Case 4: One PUCCH with repetition

  - In this case, although beam failure is detected in one of TRPs, it is still possible to transmit SR successfully thanks to repetition.

  - In order to successfully transmit SR, the UE should trigger random access if PUCCH-SR associated to the BFD-RS set is invalid (i.e. beam failure is triggered for the BFR-RS) or if the UE doesn’t know the association.

Proposal 4: If one PUCCH-SR is configured with repetition, when beam failure is detected in one of TRPs on SpCell, the PUCCH-SR should be considered as valid.

Additional issue is which PUCCH-SR is used when BFR is triggered for SCells when two PUCCH-SRs are configured. Since RAN1 concludes it is FFS whether to configure an association between a BFD-RS set on SCell and a PUCCH-SR resource for per TRP BFR, RAN2 could wait for RAN1 conclusion.

Proposal 5: RAN2 waits for RAN1 to conclude on PUCCH-SR selection for the case where beam failure is detected on SCell but not detected on SpCell.

# BFR MAC CE

In RAN2 #115e meeting, one of FFS points is related to what BFR MAC CE the UE transmits if beam failure is detected on both TRPs.

- For the case of intra cell (FFS for inter cell). If beam failure is detected on both TRPs (i.e. BFD-RS sets) of an SCell, BFR is triggered for that SCell. - - FFS whether UE transmits a) legacy BFR MAC CE or b) new BFR MAC CE indicating both failed TRPs as well as the beam failure recovery information for both TRPs. If beam failure is detected on both TRPs (i.e. BFD-RS sets) of SpCell, random access procedure is initiated on SpCell. - - FFS whether UE transmits a) legacy BFR MAC CE or b) new BFR MAC CE indicating both failed TRPs as well as the beam failure recovery information for both TRPs.

It is our understanding that “if beam failure is detected on both TRPs” means that beam failure happens on both TRPs at the same time/slot or sequentially during the short time period so that BFR is not cancelled. Therefore, the UE may send BFR of both BFD-RS sets. Considering the channel situation would be different from the different antenna panel, this case would not be usual case. In addition, it is still needed for the UE to indicate the candidate beam information for both TRPs. Therefore, it is reasonable to trigger new BFR MAC CE not to have optimization for this case.

Proposal 6: The UE transmits new BFR MAC CE whenever it is configured.

# Conclusion

In this document, we discuss two remaining issues on mTRP BFR based on RAN1 and RAN2 agreements made so far.

Observation 1: PUCCH-SR for BFR should be used for SpCell BFR for the case where beam failure recovery is triggered on one of TPRs in SpCell.

Proposal 1: If two PUCCH-SRs are configured with explicit association with BFD-RS set, when beam failure is detected in one of TRPs on SpCell, PUCCH-SR associated to the beam failed TRP should be considered as invalid.

Proposal 2: RAN2 asks RAN1 if two PUCCH-SRs without association should be supported. If supported,  when beam failure is detected in one of TRPs on SpCell, both PUCCH-SR should be considered as invalid.

Proposal 3: If one PUCCH-SR is configured without repetition, when beam failure is detected in one of TRPs on SpCell, the PUCCH-SR should be considered as invalid.

Proposal 4: If one PUCCH-SR is configured with repetition, when beam failure is detected in one of TRPs on SpCell, the PUCCH-SR should be considered as valid.

Proposal 5: RAN2 waits for RAN1 to conclude on PUCCH-SR selection for the case where beam failure is detected on SCell but not detected on SpCell.

Proposal 6: The UE transmits new BFR MAC CE whenever it is configured.

# Annex A: RAN2 agreements related to mTRP BFR

MAC entity maintains separate beamFailureDetectionTimer and BFI_COUNTER for each BFD-RS set of a serving cell configured with multiple BFD-RS sets. beamFailureDetectionTimer and beamFailureInstanceMaxCount configuration is configured independently for each TRP of serving cell. If the MAC entity receives beam failure instance indication for a BFD-RS set of a serving cell, it shall perform the following: - - (re-)start beamFailureDetectionTimer corresponding to that BFD-RS set of the serving cell; - - increment BFI_COUNTER corresponding to that BFD-RS set of the serving cell by 1. - - If BFI_COUNTER >= beamFailureInstanceMaxCount corresponding to that BFD-RS set of the serving cell: - - trigger a BFR for the BFD-RS set of the Serving Cell; - For the case of both intra cell and inter cell: BFD-RS set ID is included in BFR MAC CE to identify the failed TRP. - For the case of intra cell (FFS for inter cell). If beam failure is detected on both TRPs (i.e. BFD-RS sets) of an SCell, BFR is triggered for that SCell. - - FFS whether UE transmits a) legacy BFR MAC CE or b) new BFR MAC CE indicating both failed TRPs as well as the beam failure recovery information for both TRPs. If beam failure is detected on both TRPs (i.e. BFD-RS sets) of SpCell, random access procedure is initiated on SpCell. - - FFS whether UE transmits a) legacy BFR MAC CE or b) new BFR MAC CE indicating both failed TRPs as well as the beam failure recovery information for both TRPs. FFS what is meant in detail by “beam failure is detected on both TRPs”

# Annex B: RAN1 agreements related to mTRP BFR

RAN1#103-e

Agreement

- For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set.

  - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP

  - Support at least one of explicit and implicit BFD-RS configuration

    - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured

      - FFS: Further study QCL relationship between BFD-RS and CORESET

    - FFS: How to determine implicit BFD-RS configuration, if supported

- For M-TRP new beam identification

  - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured

    - FFS: detail on association of BFD-RS and NBI-RS

    - Support the same new beam identification and configuration criteria as Rel.16, including  L1-RSRP, threshold

RAN1#104-e

Agreement

For BFRQ of M-TRP BFR

- Option 3: Up to two dedicated PUCCH-SR resources in a cell group

- FFS: Whether PUCCH-SR for SCell can be reused for M-TRP

- Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found

  - Support at least indication of a single TRP failure

    - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE

    - FFS: whether/how to support  indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell)

- FFS: UE behavior when TRP failure status is different across cells

- FFS: Whether PUCCH SR resource can be configured with 2 spatial relations

RAN1#104b-e

Agreement

- Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs.

- FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

Agreement

A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group

- NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group

RAN1#106-e

Agreement

For multi-TRP BFR, a single MAC-CE is used at least for BFRQ for all TRPs in all CCs in a cell group, which includes

- Indices of failed BFD-RS set (as an indication of failed TRP link)

- Indices of CC containing the failed TRP link

- An indicator whether a new candidate beam is identified in the NBI-RS set associated with the failed BFD-RS set, and an resource indicator representing the new candidate beam (if identified) based on the number of NBI-RS resources in the corresponding NBI-RS set.

- FFS: Content of MAC-CE related to SpCell when transmitted on msg3, msgA

- Note: MAC-CE signaling design details are up to RAN2

- The term “failed TRP link” is used here for discussion purposes only

RAN1#106bis-e

Agreement

Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR.

- FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR

A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.

Agreement

RACH-based transmission can be triggered on a SpCell at least in the following scenarios

- Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell

- FFS: other scenarios

  - Scenario 2: at least one TRP fails on SpCell

  - Scenario 3: at least one pre-defined TRP fails on SpCell

  - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available

  - Scenario 5: If MAC-CE based reporting does not work (details FFS)

  - Scenario 6: When no PUCCH-SR is configured