3GPP TSG RAN WG2 Meeting #116-e                                                        R2-2109664

E-Meeting, 1st – 12th November, 2021                                  (Revision of R2-2107672)

Agenda item:		8.11.4

Source:			Intel Corporation

Title:		Support of On-Demand PRS request

Document for:	 	Discussion, Decision

# Introduction

In RAN2#114 e-meeting, there was some discussion for support of both UE-initiated and LMF- initiated on-demand transmission and reception of DL PRS for DL+UL positioning for UE based and UE assisted solutions was discussed. The following agreements were made in this regard [1]:

Agreement: The network can signal predefined PRS configurations to the UE and the UE can select one to request. FFS if the UE can request a configuration with different parameters and exactly which parameters are flexible. Agreement: Proposal 2: Define a new LPP assistance data IE which can contain a set of possible on-demand DL-PRS configurations, where each on-demand DL-PRS configuration has an associated identifier. Proposal 3 (modified): The new LPP assistance data IE from Proposal 2 can be included in an LPP Provide Assistance Data message and/or a new posSIB. Agreement: Proposal 4 (modified): The procedure(s) for on-demand DL-PRS should support at least the following functionality (up to RAN3 what is in NRPPa vs. OAM, etc.): - Providing the requested on-demand DL-PRS configuration information from an LMF to the gNB (e.g., explicit parameter or identifier of a predefined DL-PRS configuration), and confirmation of the request by the gNB - Provision of (possible/allowed) on-demand DL-PRS configurations that the gNB can support from a gNB to an LMF - TRP capability transfer (e.g., whether the RAN node supports the reconfiguration of DL-PRS, etc.)

Subsequently in RAN2#115 e-meeting, while several proposals were discussed, the following set of agreements were made [2]:

Agreements: Before providing available DL-PRS configuration to the UE, the LMF may obtain configuration information on what DL-PRS can be supported from one or more TRPs via NRPPa. Capture the steps provided above as a baseline, along with a note indicating it remains FFS if the UE can send the MO-LR to request on-demand PRS. FFS if we indicate to SA2 that MO-LR can be used to trigger on-demand PRS procedure. It is up to Network (LMF) implementation on the steps to follow (accept/reject/ignore) on receiving request from UE for changing the DL-PRS configurations.

In this contribution, we further discuss the support of on-demand PRS transmissions and address the remaining open aspects.

# Discussion

A key question for the support of the on-demand PRS request procedure is how this request is carried/signaled. For the UE initiated PRS request case, while we have agreed that the network can signal a set of predefined PRS configurations to the UE to select one to request, how the UE sends that request needs to be specified. In our view, the most straightforward way is to follow what was agreed for provisioning of the predefined PRS configuration sets, i.e. LPP signaling. Specifically, the LPP Request Assistance Data message can be utilized by the UE to carry the on-demand DL-PRS request in response to the network indicated PRS configuration sets and indicate the UE’s preference/selection to the network, at least for the MT-LR case (note that MO-LR case is under discussion in [3]In this regard, a new LPP assistance data IE shall be defined to indicate the identifier and/or parameters for the corresponding selected DL-PRS configuration.

Proposal 1:	The on-demand DL-PRS request is provided within the LPP Request Assistance Data message, at least for MT-LR. FFS for MO-LR case.

Proposal 2:	A new LPP assistance data IE shall be defined to indicate the requested DL-PRS configuration parameters/associated identifier.

While it has been agreed that the network can signal a set of predefined PRS configurations to the UE to select one to request, there are some open questions. For instance, one open issue is regarding network control over when the UE is able to request on-demand PRS. Our understanding is that the UE can only send the request to the LMF if the LMF supports the on-demand PRS feature. If the LMF provides the allowed configuration sets to the UE to select the requested parameters within the set, it can be implicitly assumed that the UE is allowed to trigger on-demand PRS request.

Proposal 3:	The UE can only send the on-demand PRS request if LMF enables this via LPP message ProvideAssistanceData, i.e. by providing preconfiguration sets to the UE.

In addition to this, there is extended discussion as part of the post meeting email discussion [4] on whether there needs to be some triggering criteria for the on-demand PRS request, e.g. based on QoS requirements, measurement quality, radio conditions, etc. While the options seem interesting on first glance, we think there is no real need to specify any restrictions on the UE triggering of on-demand request. The main reason for that is that since it is expected to utilize LPP signaling procedure, it is not a straightforward task to specify additional triggers for when the UE sends this message. For instance, there can be several cases to consider, e.g. when the PRS is switched on/off, when the UE wants more accurate positioning with PRS having larger bandwidth, when the UE may want to save power with PRS having smaller bandwidth, when the UE may want lower latency with PRS having smaller periodicity, etc. Having to specify all such criteria in a detailed manner is likely to require extended specification effort, which can easily be avoided if it is left to UE implementation.

Proposal 4:	No additional triggering criteria or events need to be specified to allow/disallow the UE from triggering on-demand PRS request.

Moreover, the UE behavior in terms of selecting and indicating particular PRS configuration set(s) needs to be specified. One open aspect from the last meeting is whether there is any need for the UE to be able to request a potentially different configuration with different parameters in case none of the indicated configuration sets are deemed suitable. Another option could be for the UE to indicate multiple DL-PRS configuration sets in some kind of priority ordering to allow for more flexibility for the network. However, in our understanding, the UE shall only select the requested parameters within the sets indicated by the LMF. We think that allowing the UE to pick and choose certain parameters from the pre-configured sets can lead to unneeded complexity, which does not seem worth the added benefits it brings.

Proposal 5:	The UE shall only select the DL-PRS requested parameters within the indicated configuration sets and indicate the selected PRS configuration set ID to the LMF.

Finally, one open question from the last meeting is on whether the UE needs to provide some additional assistance information to the LMF in determining the need to update the DL-PRS configurations. We think this question is more relevant for the LMF initiated case, where the LMF may decide to update the PRS configuration, e.g. based on RRM measurement results, positioning measurement results, etc. In this case, the we think the LMF can make this determination based on the PRS measurement reports that the UE provides anyway and can be considered as on-demand PRS “feedback”. Since it is already supported as part of LPP signaling (Location information transfer), no new signaling needs to be defined.

Proposal 6:	No additional signaling needs to be defined to provide assistance information to LMF to assist in the determination of on-demand DL-PRS configuration(s).

# Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1:	The on-demand DL-PRS request is provided within the LPP Request Assistance Data message.

Proposal 2:	A new LPP assistance data IE shall be defined to indicate the requested DL-PRS configuration parameters/associated identifier.

Proposal 3:	The UE can only send the on-demand PRS request if LMF enables this via LPP message ProvideAssistanceData, i.e. by providing preconfiguration sets to the UE.

Proposal 4:	No additional triggering criteria or events need to be specified to allow/disallow the UE from triggering on-demand PRS request.

Proposal 5:	The UE shall only select the DL-PRS requested parameters within the indicated configuration sets and indicate the selected PRS configuration set ID to the LMF.

Proposal 6:	No additional signaling needs to be defined to provide assistance information to LMF to assist in the determination of on-demand DL-PRS configuration(s).

# References

| [1] | “RAN2#114 e-Meeting, Chairman Notes”. |
| --- | --- |
| [2] | “RAN2#115 e-Meeting, Chairman Notes”. |
| [3] | “R2-210xxx, [Post115-e][606][POS] MO-LR for on-demand PRS (CATT)”. |
| [4] | “R2-210xxxx, [Post114-e][603][POS] Procedures and signalling for on-demand PRS (Ericsson)”. |